3GPP TSG-RAN WG4 Meeting #114	R4-2500512

Athens, Greece, 17th – 21th February, 2025

Agenda Item:	7.19.2

Source:	CAICT

Title:	Discussion on RRM core requirements for CSI prediction

Document for:	Discussion

1	Introduction

In this contribution, we provide relevant discussion on core requirements for CSI prediction.

2	Discussion

2.1 Requirements for CSI prediction

In last two RAN4 meetings, we had following agreement on this issue.

RAN4 #112bis Agreement - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management RAN4 #113 Agreement: - For channel model for CSI prediction tests, TDL channel models will be used. - FFS on whether CDL channel model is needed for generalization test. Agreement: - reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook Agreement: - use static conditions as baseline. - FFS on whether to have non-static condition

- For the bullet “FFS which legacy requirement/feature are relevant”, in our understanding, the relevant feature in current specification is non-AI based CSI prediction scheme, a.k.a, TypeII-Doppler-r18 codebook, which has also been taken as baseline in RAN1’s evaluation. The Demod and PMI reporting requirements specified for TypeII-Doppler-r18 codebook could be considered as starting point for AI-based CSI prediction. The requirement of TypeII-Doppler-r18 codebook was discussed in NR_MIMO_evo_DL_UL item. Some agreements are excerpted below for reference.

RAN4 #109 Issue 1-1-1: clarify criteria of feasibility for ‘typeII-Doppler-r18’ codebook Agreement: - Define PMI reporting requirements with ‘typeII-Doppler-r18’ using option 2 if both option 1 and option 2 could be fulfilled. Otherwise, if only option 1 is fulfilled, further discuss if feasible to define PMI reporting requirement using option 1 only. - Option 1: UE throughput with ‘typeII-Doppler-r18’ codebook could outperform Rel-16 Type II codebook with the same CSI-RS configurations and medium/high UE speed. - Option 2: UE throughput with ‘typeII-Doppler-r18’ codebook could outperform random precoding based on Single Panel Type I codebook with the same CSI-RS configurations and medium/high UE speed. Issue 1-1-2: clarify test metric for PMI reporting requirements with ‘typeII-Doppler-r18’ codebook Agreement: - Test metric defined as γ_1=t_(typeII-doppler)/t_rnd as a starting point, where t_(typeII-doppler) is X % (e.g. X=90) of the maximum throughput obtained at SNR_(typeII-doppler) using the typeII-Doppler-r18 precoder configured according to the UE reports, and t_rnd is the throughput measured at SNR_(typeII-doppler) with random precoding based on Type I Single Panel codebook. RAN4 #110bis Issue 1-1-3: X% of the maximum throughput in Test metric Agreement: - 90% of the maximum throughput in Test metric RAN4 #111 Issue 1-1-2: Test requirements of TypeII-Doppler-r18 codebook Agreement: - Set 2.5 as the test requirement for both 2Rx and 4Rx FR1 TDD cases for TypeII-Doppler-r18 codebook. Issue 1-1-3: Test setup for FR1 FDD case of TypeII-Doppler-r18 codebook Agreement: - Define FR1 FDD test requirement for TypeII-Doppler-r18 codebook based on current simulation results with current configuration agreed in RAN4#110bis meeting. Issue 1-1-2: Test requirements of TypeII-Doppler-r18 codebook Agreement: - Set 1.8 as the test requirement for both 2Rx and 4Rx FR1 FDD cases for TypeII-Doppler-r18 codebook.

- For the bullet “whether AI/ML based performance should be better than the legacy”, this could be further discussed and decided based on extensive evaluation in RAN4. The test setup adopted for TypeII-Doppler-r18 codebook could also be taken as starting point, shown as following.

RAN4 #110bis Issue 1-1-5: Test setup for FR1 FDD case of TypeII-Doppler-r18 codebook Agreement: - For FR1 FDD 15kHz SCS case, test setup is as below descriptions: - Non-CSI slots: scheduled in mod(i, 10) = {1,3,5,7,8,9}, i={0,1,2,…,19} - CSI-RS is scheduled in mod(i, 10) = {0,2,4,6}, i={0,1,2,…,19} - SSB slots: i=0 - TRS slots: i=10 and 11. - CSI report is transmitted in slot#10n+10 to estimate precoder in slot#10n+11 (i.e., δ=1) - Test equipment apply the estimated precoder in slot#10n+{15, 17, 18, 19, 21, 23} Issue 1-1-6: Test setup for FR1 TDD case of TypeII-Doppler-r18 codebook Agreement: - For FR1 TDD 30kHz SCS case, test setup is as below descriptions for the simulation of following Rel18 Doppler codebook vs random Type I single panel codebook: - Scheduling pattern for TDD SCS=30kHz with the periodicity of 20ms (40 slots): - Non-CSI slots: scheduled in mod(i, 10) = {5,6}, i={0,1,2,…,39} - CSI slots: scheduled in mod(i, 10) = {0,1,2,3}, i={0,1,2,…,39} - SSB slots: i=0 - TRS slots: i=20 and 21. - CSI report is transmitted in slot#10n+9 to estimate precoder in slot#10n+10 (i.e., δ=1) - Test equipment apply the estimated precoder in slot#10n+{15, 16}

Proposal 1: The Demod and PMI reporting requirements defined for TypeII-Doppler-r18 codebook could be considered as starting point for AI-based CSI prediction.

Proposal 2: Whether to define better requirements for AI-based CSI prediction shall be decided based on further evaluations. The test setup adopted for TypeII-Doppler-r18 codebook could be taken as starting point.

Conclusion

In this contribution, the issue of AI/ML performance is discussed with following proposals:

Proposal 1: The Demod and PMI reporting requirements defined for TypeII-Doppler-r18 codebook could be considered as starting point for AI-based CSI prediction.

Proposal 2: Whether to define better requirements for AI-based CSI prediction shall be decided based on further evaluations. The test setup adopted for TypeII-Doppler-r18 codebook could be taken as starting point.