3GPP TSG-RAN WG4 Meeting #114	R4-2500513

Athens, Greece, 17th – 21th February, 2025

Agenda Item:	7.19.3

Source:	CAICT

Title:	Discussion on RRM core requirements for beam management

Document for:	Discussion

1	Introduction

In this contribution, our preliminary consideration on relevant requirements of beam prediction is presented in section 2.1, and extensive discussion on absolute RSRP accuracy and relative RSRP accuracy are presented in section 2.2 and 2.3, respectively.

2	Discussion

2.1 Requirements of beam prediction

In RAN4 #112bis meeting, we had an agreement on this issue.

Issue 2-5: Requirements for beam prediction Agreement: - Requirements to be defined: - Measurement period for prediction (how long will set B be measured before making a prediction) - FFS on whether the legacy measurement period can be reused or not. - Prediction accuracy - FFS how/what to define depending on the chosen KPI (e.g. predicted RSRP, beam ID, Top-K/1 or Top 1/K, etc) - TCI state known/unknown - FFS on whether the other requirements are needed

Regarding to the 1st highlighted FFS, from our perspective, the principle to define the measurement period TL1-RSRP_Measurement_Period_SSB for FR2 in current specification TS 38.133 could be reused, copied as following.

The detailed value of {M, N, P} in the definition needs further study.

- The value of M depends on the minimum number of required measured instances as model input for inference. The value would be varied for different patterns of Set B.

- The value of N depends on UE Rx beam assumption, and whether Rx beam sweeping is enabled for inference.

- The value of P depends on the periodicity of SSB, SMTC, measurement gap and other related configurations. Current spec could be taken as starting point.

Proposal 1: The principle to define the measurement period TL1-RSRP_Measurement_Period_SSB for FR2 in current specification TS 38.133 could be reused. The detailed values of {M, N, P} needs further study.

Regarding to the 2nd highlighted FFS, we think several points need to be considered,

1) With the introduction of AI-based beam prediction, NW is anticipated to only configure measurement resources for beams in set B. This would induce different periodicities for beams in and out of set B, which from our perspective, may impact the performance requirements related to L1-RSRP measurements of UE side. Taking SSB-based beam failure detection as an example, “UE shall be able to evaluate whether the downlink radio link quality on the configured SSB resource in set q0 estimated over the last Tevaluate_BFD_SSB ms period becomes worse than a threshold”. In the definition of Tevaluate_BFD_SSB, whether existing value sets of {N, P} are able to meet certain requirements under different periodicities for beams in set q0 requires further verification.

Proposal 2: RAN4 to identify the impact of reduced resource configuration on existing RAN4 requirements with L1-RSRP related measurements. Table 1 could be considered as starting list.

Table 1

| RRM Req. Category | RRM procedure |
| --- | --- |
| Signaling characteristics | SSB /CSI-RS based Radio Link Monitoring |
|  | Link recovery procedures (SSB/CSI-RS based beam failure indication, L1 indication, Candidate beam detection) |
|  | Active TCI state switching delay |
|  | Uplink spatial relation switch delay |
|  | Pathloss reference signal switching delay |
|  | Active downlink TCI switching delay for unified TCI |
|  | Active uplink TCI switching delay for unified TCI |
| Measurement procedure | NR intra-frequency measurements (Number of cells and number of SSB, measurement reporting requirements, etc) |
|  | L1-RSRP measurements for reporting |
| Measurement performance requirements – NR measurements | Intra-frequency RSRP accuracy requirements for FR1/FR2 |
|  | RSRP measurement report mapping |
|  | L1-RSRP accuracy requirements for FR1/FR2 |

2) Based on the agreements in RAN1, the beam measurement report would be enhanced compare to legacy specification. Specifically, reporting more than 4 beams in L1 signaling would be supported for BM-case 1, and reporting inference results of multiple time instances would be supported for BM-case 2. Discussion on related requirements shall wait for other WGs to determine detailed reporting mechanism.

Agreement (RAN1 #116) For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, support the report of more than 4 beam related information in L1 signaling - Note: Purpose, such as above “For NW-sided model, for inference”, will not be specified in RAN 1 specifications - FFS on the report content for beam related information - FFS on max number of reported beam related information in one report Agreement (RAN1 #116bis) For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N(N>=1, FFS on N) future time instance(s) in one report - wherein information of inference results of one time instance is as in one report for BM-Case 1 - Note: overhead reduction is not precluded - FFS on details

Proposal 3: Postpone discussion on reporting related requirements until other WGs define detailed reporting mechanism.

3) In RAN4 #111, we had following agreement on RRM requirements for LCM procedures.

Agreement (RAN4 #111): - RAN4 considers defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning), and candidates are: - Model identification - Selection - Activation - Deactivation - Switching - fallback to non-AI operation - performance monitoring - Others are not precluded

In our understanding, RRM core requirements for LCM procedures is mainly to define delay requirements with specified signaling. Although the signaling aspects of LCM procedures has not been finalized in other WGs, RAN4 can initiate the discussion on delay components of these procedures. For example, model switching delay is analogous to active BWP switch delay, thus the delay components may include, such as

- UE processing time of the signaling with model switching command

- UE processing time of switching model

- UE processing time of transmitting UL acknowledgement, if any

- Signaling of RRC-reconfiguration for the target model, if any

- Whether UE has the associated ID and/or other additional conditions for the target model.

Proposal 4: RAN4 to discuss the delay components of LCM procedures, which may include such as,

- UE processing time of the signaling with model switching command

- UE processing time of switching model

- UE processing time of transmitting UL acknowledgement, if any

- Signaling of RRC-reconfiguration for the target model, if any

- Whether UE has the associated ID and/or other additional conditions for the target model.

2.2 Issue: Absolute RSRP accuracy

In RAN4 #112bis meeting, we had following agreement on definition of absolute RSRP accuracy.

Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

In current tests of L1-RSRP measurement accuracy in specification, the power level of beams and channel conditions are specified, such that the benchmark of measured value is deterministic. With the deterministic benchmark value, RAN4 is able to define a range of reporting that could meet the accuracy requirements. Based on our understanding, the part “ground-truth of L1-RSRP of beam index i” in the definition of absolute RSRP accuracy serves the same as benchmark value. Therefore, the measurement under certain high SINR level for ground-truth shall meet a tight accuracy requirement compared to legacy.

Proposal 5: Suggest to discuss the feasibility of defining a tight requirement for measurement used to generate ground truth and how to verify the measurement performance.

2.3 Issue: Relative RSRP accuracy

Issue 3-1: Relative RSRP Accuracy - Proposals - Option 1: The definition of relative RSRP accuracy shall be: The relative RSRP accuracy for AI/ML-based beam prediction = the ground truth of L1-RSRP of the predicted beam index i - the ground truth of the L1-RSRP of the Top-1 ideal beam index j - Option 2: The definition of relative RSRP accuracy shall be: predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest RSRP among the predicted beams. - Option 3: The definition of relative RSRP accuracy shall be: the RSRP difference between the predicted L1-RSRP of different beams. - Option 4: others - Recommended WF - Option 1

It is essential firstly to identify the motivation of defining relative RSRP accuracy. In our understanding, for models that are able to predict beam index and RSRP, absolute RSRP accuracy is adequate to verify the mode performance, considering RSRP prediction accuracy and beam prediction accuracy are positively correlated. A more accurate prediction of RSRP value is more likely to induce an accurate prediction of beam index.

While for the models that only predict beam indices, the critical point is whether beam prediction accuracy, proposed under “issue: Metrics/KPIs for beam prediction”, is adequate to verify the performance or not. We understand this may relate to the design of beam reporting from UE and subsequent NW’s reaction:

- UE reports Top-K beams with strongest beam index (if specified),

  - NW directly chooses the strongest beam based on UE reporting: the holistic performance deeply relies on the accuracy of beam prediction. For example, in figure 1, UE measures 3 beams and reports Top-2 (K=2) beams. The solid bar indicates the ground truth value and dash bar indicates the ranking marks inside the model of each predicted beam. Base on prediction results, UE would report beam #1 and #2 to NW and NW shall choose beam #1 for DL transmission. It can be seen that though the prediction satisfies Top-K/1 and Top-1/K metrics, the actual RSRP gap is large between the two beams and thus a degraded performance would be expected. Consequently, it is better to have relative RSRP accuracy requirement in addition to beam prediction accuracy. The definition of relative RSRP accuracy can go with option 1 in the proposal.

  - NW randomly chooses one out of the K beams: the randomness diminishes the dependency on UE reporting, but NW still possibly falls into an inferior beam, and the probability grows with a larger reported value K. From this perspective, it would also be beneficial to have relative RSRP accuracy requirement to guarantee a set of sufficient high-quality candidate beams.

  - NW initiates another round of beam measurement for reporting: NW’s decision on DL transmission beam depends on another round of beam measurement reporting, thus beam prediction accuracy together with measurement RSRP accuracy are adequate to verify the performance.

- UE reports Top-K beams without sorting

  - NW randomly chooses one out of the K beams: similar to case 1.b.

  - NW initiates another round of beam measurement for reporting: similar to case 1.c.

Figure 1 Illustrative example of beam prediction

Overall, relative RSRP accuracy shows the benefit for models that only output predicted beam indices, and shall be defined as option 1.

The definition in option 2 and 3 requires clarification before further decision: Option 3 follows the definition of relative RSRP accuracy in legacy specification with replacing measurement by prediction, as copied below. The vague part of option 2 is “owns largest RSRP”, whether it is largest measurement value or largest predicted value. If taking predicted value, option 2 has no much difference with option 3. If taking measurement value, how the metric promotes the performance is unclear as shown in the example.

Observation 1: Relative RSRP accuracy shows the benefit for models of beam prediction that only output beam indices.

Observation 2: Definitions in option 2 and 3 require more clarification before further decision.

Proposal 6: Suggest to specify relative RSRP accuracy requirement with the definition in option 1.

Conclusion

In this contribution, RRM core requirements for beam management are discussed with following proposals:

Proposal 1: The principle to define the measurement period TL1-RSRP_Measurement_Period_SSB for FR2 in current specification TS 38.133 could be reused. The detailed values of {M, N, P} needs further study.

Proposal 2: RAN4 to identify the impact of reduced resource configuration on existing RAN4 requirements with L1-RSRP related measurements. Table 1 could be considered as starting point.

Table 1

| RRM Req. Category | RRM procedure |
| --- | --- |
| Signaling characteristics | SSB /CSI-RS based Radio Link Monitoring |
|  | Link recovery procedures (SSB/CSI-RS based beam failure indication, L1 indication, Candidate beam detection) |
|  | Active TCI state switching delay |
|  | Uplink spatial relation switch delay |
|  | Pathloss reference signal switching delay |
|  | Active downlink TCI switching delay for unified TCI |
|  | Active uplink TCI switching delay for unified TCI |
| Measurement procedure | NR intra-frequency measurements (Number of cells and number of SSB, measurement reporting requirements, etc) |
|  | L1-RSRP measurements for reporting |
| Measurement performance requirements – NR measurements | Intra-frequency RSRP accuracy requirements for FR1/FR2 |
|  | RSRP measurement report mapping |
|  | L1-RSRP accuracy requirements for FR1/FR2 |

Proposal 3: Postpone discussion on reporting related requirements until other WGs define detailed reporting mechanism.

Proposal 4: RAN4 to discuss the delay components of LCM procedures, which may include such as,

- UE processing time of the signaling with model switching command

- UE processing time of switching model

- UE processing time of transmitting UL acknowledgement, if any

- Signaling of RRC-reconfiguration for the target model, if any

- Whether UE has the associated ID and/or other additional conditions for the target model.

Proposal 5: Suggest to discuss the feasibility of defining a tight requirement for measurement used to generate ground truth and how to verify the measurement performance.

Observation 1: Relative RSRP accuracy shows the benefit for models of beam prediction that only output beam indices.

Observation 2: Definitions in option 2 and 3 require more clarification before further decision.

Proposal 6: Suggest to specify relative RSRP accuracy requirement with the definition in option 1.