3GPP TSG-RAN WG4 Meeting #114	R4-2500514

Athens, Greece, 17th – 21th February, 2025

Agenda Item:	7.19.4

Source:	CAICT

Title:	Discussion on RRM core requirements for positioning

Document for:	Discussion

1	Introduction

This contribution provides discussion on following issue from last meeting.

- Issue: Requirements for case 1

2	Discussion

2.1 Issue: Requirements for case 1

In RAN4 #112bis, the group had the following agreement on this issue.

Issue 3-1: Requirements for case 1 Agreement: Companies are invited to bring further analysis on feasibility of defining requirements for case 1 and how to define requirements, if feasible

The definition of case 1 excerpted from TR38.843

- Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

Case 1, AI/ML assisted positioning: The model is implemented inside the UE and the model output is intermediate results that are invisible to outside.  In our understanding, there is no need to define requirement for this case.

Case 1, direct positioning: this is the focusing case of discussion in WGs. The model output is explicit UE locations, of which corresponding requirements are absent in current specification. The critical aspect in specifying tests and requirements of case 1 is test setup.

Existing tests on positioning, for example TS37.571-1, is emulation-based, i.e., TE vendor implements certain channel model/propagation condition in channel emulator to simulate the environment. Some concerns arise on adopting this manner for direct position accuracy test:

- Existing tests assume at most 2 transmission points and utilize simple channel conditions, such as AWGN channel or a two-tap propagation model. How to emulate sophisticated propagation conditions at different UE locations under multi-TRP scenario (typical assumption is 18 sites in RAN1 evaluation) requires study on feasibility and complexity.

- The discrepancy between emulated environment and realistic deployment, may induce the DUT performing well in the test but poorly in the field.

Observation 1: Emulation-based tests for case 1 face issues, such as emulating sophisticated propagation conditions at different UE locations under multi-TRP scenario, discrepancy between emulated environment and realistic deployment, and etc.

An alternative of test setup is to build a miniature testing site. Attempting this new type of test also face some issues,

- How many and which subset of testing scenarios are going to be implemented in the miniature site?

- What is the scale of such miniature site comparing to real deployment scenario that would be able to verify the performance?

- How DUT to obtain the data for model training, via simulation or up to implementation or collected from the site and provided by TE?

- Last but not least, expenditure on building and maintaining such site.

Observation 2: Site-based tests for case 1 face issues, such as selection of scenarios, scale of site, training data collection, expenditure, and etc.

Overall, possible approaches and feasibility of each shall be firstly discussed in RAN4 before further decision on specifying requirements for case 1.

Proposal 1: Suggest to firstly discuss possible approaches (e.g., emulation-based, miniature testing site, etc.) and feasibility of each in RAN4 before further decision on specifying requirement for case1.

Conclusion

In this contribution, testability and interoperability issues for positioning are discussed with following observations and proposal:

Observation 1: Emulation-based tests for case 1 face issues, such as emulating sophisticated propagation conditions at different UE locations under multi-TRP scenario, discrepancy between emulated environment and realistic deployment, and etc.

Observation 2: Site-based tests for case 1 face issues, such as selection of scenarios, scale of site, training data collection, expenditure, and etc.

Proposal 1: Suggest to firstly discuss possible approaches (e.g., emulation-based, miniature testing site, etc.) and feasibility of each in RAN4 before further decision on specifying requirement for case1.