3GPP TSG-RAN WG4 Meeting #114	R4-2500592

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.17.2.1

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

During last meeting, RAN4 reached some agreements, but there are still many open issues and it is summarized in WF [1]. This document focus on the beam sweeping factor reduction and proposes possible enhancements.

- Discussion

  - Conditions to quit optimizing Rx BSF

In the last meeting, companies proposed what should be the conditions to quit optimizing Rx BSF. Since proposals are quite diverse, it needs more discussion to reach the agreement. The summary is copied below.

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” : - Option 1 (NTT DCM): - For scenario 1/2/3/4, Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point. However, the actual trigger to quit optimizing Rx BSF should be done by NW. UE autonomous quitting should be avoided. - For scenario 5/7, it should be disallowed that the fallback of UE Rx beam sweeping factor to the existing N value until the signalled configuration is no longer in effect or the relevant task has been completed.. - Option 2 (LGE, OPPO, Ericsson, MTK, CTC): UE indication to the NW when UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time). - Option 2a (LGE, OPPO, NTT DCM(scenario 1/2/3/4))Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point. - Option 3 (HW, Samsung): Apart from the threshold conditions, UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results, since NW already has the candidate target cells for potential HO or SCell modification. - Option 4 (vivo): RAN4 to consider additional conditions for triggering FBS for L3 measurement for power consumption issue, including active condition and de-active conditions. - Option 5 (Ericsson): RAN4 to study the below FBS deactivation conditions: - After the UE have activated FBS: - The condition of activating FBS is invalid. - Define a minimal during time for FBS/non-FBS. - The condition of activating FBS still is valid but, - L3 measurement with FBS has been conducted for a time period with respect to a pre-defined measurement period or time. - Internal overheating is detected or other limitation from UE perspective. (similar as option 2) - Meet activation conditions but not activate FBS yet: - Internal overheating is detected or other limitation from UE perspective. (similar as option 2) - UE shall report NW of deactivating FBS once the deactivation condition is met. - Option 6 (Samsung): Introduce NW based solution to enable the fallback for UE FBS based quick L3 measurement when it is off - Dynamic indication; Semi-static configuration - Option 7 (Samsung, Ericsson): - Introduce validity timer configuration with certain ranges via dedicated RRC signalling - Option 8 (Samsung): - Introduce/Define TTT parameters for UE using FBS (i.e., N=[reduced number]) based L3 measurement event - Option 9 (Nokia): - When L3 event-triggered reporting conditions are used as a trigger to start FBS (e.g. event A2), UE is allowed to return back to normal beam sweeping when event condition (e.g. A2) is no longer met, or when another event condition (e.g. A1) is met, depending on network configuration. - Recommended WF [Moderator]: discuss the following options(wording can be revised/polished) and companies can add more details if needed. For conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”: - Option 2 (LGE, OPPO, Ericsson, MTK, CTC): UE indication to the NW when UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time). - Option 2a (LGE, OPPO, NTT DCM(scenario 1/2/3/4))Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point. - Option 3 (HW, Samsung, Ericsson): Apart from the threshold conditions, UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results, since NW already has the candidate target cells for potential HO or SCell modification. - Option 7 (Samsung, Ericsson): Introduce validity timer configuration with certain ranges via dedicated RRC signalling

As same as proposals in last meeting, for scenario 1/2/3/4, Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point. However, the actual trigger to quit optimizing Rx BSF should be done by NW. UE autonomous quitting should be avoided. For scenario 5/7, it should be disallowed that the fallback of UE Rx beam sweeping factor to the existing N value until the signalled configuration is no longer in effect or the relevant task has been completed.

Proposal 1: UE autonomous quitting should be avoided since it may cause inconsistency between NW and UE.

Proposal 2: Only the UE indication due to overheating can be considered for scenario 1/2/3/4 based on Rel-18 UAI ‘multiRx-PreferenceFR2’.

  - SSB processing delay

In the last meeting, SSB processing delay/time was discussed. The summary is copied below.

Issue 1-1-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC Agreement: - For scenario 1/2/3/4 - additional processing time can be added after the last SMTC periodicity for intra/inter-frequency L3 measurement. - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms - where TSSB_processing = 2 ms Agreement: - For scenario 7 - to consider an additional processing delay (2ms) after the last SMTC periodicity [Wayforward] - For scenario 5 - FFS: - Option 1: to consider an additional processing delay (2ms) in the handover delay equation - Option 2: In the handover delay equation, SSB processing time (2ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - Option 3: no need to add additional processing time in the delay requirement.

The necessity of additional SSB processing delay/time is concluded that the delay of 2ms is needed for scenario 1/2/3/4 and 7. However for scenario 5, it needs more discussion. The reason why additional SSB processing delay is needed is the number of searchers assumption. Since RAN4 agreed that 1 searcher is assumed for multi-Rx based L3 measurement, the additional SSB processing delay to identify the cell was introduced for scenario 1/2/3/4 and 7.

Observation 1: Since RAN4 agreed that 1 searcher is assumed for multi-Rx based L3 measurement, the additional SSB processing delay to identify the cell was introduced for scenario 1/2/3/4 and 7.

Considering scenario 5, the situation is not same. During HO, the target cell is already determined, thus there is no searcher problem under multi-Rx L3 measurement. In addition, Tmargin originally included in HO delay is the time for timing tracking. This delay term is also not related to multi-Rx L3 measurement.

Observation 2: During HO, the target cell is already determined, thus there is no searcher problem under multi-Rx L3 measurement.

Based on this observation, the additional delay for scenario 5 is not needed.

Proposal 2: No need to add additional processing time in the delay requirement for scenario 5.

  - Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

In the last meeting, solutions to apply/specify L3 measurement delay reduction could not be discussed due to time limitation. We propose same proposal for discussion starting point.

Proposal 3: For each scenario, the highlighted factors should be replaced with capability value.

| Scenarios | Factors |
| --- | --- |
| 1 | Mpss/sss_sync_w/o_gaps =24 = 8*3 Mmeas_period_w/o_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6 |
| 2 | Mpss/sss_sync_with_gaps =24 = 8*3 Mmeas_period_with_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6 |
| 3 | Mpss/sss_sync_inter = 40 = 8*5 Mmeas_period_inter =40 = 8*5 MSSB_index_inter = 24 = 8*3 |
| 4 | Same as scenario 3 |
| 5 | If the target cell is unknown, and the target cell Es/Iot≥-2 dB, then Tsearch = N* Trs ms. Where N = 8 when the target cell is in FR2-1. |
| 7 | RRC Re-establishment 80, 3520, 4000 come from 8*10, 8*22*20, 8*25*20 respectively Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); MAX (800 ms, 10 x TSMTC). ≥ -8; FR2-1; N/A; MAX (1000 ms, 80 x TSMTC)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 120 x TSMTC)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 3520Note1. < -8; FR2-2; N/A; 5280Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB.. Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC, i); MAX (800 ms, 13 x TSMTC, i). ≥ -8; FR2-1; N/A; MAX (1000 ms, 104 x TSMTC, i)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 156 x TSMTC, i)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 4000Note1. < -8; FR2-2; N/A; 6000 Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB.. RRC Connection Release with Redirection Table 6.2.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR FR of target NR cell; Tidentify-NR. FR1; MAX (680 ms, 11 x Trs). FR2-1; MAX (880 ms, 8x11 x Trs). FR2-2; MAX (880 ms, 12x11 x Trs). Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.. |

  - Requirement at transition period

In the last meeting, necessity of the requirement at transition between the normal L3 measurement and FBS L3 measurement was discussed. The summary is copied below.

Issue 1-2-3: FFS: requirement at transition between the normal L3 measurement and FBS L3 measurement - Option 1 (CATT, Apple): - Do not define the requirement at transition between the normal L3 measurement and FBS L3 measurement. - Option 2 (Ericsson): Regarding the transition time, - For the case from non-FBS to FBS, RAN4 specify a delay from the time instant when the condition is met. - FBS activation delay shall at the least includes below items. - When the UE receives a RRC message implying FBS the UE shall be ready to start the L3 measurements with FBS operation within DFBS seconds from the end of the last TTI containing the RRC command. DFBS = TRRC + TEvent_FBS + Tmeasure + Tprocessing_FBS - Where: - TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2]. - TEvent_FBS is the delay uncertainty which is the time from when the UE successfully decodes the RRC command until a condition exists at the measurement reference point which will trigger the FBS. - Tprocessing_FBS is for UE processing and preparing FBS operation before UE applying FBS. - For the case from FBS to non-FBS, the necessity of a delay during transition is unclear since the fallback solution isn’t fixed. - Proposal (ZTE): - The requirement at transition between the normal L3 measurement and FBS L3 measurement should be identified. There are two options: - Option 1: Reuse similar solution as the requirements at transition between measurement within gap and measurement outside gap, i.e. the longer delay is the period when transition happens. - Option 2: Once the transition happens, apply the requirements after the transition, no matter it is the normal L3 measurement or the FBS L3 measurement.

In Rel-18 multi-Rx requirement, there is no transition requirement. In addition, based on previous agreement, FBS L3 measurement is triggered by serving cell signal quality, thus UE can use sufficient time for transition between signal quality measurement. Thus, no requirement is needed.

Proposal 4: Do not define the requirement at transition between the normal L3 measurement and FBS L3 measurement.

- Conclusion

In this contribution, we propose possible enhancements.

Proposal 1: UE autonomous quitting should be avoided since it may cause inconsistency between NW and UE.

Proposal 2: Only the UE indication due to overheating can be considered for scenario 1/2/3/4 based on Rel-18 UAI ‘multiRx-PreferenceFR2’.

Observation 1: Since RAN4 agreed that 1 searcher is assumed for multi-Rx based L3 measurement, the additional SSB processing delay to identify the cell was introduced for scenario 1/2/3/4 and 7.

Observation 2: During HO, the target cell is already determined, thus there is no searcher problem under multi-Rx L3 measurement.

Proposal 2: No need to add additional processing time in the delay requirement for scenario 5.

Proposal 3: For each scenario, the highlighted factors should be replaced with capability value.

| Scenarios | Factors |
| --- | --- |
| 1 | Mpss/sss_sync_w/o_gaps =24 = 8*3 Mmeas_period_w/o_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6 |
| 2 | Mpss/sss_sync_with_gaps =24 = 8*3 Mmeas_period_with_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6 |
| 3 | Mpss/sss_sync_inter = 40 = 8*5 Mmeas_period_inter =40 = 8*5 MSSB_index_inter = 24 = 8*3 |
| 4 | Same as scenario 3 |
| 5 | If the target cell is unknown, and the target cell Es/Iot≥-2 dB, then Tsearch = N* Trs ms. Where N = 8 when the target cell is in FR2-1. |
| 7 | RRC Re-establishment 80, 3520, 4000 come from 8*10, 8*22*20, 8*25*20 respectively Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); MAX (800 ms, 10 x TSMTC). ≥ -8; FR2-1; N/A; MAX (1000 ms, 80 x TSMTC)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 120 x TSMTC)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 3520Note1. < -8; FR2-2; N/A; 5280Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB.. Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC, i); MAX (800 ms, 13 x TSMTC, i). ≥ -8; FR2-1; N/A; MAX (1000 ms, 104 x TSMTC, i)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 156 x TSMTC, i)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 4000Note1. < -8; FR2-2; N/A; 6000 Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB.. RRC Connection Release with Redirection Table 6.2.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR FR of target NR cell; Tidentify-NR. FR1; MAX (680 ms, 11 x Trs). FR2-1; MAX (880 ms, 8x11 x Trs). FR2-2; MAX (880 ms, 12x11 x Trs). Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.. |

Proposal 4: Do not define the requirement at transition between the normal L3 measurement and FBS L3 measurement.

References

[1] 3GPP, R4-2420093, “WF on RRM requirements for NR_RRM_Ph5_Part1”, Apple