3GPP TSG-RAN WG4 Meeting #114	R4-2500593

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.19.3

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on RRM core requirement for beam management

Document for:	Discussion

- Introduction

During the past meetings, requirements for beam management has been discussed and several agreements were obtained. This paper further discusses remaining issues in the summary [1].

- Discussion

  - Beam prediction KPIs

During past meetings, the metrics/KPIs for beam prediction has been extensively discussed. However no agreements have been obtained. The candidate proposals are copied in below.

Beam prediction KPIs Different metrics/KPIs have been discussed and were captured in the TR: For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy -Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" -Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" -Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, -Related measurement accuracy can be considered to determine x - Option 4: combinations of above options Issue 3-4: Metrics/KPIs for beam prediction Proposals - Option 1: Use Option 1 - Option 2: Use Option 2 - Option 3: Use combined Option 2 and 3: - Option-New: The successful rate for the correct prediction which is considered as "the Top-1 predicted beam is one of the Top-K strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB” - FFS: K = 2 and x = 3 as baseline - Option 4: Add new metric: - Top-K/N (%): the percentage of "the Top-N genie-aided beam is one of the Top-K predicted beams". FFS how N is derived. - Option 5: Add a requirement based on relative difference between predictions under different conditions/inputs (for example varying signal levels by N dBs but maintain same relative difference). - consider defining multiple tests under different conditions but with the same outcome(same predicted beam) - Option 6: both RSRP accuracy and beam prediction accuracy are used as KPIs for beam prediction - RSRP absolute accuracy applies to all predicted beams Relative RSRP accuracy Issue 3-1: Relative RSRP Accuracy - Proposals - Option 1: The definition of relative RSRP accuracy shall be: The relative RSRP accuracy for AI/ML-based beam prediction = the ground truth of L1-RSRP of the predicted beam index i - the ground truth of the L1-RSRP of the Top-1 ideal beam index j - Option 2: The definition of relative RSRP accuracy shall be: predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest RSRP among the predicted beams. - Option 3: The definition of relative RSRP accuracy shall be: the RSRP difference between the predicted L1-RSRP of different beams. - Option 4: others

The key point to determine KPIs for AI/ML based beam management is what is the goal and what should be satisfied for the goal. The goal of AI/ML based beam management is to predict appropriate beam through RSRP value prediction. In order to satisfy this goal, the requirements can be defined by two factors, the selection probability of appropriate beam and predicted RSRP value accuracy of selected beam.

Observation 1: The requirements can be defined by two factors, the selection probability of appropriate beam and predicted RSRP value accuracy of selected beam.

The former requirement can be satisfied following metric proposed during last meeting.

- Option-New: The successful rate for the correct prediction which is considered as "the Top-1 predicted beam is one of the Top-K strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB”

  - FFS: K = 2 and x = 3 as baseline

In order to guarantee optimality of the selected beam, K should be 2 and x should be less than 3. Under this condition, if the ground truth of top 1 predicted beam is not the strongest beam, it can be used as appropriate beam for the UE.

Figure 1: Relationship between ground truth and predicted value

If only this KPI is used, overestimation cannot be avoided shown in the figure below.

Figure 2: Prediction reliability issue

If absolute RSRP accuracy is introduced, this issue can be resolved. The absolute RSRP accuracy in this context means the acceptable difference between predicted RSRP value and ground truth of the selected beam. The existing absolute accuracy requirement can be the starting point for the absolute predicted RSRP accuracy. Figure 3 shows overall picture.

Figure 3: Proposal for beam prediction KPIs

In summary, we propose followings for beam prediction KPIs.

Observation 1: The requirements can be defined by two factors, the selection probability of appropriate beam and predicted RSRP value accuracy of selected beam.

Proposal 1: Prediction accuracy and absolute predicted L1-RSRP accuracy shall be used for beam prediction KPIs. The definitions are as follows.

- Prediction accuracy: the Top-1 predicted beam is one of the Top-2 strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB. x shall be less than 3.

- Absolute predicted L1-RSRP accuracy: the acceptable difference between predicted RSRP value and ground truth of the selected beam. The existing absolute accuracy requirement can be the starting point.

- Conclusion

In this contribution, we proposed our views on KPIs for beam prediction.

Observation 1: The requirements can be defined by two factors, the selection probability of appropriate beam and predicted RSRP value accuracy of selected beam.

Proposal 1: Prediction accuracy and absolute predicted L1-RSRP accuracy shall be used for beam prediction KPIs. The definitions are as follows.

- Prediction accuracy: the Top-1 predicted beam is one of the Top-2 strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB. x shall be less than 3.

- Absolute predicted L1-RSRP accuracy: the acceptable difference between predicted RSRP value and ground truth of the selected beam. The existing absolute accuracy requirement can be the starting point.

References

[1] 3GPP, R4-2418151, “Topic summary for [113][129] NR_AIML_air”, Qualcomm Incorporated