3GPP TSG-RAN WG4 Meeting #114	R4-2500594

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.21.2

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on RAN4 requirement impacts of AIML for mobility

Document for:	Discussion

- Introduction

In the last meeting, AI/ML for mobility was discussed and open issue was summarized in summary [1]. According to this summary, this paper proposes possible RAN4 requirement impacts.

- Discussion

During previous meeting, the issue related to cell identification could not be discussed due to time limitation. According to RAN2 assumptions, the prediction is only for obtaining the cell quality. However, RAN4 requirements for intra/inter-frequency measurement, cell identification is needed and it requires PSS/SSS sync and SSB time index acquisition.

Observation 1: RAN4 requirements for intra/inter-frequency measurement, cell identification is needed and it requires PSS/SSS sync and SSB time index acquisition.

For scenario 2 and 4, since this is intra-frequency prediction, PSS/SSS sync may be achieved by prediction. However in sub-use case 2, it needs further study because the input is L3 cell-level measurement results. For Scenario 3, since no target cell measurement is conducted, RAN4 should discuss the feasibility on the cell detectability for all sub-use cases. The feasibility of SSB time index acquisition is need more discussions for all the scenarios.

Proposal 1: RAN4 needs study the impacts on cell identification for L3 cell-level measurement prediction by AI/ML.

Proposal 2: For PSS/SSS sync,

- In scenario 2 and 4, it may be achieved by prediction. However in sub-use case 2, it needs further study because the input is L3 cell-level measurement results.

- In scenario 3, since no target cell measurement is conducted, RAN4 should discuss the feasibility on the cell detectability for all sub-use cases.

Proposal 3: For SSB time index acquisition is need more discussions for all the scenarios.

- Conclusion

In this contribution, we proposed our views on RAN4 requirement impacts of AI/ML for mobility.

Observation 1: RAN4 requirements for intra/inter-frequency measurement, cell identification is needed and it requires PSS/SSS sync and SSB time index acquisition.

Proposal 1: RAN4 needs study the impacts on cell identification for L3 cell-level measurement prediction by AI/ML.

Proposal 2: For PSS/SSS sync,

- In scenario 2 and 4, it may be achieved by prediction. However in sub-use case 2, it needs further study because the input is L3 cell-level measurement results.

- In scenario 3, since no target cell measurement is conducted, RAN4 should discuss the feasibility on the cell detectability for all sub-use cases.

Proposal 3: For SSB time index acquisition is need more discussions for all the scenarios.

References

[1] 3GPP, R4-2418278, “Topic summary for [213][220] FS_NR_AIML_Mob”, Nokia