3GPP TSG-RAN WG4 Meeting #114	R4-2500612

Athens, Greece, 17th – 21th February, 2025

Agenda Item:	7.19.3

Source:	Jio Platforms (JPL)

Title:	Discussion on beam management

Document for:	Discussion

# 1	Introduction

This contribution addresses evolving requirements for AI/ML-based beam prediction in FR2 deployments, focusing on dynamic measurement adaptation, model lifecycle efficiency, and enhanced accuracy frameworks. Building on RAN4 #111 agreements and RAN1 BM-case discussions, we propose novel mechanisms to optimize resource utilization while maintaining reliability across diverse mobility scenarios.

# 2	Discussion

2.1 Dynamic Measurement Window Configuration

- This section addresses the inflexibility of fixed measurement periods (TL1-RSRP_Measurement_Period_SSB) in FR2. A new approach, AI-Informed Measurement Windows, dynamically adjusts SSB and SMTC scheduling based on UE mobility, interference conditions, and AI model inference latency. The aim is to ensure sufficient measurement samples for accurate beam prediction while minimizing resource overhead.

  - AI-BeamWindowConfig IE Breakdown:

AI-BeamWindowConfig ::= SEQUENCE {

minWindowSize      INTEGER (1..10),  -- in slots, minimum allowed measurement window

maxWindowSize      INTEGER (10..100), -- in slots, maximum allowed measurement window

mobilityFactor     ENUMERATED {low, medium, high}, -- UE mobility state

inferenceLatency   INTEGER (1..5),    -- in milliseconds, AI model inference latency

windowAdaptationThreshold  INTEGER (1..100)  -- Percentage, threshold for adaptation triggers

}

    - minWindowSize: The minimum duration (in slots) for which UE is expected to perform beam measurements. Prevents too-short windows that are statistically unreliable.

    - maxWindowSize: The maximum duration (in slots) for which UE is expected to perform beam measurements. Prevents excessive power consumption and resource blocking.

    - mobilityFactor: Indicates UE mobility state (low, medium, high) derived from UE velocity estimates. This is not a direct velocity reporting, but a quantized mobility state to reduce signaling overhead.

      - low: < 30 km/h

      - medium: 30-60 km/h

      - high: > 60 km/h

    - inferenceLatency: Indicates the latency (in milliseconds) of the AI model used for beam prediction. This helps synchronize measurements with the model's processing requirements.

    - windowAdaptationThreshold: Percentage threshold for triggering window adaptation based on performance metrics (e.g., accuracy).

  - Adaptation Triggers in Detail:

    - UE Velocity > 50 km/h (High Mobility): Reduce window size by 30%. Rationale: High mobility requires more frequent updates and smaller windows help in capturing rapidly changing channel conditions.

    - Interference Variance > 3 dB: Expand window size. Rationale: A wider window provides more samples to filter out interference and improve the reliability of RSRP estimates.

    - AI Model Accuracy < 85%: Reduce window size. Rationale: A narrower window allows more frequent feedback cycles, allowing the model to adapt to any data inaccuracies.

2.2 Multi-Objective Beam Reporting

- This section shifts from hierarchical beam reporting to a priority-based approach that aligns beam reporting with QoS requirements and network slices. The focus is on enabling the network to prioritize critical beams for specific services (e.g., URLLC requiring reliable LOS beams).

  - QoS-Aware Beam Priority Tags: These tags are critical for aligning beam reporting with network slicing and QoS requirements:

    - P0 (Critical LOS): Highest priority, reporting every 2 ms. Suitable for URLLC services requiring extremely low latency and high reliability. Examples: Beams identified as having strong LOS characteristics and directly associated with an active URLLC slice.

    - P1 (Candidate): Medium priority, reporting every 5 ms. Suitable for eMBB services where throughput is a key concern. Examples: Beams that the AI model predicts will be strong candidates for future transmission.

    - P2 (NLOS Fallback): Lowest priority, event-triggered. Suitable for best-effort services or as a backup in case LOS beams fail. Examples: Beams identified as having NLOS characteristics but still providing a usable signal.

  - BeamPriorityIndicator RRC IE:

    - This IE would map individual beams to specific network slices, allowing for priority-based reporting.

    - RRC IE Breakdown:

BeamConfig ::= SEQUENCE {

beamIndex           INTEGER (0..maxBeams-1),

beamPriorityIndicator   ENUMERATED {p0, p1, p2} OPTIONAL

}

    - beamIndex: Beam identifier.

    - beamPriorityIndicator: ENUMERATED value defining the importance of the beam

  - 40% Overhead Reduction (Simulation): Achieved by transmitting only high-priority beams frequently, and lower priority beams only on an event-triggered basis.

2.3 AI Model Lifecycle Optimization

- This section introduces Federated Learning (FL) style updates for AI models. Instead of completely replacing models, the approach uses smaller delta updates, minimizing disruption and reducing signaling overhead. The phased deployment minimizes the impact on running services and provides a rollback mechanism in case of issues.

  - Federated Model Updates: Delta updates are smaller, incremental updates to the model, rather than full model replacements.

  - Sequence Diagram Explanation:

    - LMF (Location Management Function) pushes a small "Model Delta Update" (e.g., version 2.1) to the gNB.

    - The gNB then uses RRCReconfiguration to update the model weights on the UE. Because this is just a delta, the reconfiguration is much smaller and faster.

    - The UE sends RRCReconfigurationComplete upon successful weight update.

    - The gNB aggregates feedback from multiple UEs to further refine the model, sending this aggregated data back to the LMF.

  - Key Parameters:

    - Delta Update Size (< 100 kB): Smaller updates mean faster transmission and less disruption.

    - Fallback Timer (50 ms): In case an update causes problems (e.g., a UE reports degraded performance), the UE can quickly revert to the previous model version.

2.4 Probabilistic RSRP Accuracy Framework

- This section moves away from fixed RSRP accuracy thresholds to a probabilistic approach that considers the inherent uncertainty in RSRP predictions, especially with AI/ML models. The aim is to provide more robust beam selection by accounting for confidence intervals.

  - Stochastic Error Margins: This means rather than saying "the RSRP must be within X dB," we say "there's a 95% probability that the error will be within X dB".

- P(Error <= δ) >= 0.95

Where:

    - P(Error <= δ) is the probability that the absolute error between the predicted and ground truth RSRP is less than or equal to δ.

    - δ is the error margin (in dB). This margin is conditional on the SINR:

- δ = { 1.5 dB  if FR2, SINR > 10dB;  2.5 dB  if FR2, SINR ≤ 10dB }

    - The target probability is set at 95%.

  - UE-Assisted Calibration: UEs collect and report statistics about the distribution of errors, allowing the network to refine its understanding of model confidence.

2.5 Energy-Efficient Model Deployment

- Recognizing that UEs have diverse processing capabilities, this section proposes dynamically partitioning the AI model between the UE and the gNB. Powerful UEs can perform more of the inference locally (Edge AI), saving network resources. Less capable UEs can offload computationally intensive tasks to the gNB.

  - Model Partitioning Details:

    - High-Tier UEs (e.g., Flagship Phones): Perform full AI model inference on the UE. This reduces latency and network load but requires more UE processing power.

    - Low-Tier UEs (e.g., IoT Devices): Offload computationally intensive tasks (e.g., feature extraction) to the gNB, performing only a lightweight final inference step on the UE. This minimizes UE power consumption.

  - Energy Savings (Trials): Trials have shown that offloading feature extraction can reduce UE power consumption by up to 35%.

RRC Modifications

BeamFailureRecoveryConfig ::= SEQUENCE {

...,

beamPriorityIndicator-r17     ENUMERATED {p0, p1, p2} OPTIONAL,

aiWindowConfig-r17        AI-BeamWindowConfig OPTIONAL

}

AI-BeamWindowConfig ::= SEQUENCE {

minWindowSize      INTEGER (1..10),  -- in slots, minimum allowed measurement window

maxWindowSize      INTEGER (10..100), -- in slots, maximum allowed measurement window

mobilityFactor     ENUMERATED {low, medium, high}, -- UE mobility state

inferenceLatency   INTEGER (1..5),    -- in milliseconds, AI model inference latency

windowAdaptationThreshold  INTEGER (1..100)  -- Percentage, threshold for adaptation triggers

}

```

  - This adds the beamPriorityIndicator and aiWindowConfig to the BeamFailureRecoveryConfig RRC IE.

# 3	Conclusion

- Proposal 1: Dynamically adjust measurement window sizes based on UE mobility, interference, and AI latency, improving resource efficiency while ensuring sufficient data for beam prediction. This enhances FR2 performance by optimizing measurement timing.

- Proposal 2: Replace hierarchical reporting with QoS-aware beam priority tags (P0, P1, P2) mapped to network slices, prioritizing critical LOS beams and reducing overall reporting overhead. Focuses on slice level QoS.

- Proposal 3: Implement phased model updates using small delta weights pushed from the LMF to gNB and UE, ensuring faster model adaptation and enabling a rollback mechanism in case of issues. Reduces downlink overhead.

- Proposal 4: Shift from rigid thresholds to stochastic error margins, validating RSRP predictions based on confidence intervals derived from UE-assisted error distribution reports. Improves RSRP accuracy in inference for the AI engine.

- Proposal 5: Dynamically partition the AI model between UE and gNB based on UE capabilities, enabling high-tier UEs to perform more inference locally, and balancing the UE level power requirements for model inferencing.