# 3GPP TSG-RAN WG4 Meeting #114	R4-2500007

Athens, Greece, 17th – 21st Feb, 2025

3GPP TSG RAN WG1 Meeting RAN1#119 		R1-2410890

Orlando, US, November 18th – 22nd, 2024

Title:	Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation

Response to:	R1-2409352 (R4-2416919)

Release:	Rel-19

Work Item:	NR_duplex_evo-Core

Source:	RAN1

To:	RAN4

Cc:

Contact person:	Xinghua Song

songxinghua@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

When discussing L1 UE-to-UE CLI measurement reporting, RAN1 has not made a distinction between the two application scenarios mentioned in the RAN4 LS (intra-cell vs. inter-cell UE-to-UE L1 CLI measurement). L1 UE-to-UE CLI measurement and reporting specified by RAN1 can be applied for both application scenarios.

For the case of intra-cell L1 SRS-RSRP measurement and reporting, a victim UE may be configured with a set of SRS-RSRP measurement resources that overlap with the resources used by the aggressor UE(s) to transmit SRS since the gNB has knowledge the aggressor UE’s SRS configurations.

However, for inter-cell L1 SRS-RSRP measurement, SRS resource set configurations may need to be exchanged among gNBs. But specification impact of this in Xn/F1 has been excluded according the following note from the WID

  - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs

From RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement.

# 2	Actions

To RAN4

ACTION: RAN1 respectfully asks RAN4 to take the above reply into account in their future work.

# 3	Dates of next TSG RAN WG1 meetings

RAN1#120	17th – 21st February 2025	 	Athens, Greece

RAN1#120bis	7th – 11th April 2025	 		China