TSG-RAN Working Group 4 (Radio) meeting #64                  R4-126161

New Orleans, USA, 12 – 16 Nov, 2012

Source:	ZTE

Title:	Simulation Results on Cell Identification for FeICIC

Agenda item:	6.15.1.3

Document for:	Discussion

## Introduction

In RAN4 #64bis meeting the link level simulation assumptions for cell identification with FeICIC [1] was agreed. It was expected that initial results were to be provided at this meeting.

In this contribution, we provided our simulation results of cell identification with FeICIC.

## Simulation Assumptions

Table 1 provides scenarios for link level cell identification studies.

Table 1: Scenarios for link-level cell identification studies

| Scenario name | Scenario description |
| --- | --- |
| ‘cid1’ | FDD, non-MBSFN ABS |
| ‘cid2’ | FDD, MBSFN ABS |
| ‘cid3’ | TDD, non-MBSFN ABS |
| ‘cid4’ | TDD, MBSFN ABS |

The following IC scenarios to be simulated:

- ‘IC1’: IC for the first aggressor cell,

- ‘IC1-2’: IC for both aggressor cells.

Tables 2-4 provide assumptions related to PSS/SSS.

Table 2: Link-level simulation parameters for PSS/SSS

| Parameter | Unit | Cell 1, 2 | Cell 0 |
| --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 |
| Cell type |  | Aggressor | To be identified |
| Cell Identifier |  | unknown to UE | known to UE (Rel-10) |
| Data and Control PSD relative to RS PSD | dB | 0 | 0 |
| P-SCH and S-SCH PSD relative to RS PSD | dB | 0 | 0 |
| System bandwidth | RB | 6 | 6 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Frame Structure Type | - | 1 | 1 |
|  |  | 2 | 2 |
| DRX |  | OFF | OFF |
| CP Length | - | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| Relative delay of 1st Path w.r.t. cell 0: (fixed delay) | ms | CP/2 | 0 |
| SNR | dB | SNR1 = 4 dB, SNR2 =2 dB | -4 |
| Es/Iot | dB | Calculated based on SNR levels | (Es/Iot)0=-11.07 dB |
| Number of Tx antennas | - | 1 | 1 |
| TDD Uplink-downlink configuration |  | 1 | 1 |
| TDD Special sub-frame configuration |  | 6 | 6 |
| Number of Rx antennas (uncorrelated with equal gain) |  | 2 | 2 |
| Propagation conditions |  | AWGN, EPA5, ETU30, ETU70 |  |
| Note: Timings of cell 1 and cell 2 are unknown to the UE. |  |  |  |

Table 3: SSS sequences in different cells

| case # | Cell 0 (Target cell) |  |  | Cell 1 (Strongest interferer) |  |  | Cell 2 (Weaker interferer) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | psc3 | fssc1a, fssc1b | PCI= 17 | psc3 | fssc2a, fssc1b | PCI=269 | psc1 | fssc2a, fssc1b | PCI=268 |
| 2 | psc3 | fssc1a, fssc1b | PCI= 17 | psc3 | fssc3a, fssc1b | PCI=347 | psc2 | fssc1a, fssc1b | PCI= 15 |
| 3 | psc1 | fssc1a, fssc2b | PCI=106 | psc1 | fssc4a, fssc2b | PCI=190 | psc2 | fssc3a, fssc2b | PCI=423 |
| 4 | psc3 | fssc1a, fssc2b | PCI=107 | psc3 | fssc3a, fssc2b | PCI=425 | psc3 | fssc3a, fssc1b | PCI=347 |
| 5 | psc3 | ssc3a, ssc3b | PCI=113 | psc1 | ssc1a, ssc1b | PCI=109 | psc2 | ssc2a, ssc2b |  |
| 6 | psc1 | ssc3a, ssc3b | PCI=112 | psc1 | ssc1a, ssc1b | PCI=109 | psc2 | ssc2a, ssc2b |  |
| 7 | psc1 | ssc1a, ssc3b | PCI=196 | psc1 | ssc1a, ssc1b | PCI=109 | psc2 | ssc2a, ssc2b |  |
| 8 | psc3 | ssc1a, ssc1b | PCI=110 | psc1 | ssc1a, ssc1b | PCI=109 | psc2 | ssc2a, ssc2b |  |

Table 4: PSS, SSS indices for simulations

| Label | Code index | Cell group index |
| --- | --- | --- |
| psc1 | 29 | - |
| psc2 | 25 | - |
| psc3 | 34 | - |
| (fssc1a, fssc1b) | (5, 6) | 5 |
| (fssc2a, fssc1b) | (2 6) | 89 |
| (fssc3a, fssc1b) | (1,6) | 115 |
| (fssc1a, fssc2b) | (5,7) | 35 |
| (fssc4a, fssc2b) | (4,7) | 63 |
| (fssc3a, fssc2b) | (1,7) | 141 |
| (ssc1a, ssc1b) | (6, 8) | 36 |
| (ssc2a, ssc2b) | (10, 12) | 40 |
| (ssc3a, ssc3b) | (7, 9) | 37 |
| (ssc1a, ssc3b) | (6, 9) | 65 |

## Simulation Results

The cell identification requirement to be defined by RAN4 is expected to specify the maximum cell detection time to identify an LTE cell for some given propagation conditions. The processing steps included in LTE cell identification are expected to be at least the following steps:

Step 1 - Detect and synchronise to a newly appearing LTE cell (i.e. identify cell id and frame boundary position).

Step 2 - Estimate the cell signal strength/quality the newly identified cell over the relevant measurement period (e.g. 200 ms for RSRP)

Step 3 - Perform an event-triggered RRC measurement report

Cell identification for feICIC consists of eICIC-type processing with PSS/SSS IC capability. eICIC-type cell identification can be explained in [2][3].

The cell identification duty cycle is 50%, i.e. there are 8 pairs of PSS/SSS within 40ms DRX cycle and only 4 continuous of them are used for cell identification. The 4 pairs of PSS/SSS are searched separately which means there are 4 search attempts in one DRX cycle and cell ID is considered to be identified if anyone of these four attempts is succeeded.

Table 5 – 7 shows the cell detection delay performance under EPA5, ETU30 and ETU70 channel model. L1 measurement period (200ms) is not included in the tables.

Table 5: cell detection delay under EPA5 channel model (ms)

| case # | NO IC |  | 1 Cell IC |  | 2 Cell IC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 90%-ile | mean | 90%-ile | mean | 90%-ile | mean |
| 1 | 340 | 285 | 40 | 35 | 40 | 32 |
| 2 | 340 | 290 | 80 | 40 | 40 | 37 |
| 3 | 400 | 344 | 40 | 23 | 40 | 22 |
| 4 | 360 | 338 | 80 | 49 | 40 | 36 |
| 5 | 300 | 247 | 200 | 154 | 120 | 86 |
| 6 | 420 | 316 | 160 | 121 | 80 | 52 |
| 7 | 400 | 292 | 40 | 38 | 40 | 36 |
| 8 | 320 | 222 | 380 | 352 | 120 | 90 |

Table 6: cell detection delay under ETU30 channel model (ms)

| case # | NO IC |  | 1 Cell IC |  | 2 Cell IC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 90%-ile | mean | 90%-ile | mean | 90%-ile | mean |
| 1 | 200 | 193 | 40 | 33 | 40 | 30 |
| 2 | 400 | 348 | 40 | 33 | 40 | 32 |
| 3 | 200 | 152 | 40 | 38 | 40 | 35 |
| 4 | 240 | 207 | 80 | 48 | 80 | 40 |
| 5 | 560 | 459 | 340 | 275 | 160 | 97 |
| 6 | 320 | 283 | 200 | 185 | 80 | 66 |
| 7 | 240 | 212 | 40 | 33 | 40 | 30 |
| 8 | 520 | 416 | 320 | 229 | 120 | 89 |

Table 7: cell detection delay under ETU70 channel model (ms)

| case # | NO IC |  | 1 Cell IC |  | 2 Cell IC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 90%-ile | mean | 90%-ile | mean | 90%-ile | mean |
| 1 | 700 | 618 | 160 | 131 | 80 | 65 |
| 2 | 1040 | 1061 | 120 | 99 | 40 | 33 |
| 3 | 620 | 566 | 120 | 85 | 40 | 34 |
| 4 | 840 | 831 | 160 | 145 | 80 | 70 |
| 5 | 1320 | 1068 | 480 | 202 | 360 | 174 |
| 6 | 840 | 708 | 280 | 134 | 120 | 89 |
| 7 | 840 | 713 | 160 | 139 | 120 | 72 |
| 8 | 1080 | 909 | 440 | 198 | 320 | 139 |

From Table 5 – 7, the following observations can be obtained:

Observation 1: ETU70 case gives the worst cell detection performance, i.e. with larger delay. However, it is still within the Rel-10 eICIC requirements of [1000] ms for its corresponding side conditions (i.e. see [2] [3]) for 1- and 2- Cell IC receiver, taking into consideration of L1 measurement period and other margins.

Observation 2: It is also observed that without IC Rel-8/9 baseline receiver could not meet the cell detection delay requirements (e.g. as given in [2][3]) for Rel-10 (eICIC) under the agreed side conditions for FeICIC. If IC receiver is assumed, regardless of either one or two cell(s) interference cancellation, the cell detection delay performance could meet the Rel-10 requirements.

## Conclusions

In this contribution, we provided our simulation results of cell detection with FeICIC. Based on the results, we proposed that:

Proposal 1: Reuse Rel-10 cell detection requirements for eICIC as Rel-11 cell detection requirements for FeICIC, which is [1000]ms latency.

Proposal 2: PSS/SSS IC receiver should be used as the baseline receiver for FeICIC.

## References

- R4-126060, Link simulation assumptions for cell identification with FeICIC, Ericsson, ST-Ericsson

- R4-115140, Additional cell identification delay performance results for eICIC, ZTE

- R4-114027, “cell identification delay performance results for eICIC”, ZTE