3GPP TSG RAN WG4 Meeting #70	                     R4-140020

Prague, Czech Republic, 10th – 14th Feb., 2014

- Agenda Item:	7.28

- Source:	LG Electronics

- Title:	R4-69AH-0089: TP for TR36.860: General TX requirements for 2ULs inter-band CA

Document for:	Approval

- Introduction

In the last RAN4 #69 meetings, RAN4 has agreed to have general SE requirements including UE-to-UE coexistence between 3GPP bands and also agreed not to consider the Harmonics/ IMD impacts on the ISM bands.

In this paper, some remaining TX requirements such as ACLR and SEM are analyzed for 2ULs inter-band CA when both CCs are activated.

- Reference

- R4-137035, “TP for TR 36.860 V0.4.0 some changes for dual-uplink inter-band CA general requirement,” Huawei, LG Electronics

- TS 36.101 v12.2.0

- TR 36.823 v11.0.0

Text proposal for TR 36.860

********************** Start of text proposal to TR 36.860 Chapter 5.2.8 ***********************

### 5.2.8		Output RF spectrum emissions

For 2UL inter-band CA, multiple carriers from different band transmit through a common antenna simultaneously, which means the spurious emission application range should exclude out-of-band emission frequency range and channel frequency range of all carriers. The figure below illustrates the spectrum emissions application range for 2UL inter-band CA.

For occupied bandwidth, general spurious emission requirement, existing requirement for each carrier could also be reused for 2UL inter-band CA. The requirements shall apply on each component carrier with both component carriers active.

Spurious domain

RB

Channel bandwidth

Spurious domain

ΔfOOB

ΔfOOB

E-UTRA Band

RB

Channel bandwidth

ΔfOOB

ΔfOOB

E-UTRA Band

Spurious domain

Figure 5.2.8-1: Transmitter RF spectrum for inter-band CA 2UL

### 5.2.8.1 	SEM requirements

Currently, RAN4 has assumed two RF front-end architectures for 2ULs inter-band CA as shown in Figure 5.2.8.1-1. In both cases, individual RF chain is used for each PCell and SCell, respectively.

Figure 5.2.8.1-1: Examples of 2ULs inter-band CA architectures

And also from the 2ULs inter-band CA combination under discussion in Table 5.2.8.1-1, the minimum band gap is observed in B2+B4 of Class A4, which is 95MHz. So the conventional SEM requirements in Rel-8/9 can be reused for the SEM of 2ULs inter-band CA.

Table 5.2.8.1-1: Dual-uplink inter-band CA combination lists

| Class |  | Band Combination | Frequency | Band Gap |
| --- | --- | --- | --- | --- |
| Dual-uplink Inter-band CA | A1 | B1+5 | 2.1G+800M | 1071MHz |
|  |  | B1+B19 | 2.1G+800M | 1075MHz |
|  |  | B3+20 | 1.8G+800M | 848MHz |
|  | A2 | B3+8 | 1.8G+900M | 795MHz |
|  |  | B4+12 | 2.1G+700M | 994MHz |
|  |  | B4+17 | 2.1G+700M | 994MHz |
|  | A3 | B1+7 | 2.1G+2.6G | 520MHz |
|  |  | B3+B7 | 1.7G+2.6G | 715MHz |
|  |  | B4+B7 | 1.7G+2.6G | 745MHz |
|  |  | B5+B12 | 2ULs (824~849M) + (698~716M) | 108MHz |
|  |  | B5+B17 | 2ULs (824~849M) + (704~716M) | 108MHz |
|  | A4 | B3+5 | 1.8G+800M | 861MHz |
|  |  | B2+B4 | 1.9G+2.1G | 95MHz |
|  |  | B7+B20 | 2.6G+800M | 1638MHz |
|  |  | B3+B26 | 1.8G+800M | 861MHz |
|  |  | B3+B19 | 1.8G+800M | 865MHz |
|  | A5 | B1+B21 | 2.1G+1.4G | 457MHz |
|  |  | B19+B21 | 800M+1.4G | 603MHz |

In 2ULs inter-band CA band combinations in future release, if band gap between two bands is less than sum of fOOB_X and fOOB_Y, the SEM requirements allowing higher PSD level for emissions is chosen as intra-band NC CA case.

From the observation, we propose as follow

Proposal 1:  The SEM requirement for 2UL CA should be the same as that in Rel-8/9 if band-gap is larger than fOOB_CBW_X + fOOB_CBW_Y.

Proposal 2 : The higher PSD levels of SEM of two individual carrier should be applied in the overlapped region if band-gap is smaller than fOOB_CBW_X + fOOB_CBW_Y.

### 5.2.8.2 	ACLR requirements

In the figure 5.2.8.2-1, we showed two cases for ACLR requirements for 2ULs inter-band CA. For current inter-band combinations, all deployments scenarios falls into case 1 in Figure 5.2.8.2-1 due to the large band gaps as shown in table 5.2.8.1-1. BWGAP is frequency gap between the upper frequency of low UL CC and the lower frequency of high UL CC.

Figure 5.2.8.2-1: ACLR requirements for 2ULs inter-band CA

However, when considering 2ULs inter-band CA in future release such that if band gap between is less the supported channel BW as shown in the case2 and case3 of Figure 5.2.8.2-1, RAN4 can follow the ACLR requirements for intra-band non contiguous CA in TR36.823.

Therefore, the typical possible solution can be proposed as follow

Proposal 3: For UTRAACLR1 and UTRAACLR2

▪    UTRAACLR1 is required to be met in the band gap when the gap bandwidth BWGAP is 5MHz  BWGAP.

▪    Both UTRAACLR1, UTRAACLR2 are required to be met in the gap when the gap bandwidth BWGAP is 15MHz BWGAP

●   Proposal 4: For E-UTRAACLR

▪    E- UTRAACLR adjacent channel power measurement bandwidth equals the channel bandwidth that it is adjacent to.

▪    In case the band gap is smaller than other of the channel bandwidth then for that channel bandwidth no E-UTRAACLR requirement is set for the band gap.

▪    In case the band gap is smaller than either of the channel bandwidths then no E- UTRAACLR requirement is set for the band gap.

### 5.2.8.3		Dual Uplink CA UE Coexistence requirements

For inter-band carrier aggregation with the uplink assigned to two E-UTRA bands the requirements in Table 5.2.8.3-1 applies for CA_1A-5A UE as an example.

NOTE: The requirements in 2 UL inter-band CA UE to UE co-existence table could be verified by measuring spurious emissions [at least] at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in 2 UL inter-band CA UE to UE co-existence table would be considered to be verified by the measurements verifying the 1 UL inter-band CA UE to UE co-existence requirements.

Table 5.2.8.3-1: Requirements

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Note |
| CA_1A-5A | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 21, 22, 28, 31, 38, 40, 42, 43, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3,34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA band 26 | FDL_low | - | FDL_high | -27 | 1 |  |
|  | E-UTRA band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 15,27 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1900 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 6, 8, 15 |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 | 15 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd, 3rd or 4th harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 6: Applicable when NS_05 in section 6.6.3.3.1 is signalled by the network. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 1930 - 1938 MHz. This requirement is applicable without any other uplink transmission bandwidth restriction for channel bandwidths within the range 1920 - 1980 MHz. NOTE XX: CA 1A-5A UE does not need to provide protection for Band 20 and 27 beyond the SEM limits. |  |  |  |  |  |  |  |

<End of TP>