3GPP TSG-RAN WG4 RAN4 #70                                         R4-140057

Prague, Cezec, 10 - 14 Feb, 2014

Agenda item:	9.4.3.2

Source: 	Samsung

Title: 	Performance evaluation of blind detection for NAICS receiver

Document for:	Discussion

1	Introduction

In 3GPP RAN4 #69 meeting, RAN4 has finalized the analysis of genie-aided NAICS receiver for inter-cell PDSCH to PDSCH interference scenario, including Phase 1 and Phase 2 performance evaluation and complexity analysis. In addition, RAN4 also agreed to further investigate the blind detection performance of interference parameters to conclude the feasibility of blind detection NAICS receiver. In this contribution, we investigate the performance of blind detection NAICS receiver, especially focusing on parameters RI, PMI, Modulation and EPRE Parameter Rho_A and Rho_B .

- Simulation Assumptions

In this contribution, we focus on the blind detection performance of R-ML receiver. Especially, we evaluate the performance loss caused by

- Blind detection of Data to RI, PMI, Modulation, RS EPRE Parameter Rho_A and Rho_B

For blind detection of RI, PMI, Modulation, a similar classification algorithm as in [1] can be applied.

For blind detection of Rho_A and Rho_B, it can be estimated based on the received signal powers from different sets of REs.

Note that per RB blind detection is applied in our simulation.

- For the other interference parameters such as TM, CRS related information, it is assumed that UE can acquire these information by network signaling and/or network coordination.

Furthermore, we evaluated the performance under TM4 with cell ID [0, 6, 1] with a set of interference profiles listed below. Detailed assumptions are shown in Annex.

- Interference profiles:  (1) INR1 = 7.77dB, INR2 = 2.29dB; (2) INR1 = 13.91dB, INR2 = 3.34dB

- Interference combinations:

Table 1 Simulated combinations

| TM (Serving/I1/I2) | Interference cell RI | Interference cell MCS | Serving cell RI | Serving cell MCS |
| --- | --- | --- | --- | --- |
| TM4/TM4/TM4 ON/OFF pattern | 1 | 5 | 1 | {5} |
|  |  |  |  | {14} |
|  | 1 | 14 | 1 | {5} |
|  |  |  |  | {14} |
|  | 2 | 5 | 1 | {5} |
|  |  |  |  | {14} |
|  | 2 | 14 | 1 | {5} |
|  |  |  |  | {14} |

- Simulation Results

In this contribution, we focus on the performance evaluation of R-ML receiver without losing the generality. Especially, three types of receivers are evaluated:

- Genie-aided R-ML receiver

- R-ML receiver with RI, PMI and MF (Modulation Format) and EPRE Parameter Rho_A and Rho_B blind detection.

- Additionally, the performance of MMSE-IRC receiver is also presented as a baseline receiver.

Firstly of all, the throughput performances of all cases are listed in Figure 1 to Figure 4.

- Figure 1: Performance under Rank 1 interference and INR1 = 7.77dB, INR2 = 2.29dB

- Figure 2: Performance under Rank 2 interference and INR1 = 7.77dB, INR2 = 2.29dB

- Figure 3: Performance under Rank 1 interference and INR1 = 13.91dB, INR2 = 3.34dB

- Figure 4: Performance under Rank 2 interference and INR1 = 13.91dB, INR2 = 3.34dB

Figure 1: Performance under Rank 1 interference and INR1 = 7.77dB, INR2 = 2.29dB

Figure 2: Performance under Rank 2 interference and INR1 = 7.77dB, INR2 = 2.29dB

Figure 3: Performance under Rank 1 interference and INR1 = 13.91dB, INR2 = 3.34dB

Figure 4: Performance under Rank 2 interference and INR1 = 13.91dB, INR2 = 3.34dB

- Observations

Based on the simulation results presented in Section 3, the performance gain of R-ML with and without blind detection under different interference profiles are summarized in Table 1 and Table 2 correspondingly. Furthermore, the average performance gain is also provided compared with the baseline receiver.

Table 1: Performance Gain under INR1 = 7.77dB, INR2 = 2.29dB

| Interference cell RI | Interference cell MCS | Serving cell RI | Serving cell MCS | SNR Gain @ 70% Throughput [dB] v.s. MMSE-IRC Receiver |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | R-ML with Full Network Information | R-ML with RI/PMI/MF/Rho_A/Rho_B Blind Detection |
| 1 | 5 | 1 | {5} | 4.9 | 3.3 |
|  |  |  | {14} | 3.7 | 1.7 |
| 1 | 14 | 1 | {5} | 3.3 | 1.3 |
|  |  |  | {14} | 2.3 | 0.3 |
| 2 | 5 | 1 | {5} | 3.1 | 0.7 |
|  |  |  | {14} | 1.5 | -0.5 |
| 2 | 14 | 1 | {5} | 1.2 | -1.0 |
|  |  |  | {14} | 0.3 | -1.3 |
| Average Gain v.s. MMSE-IRC Receiver |  |  |  | 2.5 | 0.6 |

Table 2: Performance Gain under INR1 = 13.91dB, INR2 = 3.34dB

| Interference cell RI | Interference cell MCS | Serving cell RI | Serving cell MCS | SNR Gain @ 70% Throughput [dB] v.s. MMSE-IRC Receiver |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | R-ML with Full Network Information | R-ML with RI/PMI/MF/Rho_A/Rho_B Blind Detection |
| 1 | 5 | 1 | {5} | 8.4 | 7.8 |
|  |  |  | {14} | 8.8 | 4.1 |
| 1 | 14 | 1 | {5} | 7.3 | 6.1 |
|  |  |  | {14} | 6.6 | 2.8 |
| 2 | 5 | 1 | {5} | 7.8 | 6.9 |
|  |  |  | {14} | 6.5 | 2.1 |
| 2 | 14 | 1 | {5} | 3.6 | 1.2 |
|  |  |  | {14} | 0.9 | -0.4 |
| Average Gain v.s. MMSE-IRC Receiver |  |  |  | 6.2 | 3.8 |

Based on the results, for R-ML receiver with RI/PMI/MF/Rho_A/Rho_B blind detection,

- Compared with genie-aided R-ML receiver, there is significant performance loss caused by joint RI/PMI/MF and EPRE Parameter Rho_A and Rho_B blind detection. The average performance loss is 1.9dB and 2.4dB for two interference level correspondingly.

- The performance of EPRE Parameter Rho_A and Rho_B blind detection is very sensitive to channel estimation of interference cell. In case of INR and SNR is in the similar range, the performance loss is most significant. The maximum performance loss could grow up to 4.4dB in the cases.

- Compared with MMSE-IRC receiver, there are some cases that performance loss up to -1.3dB is observed under low interference level case. Note that the interference level are developed based 5% cell edge UE. Thus, potentially a certain portion of UEs may suffer the performance loss under lower interference.

Based on the analysis above, our observations and conclusion are:

- Observation: for R-ML receiver, the observed average performance loss caused by joint RI/PMI/MF and EPRE Parameter Rho_A and Rho_B blind detection is around 2.4dB (maximum loss is 4.4dB). Thus, either dynamic network signaling or semi-static network signaling can be provided for NAICS receiver to avoid the noticeable performance loss.

- Conclusion

In this contribution, we investigate the performance of blind detection NAICS receiver, especially focusing on parameters RI, PMI, Modulation and EPRE Rho_A and Rho_B in this contribution.

Based on the simulation results, our observations are:

- Observation: for R-ML receiver, the observed average performance loss caused by joint RI/PMI/MF and EPRE Parameter Rho_A and Rho_B blind detection is around 2.4dB (maximum loss is 4.4dB). Thus, either dynamic network signaling or semi-static network signaling can be provided for NAICS receiver to avoid the noticeable performance loss.

- Reference

- D. Bai, et al, “Near ML Modulation Classification”, in Proc. of IEEE VTC 2012, pp.1-5.

- Annex

Table 5-1: Simulation Assumptions

| Parameter | Value |
| --- | --- |
| System bandwidth | 10 MHz |
| RB allocation | 6 |
| Cell ID | [0, 6, 1] |
| Transmission mode on Serving cell | TM4 |
| Transmission mode on Interference cell | TM4 |
| MIMO configuration | 2x2 and low correlation |
| Channel model and Doppler frequency for target and interference cells | EPA 5Hz Use different channel seed for between cells |
| CRS configuration | 2 CRS ports. CRS is colliding between serving cell and interference cells |
| PA | -3dB |
| PB | 0dB |
| CSI-RS configuration | None |
| Channel Estimation | CRS-IC |
| PMI | Random PMI |
| H-ARQ | 8 HARQ processes |
| PCFICH | CFI = 2 |
| PCFICH/PDCCH detection | Not considered |