3GPP TSG RAN WG4 Meeting #70	                     R4-140404

Prague, Czech Republic, 10th – 14th Feb., 2014

Agenda Item:	7.23.1

Source:	LG Electronics, LG UPlus

Title:	TP for TR 36.851: B1+B5 Harmonic and IMD analysis for 3DLs CA_1A_5A_7A band combination UE

Document for:	Approval

- Introduction

In the last RAN4 #69AH UE RF meetings, there are some clarifications for additional bandwidth combination set to support 3DLs inter-band CA [1]. The previous B1+B5 band combination UE only support 10MHz+10MHz bandwidth combination set, therefore RAN4 do not guarantee the fallback operation from 3DLs CA to 2DLs CA.

In this paper, we analyzed the harmonic and IMD products for B1+B5 UE to support additional bandwidth combination set for B1+B5+B7 3DLs inter-band CA UE.

- Reference

- R4-69AH-0023, “Carrier aggregation bandwidth support,” Qualcomm Incorporated

- TR 36.851 v0.8.0

Text proposal for TR 36.851

******************** Start of text proposal to TR 36.851 Chapter 6.1.X **********************

6.1.X	LTE Advanced Carrier Aggregation of Band 1 and Band 5

CA_1-5 is designed to operate in the operating bands in table 6.1.X-1.

Table 6.1.X-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | Channel BW (MHz) | BS transmit / UE receive |  |  | Channel BW (MHz) |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |  |
| CA_1-5 | 1 | 1920 MHz | – | 1980 MHz | 5, 10, 15, 20 (Note 1) | 2110 MHz | – | 2170 MHz | 5, 10, 15, 20 | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 5,10 (Note 1) | 869 MHz | – | 894 MHz | 5, 10 |  |
| NOTE 1: The WI considers only one uplink component carrier to be used in any of the two frequency bands at any time |  |  |  |  |  |  |  |  |  |  |

6.1.X.1		List of specific combination issues

6.1.X.1.1	Channel bandwidths per operating band for CA

Table 6.1.X.1.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| CA_1A-5A | 1 |  |  | Yes | Yes | Yes | Yes |
|  | 5 |  |  | Yes | Yes |  |  |

##### 6.1.X.1.2	Co-existence studies for CA_1-5

As shown in table 6.1.X.1.2-1, the harmonic frequencies of Band 1 and Band 5 in UL are away from the receive bands of interest in the DL and therefore we can conclude that there is no issue on harmonic interference.

Table 6.1.X.1.2-1: Impact of UL/DL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 | 4220 | 4340 | 6330 | 6510 |
| 5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 1738 | 1788 | 2607 | 2682 |

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 1 and Band 5 DL carriers can be calculated as shown in table 6.1.X.1.2-2 below:

Table 6.1.X.1.2-2: Band 1 and Band 5 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 869 | 894 | 2110 | 2170 |
| 2nd order harmonics frequency range (MHz) | 1738 | 1788 | 4220 | 4340 |
| 3rd order harmonics frequency range (MHz) | 2607 | 2682 | 6330 | 6510 |
| 2nd order IMD products | (f2-low – f1-high) | (f2-high – f1-low) | (f2_low + f1_low) | (f2_high + f1_high) |
| IMD frequency limits (MHz) | 1216 | 1301 | 2979 | 3064 |
| 3rd order IMD products | (f2_low – 2*f1_high) | (f2_high – 2*f1_low) | (2*f2_low – f1_high) | (2*f2_high – f1_low) |
| IMD frequency limits (MHz) | 322 | 432 | 3326 | 3471 |
| 3rd order IMD products | (2*f1_low + f2_low) | (2*f1_high + f2_high) | (2*f2_low + f1_low) | (2*f2_high + f1_high) |
| IMD frequency limits (MHz) | 3848 | 3958 | 5089 | 5234 |
| 3rd order IMD products | (f1_low – f2_high + f2_low) | (f1_high + f2_high – f2_low) | (f2_low – f1_high + f1_low) | (f2_high + f1_high – f1_low) |
| IMD frequency limits (MHz) | 849 | 914 | 2100 | 2180 |

It can be seen that the 3rd IMD products may fall into BS receive band of band 5. However, as described in subclause 5.1 of TR 36.850, due to the low-high band combinations use separate antennas, this issue can be regarded as covered in co-located BS scenarios and there is no need to address in inter-band carrier aggregation context.

6.1.X.1.3		ΔTIB,c and ΔRIB (1 UL)

For the UE which supports CA_1A-5A the ΔTIB,c is defined for applicable bands in table 6.1.X.1.3-1.

Table 6.1.X.1.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-5A | 1 | 0.3 |
|  | 5 | 0.3 |

For the UE which supports CA_1A-5A the ΔRIB is defined for applicable bands in table 6.1.X.1.3-2.

Table 6.1.X.1.3-2: ΔRIB

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_1A-5A | 1 | 0 |
|  | 5 | 0 |

<End of TP>