3GPP TSG-RAN WG4 (Radio) Meeting #70	R4-140457

Prague, Czech, 10 – 14 February 2014

Agenda Item:	7.35.2

Source: 	Alcatel-Lucent, Verizon

Title: 	Text Proposal on Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (2 + 4 + 13)

Document for:	Approval

# 1.	Introduction

The revised WI proposal to support the LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 4 and Band 13 was approved in RAN#62 [1]. One objective of the WI is to specify the band-combination specific Radio Frequency (RF) requirements for inter-band Carrier Aggregation (CA) of Band 2, Band 4 and Band 13 with 1 Uplink (UL) active in one of these bands.

The impacts of harmonics and IMD products caused by LTE-A BS supporting 3DL CA of Band (2 + 4 + 13) to the receiver of own or different BS was investigated in [2]. In this paper, we provide a text proposal to record the findings in the Technical Report for 3 Band Carrier Aggregation with Single Uplink [3].

# 2.	Text Proposal

<New section>

##### 6.2.1	Co-existence studies for 1 UL/3 DL

The 3rd order IMD products caused in the BS by transmitting of Band 2, Band 4 and Band 13 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 6.2.1-1 below:

Table 6.2.1-1: Band 2, Band 4 and Band 13 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f2-low | f2-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 746 | 756 | 1930 | 1990 | 2110 | 2155 |
| 3rd order IMD products | (f1-low + f2-low – f3-high) |  |  | (f1-high + f2-high – f3-low) |  |  |
| IMD frequency limits (MHz) | 521 |  |  | 636 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 866 |  |  | 981 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 3284 |  |  | 3399 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 4786 |  |  | 4901 |  |  |

It can be seen from Table 6.2.1-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (2 + 4 + 13) may fall into the BS receive band of Band 8. However, Band 8 is not intended for use in the same geographical area as Bands 2, 4 and 13.

Therefore, except the frequency bands impacted by harmonics and IMD products from the constituent 2 DL CA configurations, the 3rd order IMD products caused by BS supporting 3 DL CA of Band (2 + 4 + 13) will not fall into the BS receive band of any other frequency band operating in the same geographical area.

<End of section>

# References

[1]	RP-131684, “Revised WID: LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 4 and Band 13”, Verizon.

[2]	R4-140456, “Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (2 + 4 + 13)”, Alcatel-Lucent, Verizon.

[3]	R4-137075, “Draft New Technical Report 'LTE Advanced 3 Band Carrier Aggregation (3DL/1UL)'”, AT&T.