3GPP TSG-RAN WG4 (Radio) Meeting #70	R4-140464

Prague, Czech, 10 – 14 February 2014

Agenda Item:	7.35.2

Source: 	Alcatel-Lucent, AT&T

Title: 	Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (2 + 29 + 30)

Document for:	Discussion

# 1.	Introduction

The revised WI proposal to support the LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 29 and Band 30 was approved in RAN#62 [1]. One objective of the WI is to specify the band-combination specific Radio Frequency (RF) requirements for inter-band Carrier Aggregation (CA) of Band 2, Band 29 and Band 30 with 1 Uplink (UL) active in one of these bands.

The impacts of harmonics and InterModulation Distortion (IMD) products caused by LTE Advanced (LTE-A) Base Station (BS) supporting the constituent 2 Downlink (DL) CA configurations of this band combination to the receiver of own or different BS were investigated in [2-4]. In this paper, we investigate the impacts of harmonics and IMD products caused by LTE-A BS supporting 3DL CA of this band combination to the receiver of own or different BS.

# 2.	Discussion

For LTE Advanced BS supporting inter-band CA, the transceivers of the two or three CA bands would typically share a common antenna system. When BS for two or three bands share the same antenna, operators should ensure the following:

- One band BS transmitter Out-Of-Band Emissions (OOBE) including harmonics and IMD falling into the other BS receive band do not cause BS receiver desensitization; and one band BS transmit carrier powers do not result in the other band BS receiver blocking.

- Third order Passive Intermodulations (PIM) generated by BS antenna path (including antennas, cables and connectors) and caused by multiple transmit band carriers do not fall into one of multiple BS receive bands and result in BS receiver desensitization.

## 2.1	Harmonics and IMD Products

Harmonics are produced by the BS oscillator or generated in non-linear circuit elements; while IMD products are caused by the mixing of multiple frequencies in non-linear circuit elements. Wireless systems producing transmit signals within multiple frequency bands must consider harmonics and IMD generation as BS receiver desensitization may occur when harmonics or IMD frequencies fall within the receive band of own or different BS. Usually the non-linear element is an amplifier or mixer, but RF connectors and BS antenna systems can also generate harmonics and IMDs due to imperfect physical contact or dissimilar metals used in their construction. Wireless BS are particularly sensitive to harmonics and IMD interference which fall upon UL receiver channels that are in use by the BS. To limit the receiver desensitization to 0.4 dB, in-band harmonics and IMD interference must be limited to less than 10 dB below the receiver noise floor.

Harmonics and IMD are measured as the ratio in power levels between the distortion signal and the input signal in units of dBc. The magnitudes of the harmonics and IMD depend on the characteristics of the non-linear element, and the magnitude normally decreases as the order of the harmonics and IMD increases. For this reason, lower order harmonics and IMD products are considered the most damaging to wireless systems, while the higher order harmonics and IMD products are generally less damaging. Therefore, we will focus our investigation on the 2nd and 3rd order harmonics and IMD products caused by transmitting of Band 2, Band 29 and Band 30 carriers in the BS.

## 2.2	BS Harmonics and IMD for CA

The LTE operating bands currently defined in the 3GPP standards are provided in Table 1 below [5]:

Table 1: LTE frequency bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  |  |  | Downlink (DL) operating band BS transmit UE receive |  |  |  |  |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  |  |  | FDL_low – FDL_high |  |  |  |  |  |  |  |
| 1 | 1920 MHz | – |  | 1980 MHz |  | 2110 MHz |  | – |  |  | 2170 MHz |  | FDD |
| 2 | 1850 MHz | – |  | 1910 MHz |  | 1930 MHz |  | – |  |  | 1990 MHz |  | FDD |
| 3 | 1710 MHz | – |  | 1785 MHz |  | 1805 MHz |  | – |  |  | 1880 MHz |  | FDD |
| 4 | 1710 MHz | – |  | 1755 MHz |  | 2110 MHz |  | – |  |  | 2155 MHz |  | FDD |
| 5 | 824 MHz | – |  | 849 MHz |  | 869 MHz |  | – |  |  | 894MHz |  | FDD |
| 61 | 830 MHz | – |  | 840 MHz |  | 875 MHz |  | – |  |  | 885 MHz |  | FDD |
| 7 | 2500 MHz | – |  | 2570 MHz |  | 2620 MHz |  | – |  |  | 2690 MHz |  | FDD |
| 8 | 880 MHz | – |  | 915 MHz |  | 925 MHz |  | – |  |  | 960 MHz |  | FDD |
| 9 | 1749.9 MHz | – |  | 1784.9 MHz |  | 1844.9 MHz |  | – |  |  | 1879.9 MHz |  | FDD |
| 10 | 1710 MHz | – |  | 1770 MHz |  | 2110 MHz |  | – |  |  | 2170 MHz |  | FDD |
| 11 | 1427.9 MHz | – |  | 1447.9 MHz |  | 1475.9 MHz |  | – |  |  | 1495.9 MHz |  | FDD |
| 12 | 699 MHz | – |  | 716 MHz |  | 729 MHz |  | – |  |  | 746 MHz |  | FDD |
| 13 | 777 MHz | – |  | 787 MHz |  | 746 MHz |  | – |  |  | 756 MHz |  | FDD |
| 14 | 788 MHz | – |  | 798 MHz |  | 758 MHz |  | – |  |  | 768 MHz |  | FDD |
| 15 | Reserved |  |  |  |  | Reserved |  |  |  |  |  |  | FDD |
| 16 | Reserved |  |  |  |  | Reserved |  |  |  |  |  |  | FDD |
| 17 | 704 MHz | – |  | 716 MHz |  | 734 MHz |  | – |  |  | 746 MHz |  | FDD |
| 18 | 815 MHz | – |  | 830 MHz |  | 860 MHz |  | – |  |  | 875 MHz |  | FDD |
| 19 | 830 MHz | – |  | 845 MHz |  | 875 MHz |  | – |  |  | 890 MHz |  | FDD |
| 20 | 832 MHz | – |  | 862 MHz |  | 791 MHz |  | – |  |  | 821 MHz |  |  |
| 21 | 1447.9 MHz | – |  | 1462.9 MHz |  | 1495.9 MHz |  | – |  |  | 1510.9 MHz |  | FDD |
| 22 | 3410 MHz | – |  | 3490 MHz |  | 3510 MHz |  | – |  |  | 3590 MHz |  | FDD |
| 23 | 2000 MHz | – |  | 2020 MHz |  | 2180 MHz |  | – |  |  | 2200 MHz |  | FDD |
| 24 | 1626.5 MHz | – |  | 1660.5 MHz |  | 1525 MHz |  | – |  |  | 1559 MHz |  | FDD |
| 25 | 1850 MHz | – |  | 1915 MHz |  | 1930 MHz |  | – |  |  | 1995 MHz |  | FDD |
| 26 | 814 MHz | – |  | 849 MHz |  | 859 MHz |  | – |  |  | 894 MHz |  | FDD |
| 27 | 807 MHz | – |  | 824 MHz |  | 852 MHz |  | – |  |  | 869 MHz |  | FDD |
| 28 | 703 MHz | – |  | 748 MHz |  | 758 MHz |  | – |  |  | 803 MHz |  | FDD |
| 29 | N/A |  |  |  |  | 717 MHz | – |  |  | 728 MHz |  |  | FDD2 |
| 30 | 2305 MHz | – |  | 2315 MHz |  | 2350 MHz |  | – |  |  | 2360 MHz |  | FDD |
| 31 | 452.5 MHz |  | – |  | 457.5 MHz | 462.5 MHz |  |  | – |  |  | 467.5 MHz | FDD |
| ... |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33 | 1900 MHz | – |  | 1920 MHz |  | 1900 MHz |  | – |  |  | 1920 MHz |  | TDD |
| 34 | 2010 MHz | – |  | 2025 MHz |  | 2010 MHz |  | – |  |  | 2025 MHz |  | TDD |
| 35 | 1850 MHz | – |  | 1910 MHz |  | 1850 MHz |  | – |  |  | 1910 MHz |  | TDD |
| 36 | 1930 MHz | – |  | 1990 MHz |  | 1930 MHz |  | – |  |  | 1990 MHz |  | TDD |
| 37 | 1910 MHz | – |  | 1930 MHz |  | 1910 MHz |  | – |  |  | 1930 MHz |  | TDD |
| 38 | 2570 MHz | – |  | 2620 MHz |  | 2570 MHz |  | – |  |  | 2620 MHz |  | TDD |
| 39 | 1880 MHz | – |  | 1920 MHz |  | 1880 MHz |  | – |  |  | 1920 MHz |  | TDD |
| 40 | 2300 MHz | – |  | 2400 MHz |  | 2300 MHz |  | – |  |  | 2400 MHz |  | TDD |
| 41 | 2496 MHz | – |  | 2690 MHz |  | 2496 MHz |  | – |  |  | 2690 MHz |  | TDD |
| 42 | 3400 MHz | – |  | 3600 MHz |  | 3400 MHz |  | – |  |  | 3600 MHz |  | TDD |
| 43 | 3600 MHz | – |  | 3800 MHz |  | 3600 MHz |  | – |  |  | 3800 MHz |  | TDD |
| 44 | 703 MHz | – |  | 803 MHz |  | 703 MHz |  | – |  |  | 803 MHz |  | TDD |
| Note 1: Band 6 is not applicable. Note 2: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |  |  |  |  |  |  |  |  |

It can be seen from Table 1 that Band 2 is operating at 1850 – 1910 MHz for UL and 1930 – 1990 MHz for DL, Band 29 is operating at 717 – 728 MHz for DL only, and Band 30 is operating at 2305 – 2315 MHz for UL and 2350 – 2360 MHz for DL. The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting the constituent 2DL CA configurations of Band 2, Band 29 and Band 30, i.e. Band (2 + 29), Band (29 + 30) and Band (2 + 30) were discussed in [2-4]. The 3rd order IMD products caused in the BS by transmitting of Band 2, Band 29 and Band 30 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 2 below:

Table 2: Band 2, Band 29 and Band 30 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f2-low | f2-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 717 | 728 | 1930 | 1990 | 2350 | 2360 |
| 3rd order IMD products | (f1-low + f2-low – f3-high) |  |  | (f1-high + f2-high – f3-low) |  |  |
| IMD frequency limits (MHz) | 287 |  |  | 368 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 1077 |  |  | 1158 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 3552 |  |  | 3633 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 4997 |  |  | 5078 |  |  |

It can be seen from Table 2 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations as shown in [2-4], the 3rd order IMD products supporting 3 DL CA of Band (2 + 29 + 30) may fall into the BS receive band of Bands 42 and 43.

## 2.3	Possible Remedies for BS IMD

The possible remedies for IMD caused by BS supporting 3 DL CA of Band (2 + 29 + 30) into the BS receive band of Bands 42 and 43 are described below:

- Transmit Path Generated IMD: This type of IMD is generated in the oscillator or as signals pass through the amplifier stages of the BS transmit path. The BS transmit path will generate IMD which will appear at the BS equipment antenna connector (EAC) as spurious emissions. Spurious emissions appearing at the EAC and falling within the receive band of own or other BS will not be attenuated by the receive filter, and will pass into the receive path and desensitize the receiver. The 3rd order IMD products of Band (2 + 29 + 30) DL carriers falling within the receive band of Bands 42 and 43 can be mitigated by transmit filtering with not very sharp roll-off considering the sufficient frequency gap between Bands 2, 29 and 30 DL and Bands 42 and 43 UL.

- Receiver Path Generated IMD: This type of IMD is generated when the BS transmit signals appearing at the EAC pass through the receive filter and are applied to the front end of the receiver chain. This type of interference can be mitigated through a combination of receive filter attenuation within the transmit band and high 2nd and 3rd order Input Intercept Point (IIP2 and IIP3) of the receive path. Since there is sufficient frequency gap between Bands 2, 29 and 30 DL and Bands 42 and 43 UL, the Band 42 or 43 BS receive filter attenuation in the Band (2 + 29 + 30) BS transmit band should be substantial enough to limit the level of the transmit signals applied to the receiver front end. And the high IIP3 of the BS receive path would further reduce the potential interference to well below the Band 42 or 43 BS receiver noise floor.

- Antenna Passive Intermodulation: Even passive systems such as antennas, duplexers or filter combiners can generate IMD products via the non-linearities of dissimilar metals used in their construction, discontinuities in connectors or imperfections in the antenna system. This type of interference impacts all BS systems, but can be particularly troublesome when multiple BS operating in different frequency bands share a common antenna system. In the Band (2 + 29 + 30) case, the DL signals in the 3 constituent bands would be combined via a triplexer and applied to a common antenna system, with the 3rd order IMD products fall within Band 42 or 43 receive band and desensitize the BS receiver; in this case Band (2 + 29 + 30) BS transmitter should not share the same antenna with Band 42 or 43 BS receiver, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 42 or 43 BS receiver desensitization. The following alternatives can be used to avoid antenna sharing of Band (2 + 29 + 30) BS transmitter with Band 42 or 43 BS receiver:

- Use separate antenna elements for the transmitter and receiver. This could be done with the higher frequency bands using state-of-the-art Adaptive Active Antenna (AAA). With the performance target of current antenna system and the extra coupling loss between the transmitter and receiver antenna elements, the PIM products should be low enough in level that they would not cause BS receiver desensitization.

- Use separate transmitter chains and antenna elements for transmitting the Band 2, Band 29 and Band 30 carriers. This technique could also be used for load balancing between the multiple transmit antennas of a BS.

# 3.	Conclusions

In this paper, we have investigated the impacts of harmonics and IMD products caused by LTE-A BS supporting 3DL CA of Band (2 + 29 + 30) to the receiver of own or different BS. We have shown that in addition to the frequency bands impacted by harmonics and IMD products from the constituent 2 DL CA configurations, the 3rd order IMD products caused in the BS by transmitting of Band 2, Band 29 and Band 30 DL carriers (i.e. 3DL CA with one component carrier in each band) may also fall into the BS receive band of Bands 42 and 43, hence BS receiver desensitization may be an issue.

However, with the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 42 or 43 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band (2 + 29 + 30) BS transmitter does not share the same antenna with Band 42 or 43 BS receiver.

Therefore, it is recommended that Band (2 + 29 + 30) BS transmitter should not share the same antenna with Band 42 or 43 BS receiver to prevent BS receiver desensitization, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 42 or 43 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

# References

[1]	RP-131697, “Revised WID: LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 29 and Band 30”, AT&T.

[2]	R4-113984, “	Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band 2, 4 or 5 plus additional new 716-728 downlink”, Alcatel-Lucent.

[3]	R4-133374, “Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (29 + 30)”, Alcatel-Lucent, AT&T.

[4]	R4-134708, “Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (2 + 30)”, Alcatel-Lucent, AT&T.

[5]	3GPP TS 36.104 v12.2.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”.