3GPP TSG-RAN WG4 (Radio) Meeting #70	R4-140475

Prague, Czech, 10 – 14 February 2014

Agenda Item:	7.35.2

Source: 	Alcatel-Lucent, AT&T

Title: 	Text Proposal on Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (12 + 30)

Document for:	Approval

# 1.	Introduction

The WI proposal to support the LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 12 and Band 30 was approved in RAN#62 [1]. One objective of the WI is to specify the band-combination specific Radio Frequency (RF) requirements for Band (12 + 30) inter-band CA (2DL/1UL) scenario.

The impact of Harmonics and InterModulation Distortion (IMD) products caused by LTE Advanced Base Station (BS) supporting CA of this band combination to the receiver of own or different BS was investigated in [2]. In this paper, we provide a text proposal to record the findings in the Inter-band Carrier Aggregation Technical Report [3].

# 2.	Text Proposal

<New section>

##### 6.1.X.1.2.1	Co-existence studies for 1 UL/2 DL

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 12 and Band 30 DL carriers can be calculated as shown in Table 6.1.X.1.2.1-1 below:

Table 6.1.X.1.2.1-1: Band 12 and Band 30 DL harmonics and IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 2350 | 2360 |
| 2nd harmonics frequency limits (MHz) | 1458 | 1492 | 4700 | 4720 |
| 3rd harmonics frequency limits (MHz) | 2187 | 2238 | 7050 | 7080 |
| 2nd order IMD products | (f2-low – f1-high) | (f2-high – f1-low) | (f2-low + f1-low) | (f2-high + f1-high) |
| IMD frequency limits (MHz) | 1604 | 1631 | 3079 | 3106 |
| 3rd order IMD products | (f2-low – 2*f1-high) | (f2-high – 2*f1-low) | (2*f2-low – f1-high) | (2* f2-high – f1-low) |
| IMD frequency limits (MHz) | 858 | 902 | 3954 | 3991 |
| 3rd order IMD products | (2*f1-low + f2-low) | (2*f1-high + f2-high) | (2*f2-low + f1-low) | (2*f2-high + f1-high) |
| IMD frequency limits (MHz) | 3808 | 3852 | 5429 | 5466 |
| 3rd order IMD products | (f1-low – f2-high + f2-low) | (f1-high + f2-high – f2-low) | (f2-low – f1-high + f1-low) | (f2-high + f1-high – f1-low) |
| IMD frequency limits (MHz) | 719 | 756 | 2333 | 2377 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1-low – f2-BWmax) | (f1-high + f2- BWmax) | (f2-low – f1- BWmax) | (f2-high + f1- BWmax) |
| IMD frequency limits (MHz) | 719 | 756 | 2340 | 2370 |

It can be seen from Table 6.1.X.1.2.1-1 that the 2nd harmonics of BS transmitting in Bands 12 may fall into the BS receive band of Bands 21, while the 2nd IMD products of BS transmitting in Bands 12 and 30 may fall into the BS receive band of Band 24, and the 3rd IMD products supporting CA of Band 12 and Band 30 may fall into the BS receive band of Bands 8, 20, 28, 40 and 44. Note that the calculation in Table 6.1.X.1.2.1-1 (except the last row) assumes the BS is transmitting with the whole 17 MHz DL frequency of Band 12 and the whole 10 MHz DL frequency of Band 30. However, even if the BS is only transmitting an up to 10 MHz DL in Band 12 and an up to 10 MHz DL in Band 30 as stated in the WIDS, the 3rd IMD products may still fall into the BS receive band of Bands 8, 20, 28, 40 and 44 as shown in the last row in Table 6.1.X.1.2.1-1.

It should be noted that Bands 8, 20, 21, 28, 40 and 44 are not intended for use in the same geographical area as Bands 12 and 30. Therefore, the focus here will be on the IMD falling into Band 24.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 24 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 12 and 30 BS transmitters do not share the same antenna with Band 24 BS receiver.

Therefore, it is recommended that Bands 12 and 30 BS transmitters should not share the same antenna with Band 24 BS receiver to prevent BS receiver desensitization, unless the antenna path meets very stringent 2nd order PIM specification so that the PIM will not cause Band 24 BS receiver desensitization.

<End of section>

# References

[1]	RP-131652, “Proposed WID: LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 12 and Band 30”, AT&T.

[2]	R4-140474, “Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (12 + 30)”, Alcatel-Lucent, AT&T.

[3]	R4-140843, “TR 36.851 V0.9.0: Rel-12 Inter-band Carrier Aggregation”, Ericsson.