3GPP TSG-RAN WG4 (Radio) Meeting #70	R4-140476

Prague, Czech, 10 – 14 February 2014

Agenda Item:	7.35.2

Source: 	Alcatel-Lucent, AT&T

Title: 	TP for TR36.851 (Release 12): ∆TIB and ∆RIB values of LTE-A Inter-band Carrier Aggregation of Band 12 and Band 30 (1UL)

Document for:	Approval

# 1.	Introduction

The WI proposal to support the LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 12 and Band 30 was approved in RAN#62 [1]. In consequence of this WI approval, RAN4 should define the relevant requirements of the maximum output power relaxation and the reference sensitivity relaxation to support inter-band CA of Band 12 and Band 30. In this contribution, both ∆TIB and ∆RIB values are proposed and the relaxation requirements are based on the RAN4 approved UE RF requirements for the inter-band Carrier Aggregation scenario of Class A1 in Release 11.

# 2.	Discussion

Based on the defined inter-band carrier aggregation scenario of Class A1, the Inter-band Carrier Aggregation of Band 12 and Band 30 is a low-high pair spectrum without harmonics relationship. The relaxations for Class A1, i.e. ΔTIB,c = 0.3dB and ΔRIB = 0dB, should be hired in support of inter-band Carrier Aggregation Band 12 and Band 30.

# 3.	Conclusion

Based on the UE RF requirements for the inter-band Carrier Aggregation scenario of Class A1, we propose the following requirements for the inter-band Carrier Aggregation Band 12 and Band 30,

Proposal: ΔTIB,c = 0.3dB and ΔRIB = 0dB should be applied to both bands for CA of Band 12 + Band 30

This proposal should be applied to the TR36.851 [2] and cover the requirements of Inter-band Carrier Aggregation Band 12 and Band 30.

# References

[1]	RP-131652, “Proposed WID: LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 12 and Band 30”, AT&T.

[2]	R4-140843, “TR 36.851 V0.9.0: Rel-12 Inter-band Carrier Aggregation”, Ericsson.

# Text Proposal

<Start of change>

# 1	Scope

The present document is a technical report for Inter-band Carrier Aggregation under Rel-12 time frame. The purpose is to gather the relevant background information and studies in order to address Inter-band Carrier Aggregation requirements.

This TR covers relevant background information and studies in order to address Inter-band Carrier Aggregation requirements for the Rel-12 band combinations in table 1-1.

Table 1-1: Release 12 inter-band carrier aggregation combinations

| WI code | WI title | Class |
| --- | --- | --- |
| LTE_CA_B1_B7 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 7 | A3 |
| LTE_CA_B2_B4 | LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 4 | A4 |
| LTE_CA_B23_B29 | LTE Advanced inter-band Carrier Aggregation of Band 23 and Band 29 | A1 |
| LTE_CA_B3_B26 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 26 | A1 |
| LTE_CA_B3_B19 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 19 | A1 |
| LTE_CA_B1_B8 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 8 | A1 |
| LTE_CA_B3_B28 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 28 | A1 |
| LTE_CA_B1_B26 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 26 | A1 |
| LTE_CA_B39_B41 | LTE Advanced inter-band Carrier Aggregation of Band 39 and Band 41 | A3 |
| LTE_CA_B2_B12 | LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 12 | A1 |
| LTE_CA_B2_B13 | LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 13 | A1 |
| LTE_CA_B19_B21 | LTE Advanced inter-band Carrier Aggregation of Band 19 and Band 21 | A5 |
| LTE_CA_B8_B26 | LTE Advanced inter-band Carrier Aggregation of Band 8 and Band 26 | A3 |
| LTE_CA_B12_B25 | LTE Advanced inter-band Carrier Aggregation of Band 12 and Band 25 | A1 |
| LTE_CA_B2_B5 | LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 5 | A1 |
| LTE_CA_B1_B18 | Additional bandwidth combination set for LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 18 | A1 |
| LTE_CA_B7_B28 | LTE Advanced inter-band Carrier Aggregation of Band 7 and Band 28 | A1 |
| LTE_CA_B5_B25 | LTE Advanced inter-band Carrier Aggregation of Band 5 and Band 25 | A1 |
| LTE_CA_B1_B11 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 11 | A5 |
| LTE_CA_B8_B11 | LTE Advanced inter-band Carrier Aggregation of Band 8 and Band 11 | A5 |
| LTE_CA_B5_B7 | LTE Advanced inter-band Carrier Aggregation of Band 5 and Band 7 | A1 |
| LTE_CA_B5_B30 | LTE Advanced inter-band Carrier Aggregation of Band 5 and Band 30 | A1 |
| LTE_CA_B29_B30 | LTE Advanced inter-band Carrier Aggregation of Band 29 and Band 30 | A1 |
| LTE_CA_B12_B30 | LTE Advanced inter-band Carrier Aggregation of Band 12 and Band 30 | A1 |

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

<New section>

### 6.1.X	LTE Advanced Carrier Aggregation of Band 12 and Band 30

CA_12-30 is designed to operate in the operating bands in table 6.1.X-1.

Table 6.1.X-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | Channel BW (MHz) | BS transmit / UE receive |  |  | Channel BW (MHz) |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |  |
| CA_12-30 | 12 | 699 MHz | – | 716 MHz | 5, 10 (note 1) | 729 MHz | – | 746 MHz | 5, 10 | FDD |
|  | 30 | 2305 MHz | – | 2315 MHz | 5, 10 (note 1) | 2350 MHz | – | 2360 MHz | 5, 10 |  |
| NOTE 1: The WI considers only one uplink component carrier to be used in any of the two frequency bands at any time |  |  |  |  |  |  |  |  |  |  |

#### 6.1.X.1		List of specific combination issues

##### 6.1.X.1.1	Channel bandwidths per operating band for CA

Table 6.1.X.1.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| CA_12A-30A | 12 |  |  | Yes | Yes |  |  | 0 |
|  | 30 |  |  | Yes | Yes |  |  |  |

##### 6.1.X.1.2	Co-existence studies for CA_12-30

As shown in table 6.1.X.1.2-1, the harmonic frequencies of Band 12 and Band 30 in UL are away from the receive bands of interest in the DL and therefore we can conclude that there is no issue on harmonic interference.

Table 6.1.X.1.2-1: Impact of UL/DL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 12 | 699 | 716 | 729 | 746 | 1398 | 1432 | 2097 | 2148 | 1458 | 1492 | 2187 | 2238 |
| 30 | 2305 | 2315 | 2350 | 2360 | 4610 | 4630 | 6915 | 6945 | 4700 | 4720 | 7050 | 7080 |

##### 6.1.X.1.3		ΔTIB,c and ΔRIB (1 UL)

For the UE which supports CA_12A-30A the ΔTIB,c is defined for applicable bands in table 6.1.X.1.3-1.

Table 6.1.X.1.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_12A-30A | 12 | 0.3 |
|  | 30 | 0.3 |

For the UE which supports CA_12A-30A the ΔRIB is defined for applicable bands in table 6.1.X.1.3-2.

Table 6.1.X.1.3-2: ΔRIB

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_12A-30A | 12 | 0 |
|  | 30 | 0 |

<End of change>