3GPP TSG-RAN4 Meeting #70 	R4-140484

# Prague, Czech, 10 – 14 February 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.141 | CR | 518 | rev | - | Current version: | 12.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of multi-band BS testing to TS 36.141 (Clauses 1 – 5) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Alcatel-Lucent, CATT, Huawei, NSN, ZTE, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MB_MSR_RF-Perf |  |  |  |  |  |  | Date: |  | 2014-02-13 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | Multi-band BS testing is introduced to TS 36.141 (Clauses 1 – 5). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Multi-band BS testing is added to TS 36.141 (Clauses 1 – 5). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multi-band BS testing is not supported in TS 36.141 (Clauses 1 – 5). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 3.3, 4.5.2.1, 4.6.8, 4.7.1, 4.7.2, 4.10, 4.11 (new clause), 4.12 (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Aggregated Channel Bandwidth:	 The RF bandwidth in which a Base Station transmits and receives multiple contiguously aggregated carriers. The aggregated channel bandwidth is measured in MHz.

Base Station RF bandwidth edge: The frequency of one of the edges of the Base Station RF bandwidth.

Base station receive period: The time during which the base station is receiving data subframes or UpPTS.

Base Station RF bandwidth: The bandwidth in which a Base Station transmits and receives multiple carriers within each supported operating band.

Carrier: The modulated waveform conveying the E-UTRA or UTRA (WCDMA) physical channels

Carrier aggregation: aggregation of two or more component carriers in order to support wider transmission bandwidths

Carrier aggregation band: a set of one or more operating bands across which multiple carriers are aggregated with a specific set of technical requirements

NOTE:	Carrier aggregation band(s) for an E-UTRA BS is declared by the manufacturer according to the designations in Tables 5.5-2 to 5.5-3

Channel bandwidth: The RF bandwidth supporting a single E-UTRA RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell. The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

Channel edge: The lowest and highest frequency of the E-UTRA carrier, separated by the channel bandwidth.

Contiguous carriers: a set of two or more carriers configured in a spectrum block where there are no RF requirements based on co-existence for un-coordinated operation within the spectrum block.

Contiguous spectrum: Spectrum consisting of a contiguous block of spectrum with no sub-block gaps.

DL RS power: The resource element power of Downlink Reference Symbol.

Downlink operating band: The part of the operating band designated for downlink.

Highest Carrier: The carrier with the highest carrier centre frequency transmitted/received in a specified operating band.

Inter RF bandwidth gap: The frequency gap between two consecutive RF bandwidths that are placed within two supported operating bands.

Inter-band carrier aggregation: carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Intra-band contiguous carrier aggregation: contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: non-contiguous carriers aggregated in the same operating band.

Lower Edge:	 The lowest frequency in the Base station RF bandwidth, or the lowest frequency in the channel bandwidth of a single E-UTRA carrier, used as a frequency reference point for transmitter and receiver requirements.

Lower sub-block edge: The frequency at the lower edge of one sub-block. It is used as a frequency reference point for both transmitter and receiver requirements.

Lowest Carrier:	The carrier with the lowest carrier centre frequency transmitted/received in a specified operating band.

Maximum Base Station RF bandwidth: The maximum RF bandwidth supported by a BS within each supported operating band.

Maximum output power: The mean power level per carrier of the base station measured at the antenna connector in a specified reference condition.

Maximum radio bandwidth: Maximum frequency difference between the upper edge of the highest used carrier and the lower edge of the lowest used carrier.

Maximum throughput: The maximum achievable throughput for a reference measurement channel.

Mean power: When applied to E-UTRA transmission this is the power measured in the channel bandwidth of the carrier. The period of measurement shall be at least one subframe (1ms), unless otherwise stated.

Multi-band Base Station: Base Station characterized by the ability of its transmitter and/or receiver to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different non-overlapping operating band than the other carrier(s).

Multi-carrier transmission configuration: A set of one or more contiguous or non-contiguous carriers that a BS is able to transmit simultaneously according to the manufacturer’s specification.

Multi-band transmitter: Transmitter characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different non-overlapping operating band than the other carrier(s).

Multi-band receiver: Receiver characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different non-overlapping operating band than the other carrier(s).

Non-contiguous spectrum: Spectrum consisting of two or more sub-blocks separated by sub-block gap(s).

Occupied bandwidth: The width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage β/2 of the total mean power of a given emission.

Operating band: A frequency range (paired or unpaired) that is defined with a specific set of technical requirements, in which E-UTRA operates.

NOTE: 	The operating band(s) for an E-UTRA BS is declared by the manufacturer according to the designations in Table 5.5-1.

Output power: The mean power of one carrier of the base station, delivered to a load with resistance equal to the nominal load impedance of the transmitter.

Rated output power: Rated output power of the base station is the mean power level per carrier that the manufacturer has declared to be available at the antenna connector.

RE power control dynamic range: The difference between the power of a RE and the average RE power for a BS at maximum output power for a specified reference condition.

Reference bandwidth: The bandwidth in which an emission level is specified.

RRC filtered mean power: The mean power as measured through a root raised cosine filter with roll-off factor  and a bandwidth equal to the chip rate of the radio access mode.

NOTE 1:	The RRC filtered mean power of a perfectly modulated W-CDMA signal is 0.246 dB lower than the mean power of the same signal.

Sub-block: This is one contiguous allocated block of spectrum for transmission and reception by the same Base Station. There may be multiple instances of sub-blocks within an RF bandwidth.

Sub-block bandwidth: The bandwidth of one sub-block.

Sub-block gap: A frequency gap between two consecutive sub-blocks within an RF bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation.

Synchronized operation: Operation of TDD in two different systems, where no simultaneous uplink and downlink occur.

Throughput: The number of payload bits successfully received per second for a reference measurement channel in a specified reference condition.

Total power dynamic range: The difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

Total RF bandwidth: Maximum sum of RF bandwidths in all supported operating bands.

Transmission bandwidth: Bandwidth of an instantaneous transmission from a UE or BS, measured in Resource Block units.

Transmission bandwidth configuration: The highest transmission bandwidth allowed for uplink or downlink in a given channel bandwidth, measured in Resource Block units.

Transmitter OFF period: The time period during which the BS transmitter is not allowed to transmit.

Transmitter ON period: The time period during which the BS transmitter is transmitting data and/or reference symbols, i.e. data subframes or DwPTS.

Transmitter transient period: The time period during which the transmitter is changing from the OFF period to the ON period or vice versa.

Unsynchronized operation: Operation of TDD in two different systems, where the conditions for synchronized operation are not met.

Uplink operating band: The part of the operating band designated for uplink.

Upper Edge: The highest frequency in the Base Station RF Bandwidth or the highest frequency in the channel bandwidth of a single E-UTRA carrier; used as a frequency reference point for transmitter and receiver requirements.

Upper sub-block edge: The frequency at the upper edge of one sub-block. It is used as a frequency reference point for both transmitter and receiver requirements.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

	Roll-off factor

β	Percentage of the mean transmitted power emitted outside the occupied bandwidth on the assigned channel

BWChannel	Channel bandwidth

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz. BWChannel_CA= Fedge_high- Fedge_low.

BWChannel,block	Sub-block bandwidth, expressed in MHz. BWChannel,block= Fedge,block,high- Fedge,block,low.

BWConfig	Transmission bandwidth configuration, expressed in MHz, where BWConfig = NRB x 180 kHz in the uplink and BWConfig = 15 kHz + NRB x 180 kHz in the downlink.

CA_X	Contiguous intra-band CA for band X where X is the applicable E-UTRA operating band

CA_X-X	Non-contiguous intra band CA for band X where X is the applicable E-UTRA operating band

CA_X-Y	CA for band X and Band Y where X and Y are the applicable E-UTRA operating band

f	Frequency

f	Separation between the channel edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency

fmax 	The largest value of f used for defining the requirement

FC	Carrier centre frequency

FC,block, high	Center frequency of the highest transmitted/received carrier in a sub-block.

FC,block, low	Center frequency of the lowest transmitted/received carrier in a sub-block.

FC_high 	The carrier centre frequency of the highest carrier, expressed in MHz.

FC_low 	The carrier centre frequency of the lowest carrier, expressed in MHz.

Fedge_low 	The lower edge of aggregated channel bandwidth, expressed in MHz. Fedge_low = FC_low - Foffset.

Fedge_high 	The upper edge of aggregated channel bandwidth, expressed in MHz. Fedge_high = FC_high + Foffset.

Fedge,block,low 	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset.

Fedge,block,high 	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset.

Foffset 	Frequency offset from FC_high to the upper edge or from F C,block, high to the upper sub-block edge, FC_low to the lower edge or from FC,block, low to the lower sub-block edge.

Ffilter	Filter centre frequency

f_offset 	Separation between the channel edge frequency and the centre of the measuring filter

f_offsetmax 	The maximum value of f_offset used for defining the requirement

EA: 	EPRE (energy per resource element) of PDSCH REs (resource elements) type A, i.e. REs in OFDM symbols that do not include reference symbols

EB: 	EPRE of PDSCH REs type B, i.e. REs in OFDM symbols that include reference symbols

ERS:	EPRE of reference symbols REs

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

NDL 	Downlink EARFCN

NOffs-DL 	Offset used for calculating downlink EARFCN

NOffs-UL 	Offset used for calculating uplink EARFCN

Physical layer cell identity

NCS	Number of Cyclic shifts for preamble generation in PRACH

NRB	Transmission bandwidth configuration, expressed in units of Resource Blocks

Downlink bandwidth configuration, expressed in multiples of

NUL 	Uplink EARFCN

Resource block size in the frequency domain, expressed as a number of subcarriers

System frame number

Physical resource block number

Radio network temporary identifier

Slot number within a radio frame

Antenna port number

Pd	Probability of PRACH preamble detection

Pfa	Total probability of false detection of the PRACH preamble

Pout	Output power

PEM,N	Declared emission level for channel N

Pmax	Maximum total output power

Pmax, c	Maximum carrier output power

PREFSENS	Reference sensitivity power level

Code word number

TA	Timing advance command, as defined in [16]

Basic time unit, as defined in [12]

Wgap	Sub-block gap or inter RF bandwidth gap size

Figure 3.2-1 Illustration of maximum radio bandwidth and Total RF bandwidth for Multi-band Base Station

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AC	Alternating Current

ACLR	Adjacent Channel Leakage Ratio

CACLR	Cumulative ACLR

ACK	Acknowledgement (in HARQ protocols)

ACS	Adjacent Channel Selectivity

ATT	Attenuator

AWGN	Additive White Gaussian Noise

B	Bottom RF channel (for testing purposes)

BS	Base Station

C	Contiguous

CA	Carrier Aggregation

BW	Bandwidth

CCE	Control Channel Element

CP	Cyclic prefix

CW	Continuous Wave

DC	Direct Current

DFT	Discrete Fourier Transformation

DTT	Digital Terrestrial Television

DUT	Device Under Test

EPRE	Energy per resource element

E-TM	E-UTRA Test Model

E-UTRA	Evolved UTRA

EARFCN	E-UTRA Absolute Radio Frequency Channel Number

EPA	Extended Pedestrian A model

ETC	E-UTRA Test Configuration

ETU	Extended Typical Urban model

EVA	Extended Vehicular A model

EVM	Error Vector Magnitude

FDD	Frequency Division Duplex

FFT	Fast Fourier Transformation

FRC	Fixed Reference Channel

HARQ	Hybrid Automatic Repeat Request

ICS	In-Channel Selectivity

IQ	In-phase - Quadrature phase

ITU-R	Radiocommunication Sector of the ITU

Iuant	E-Node B internal logical interface between the implementation specific O&M function and the RET antennas and TMAs control unit function of the E-Node B

LA	Local Area

M	Middle RF channel (for testing purposes)

MC	Multi-carrier

MIMO	Multiple Input Multiple Output

MCS	Modulation and Coding Scheme

MR	Medium Range

NC	Non-Contiguous

OBW	Occupied Band Width

OFDM	Orthogonal Frequency Division Multiplex

OOB	Out-Of-Band

PBCH	Physical Broadcast Channel

PCFICH	Physical control format indicator channel

PDCCH	Physical downlink control channel

PDSCH	Physical downlink shared channel

PHICH	Physical hybrid-ARQ indicator channel

PUCCH	Physical Uplink Control CHannel

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PSD 		Power Spectral Density

QAM	Quadrature Amplitude Modulation

QPSK	Quadrature Phase-Shift Keying

RAT	Radio Access Technology

RB	Resource Block

RE	Resource Element

REG	Resource Element Group

RF	Radio Frequency

RS		Reference Symbol

RX	Receive

RRC	Root Raised Cosine

SNR	Signal-to-Noise Ratio

SQRT	SQuare RooT

SC	Single Carrier

SRS	Sounding Reference Signal

T	Top RF channel (for testing purposes)

TA	Timing Advance

TC 	Test Configuration

TDD	Time Division Duplex

TT	Test Tolerance

TX	Transmit

UE	User Equipment

UMTS	Universal Mobile Telecommunications System

UTRA	UMTS Terrestrial Radio Access

WA	Wide Area

<Next changed section>

#### 4.5.2.1	Reception with multiple receiver antenna connectors, receiver diversity

For the tests in clause 7 of the present document, the requirement applies at each receiver antenna connector for receivers with antenna diversity or in the case of multi-carrier reception with multiple receiver antenna connectors.

Receiver requirements are tested at the antenna connector, with the remaining receiver(s) disabled or their antenna connector(s) being terminated. If the manufacturer has declared the receiver paths to be equivalent, it is sufficient to apply the specified test signal at any one of the receiver antenna connectors.

For a multi-band BS, multi-band tests for blocking and intermodulation are performed with the interferer(s) applied to each antenna connector mapped to the receiver for the wanted signal(s), however only to one antenna at a time. Antenna connectors to which no signals are applied are terminated.

<Next changed section>

### 4.6.8	Manufacturer's declarations of supported RF configurations

The manufacturer shall declare which operational configurations the BS supports by declaring the following parameters:

-	Support of the BS in non-contiguous spectrum operation. If the BS does not support non-contiguous spectrum operation the parameters for non-contiguous spectrum operation below shall not be declared.

-	The supported operating bands defined in subclause 5.5;

-	The frequency range within the above operating band(s) supported by the BS;

-	The maximum Base Station RF bandwidth supported by a BS within each operating band;

•	for contiguous spectrum operation

•	for non-contiguous spectrum operation

-	The supported operating configurations (multi-carrier, carrier aggregation, and/or single carrier) within each operating band.

-	The supported component carrier combinations at nominal channel spacing within each operating band and sub-block.

- The rated output power per carrier;

•	for contiguous spectrum operation

•	for non-contiguous spectrum operation

NOTE:	Different rated output powers may be declared for different operating configurations.

- The rated total output power as a sum of all carriers;

•	for contiguous spectrum operation

•	for non-contiguous spectrum operation

-	Maximum number of supported carriers within each band;

•	for contiguous spectrum operation

•	for non-contiguous spectrum operation

If the rated total output power and total number of supported carriers are not simultaneously supported, the manufacturer shall declare the following additional parameters:

- The reduced number of supported carriers at the rated total output power;

-	The reduced total output power at the maximum number of supported carriers.

For BS capable of multi-band operation, the parameters above shall be declared for each supported operating band in single-band operation. In addition the manufacturer shall declare the following additional parameters for BS capable of multi-band operation:

-	Supported operating band combinations of the BS

-	Supported operating band(s) of each antenna connector

-	Support of multi-band transmitter and/or multi-band receiver, including mapping to antenna connector(s)

-	Total number of supported carriers for the declared band combinations of the BS

-	Maximum number of supported carriers per band in multi-band operation

-	Total RF bandwidth of transmitter and receiver for the declared band combinations of the BS

-	Maximum RF bandwidth of each supported operating band in multi-band operation

-	Maximum radio bandwidth in transmit and receive direction for the declared band combinations of the BS

-	Any other limitations under simultaneous operation in the declared band combinations of the BS which have any impact on the test configuration generation

-	Total output power as a sum over all supported operating bands in the declared band combinations of the BS

-	Maximum supported power difference between any two carriers in any two different supported operating bands

-	The rated output power per carrier in multi-band operation

-	Rated total output power of each supported operating band in multi-band operation

<Next changed section>

### 4.7.1	RF bandwidth position for multi-carrier and/or CA testing

Many tests in this TS are performed with the maximum RF bandwidth located at the bottom, middle and top of the supported frequency range in each operating band. These are denoted as BRFBW(bottom), MRFBW (middle) and TRFBW (top).

Unless otherwise stated, the test shall be performed at BRFBW, MRFBW and TRFBW defined as following:

-	BRFBW: maximum RF bandwidth located at the bottom of the supported frequency range in each operating band;

-	MRFBW: maximum RF bandwidth located in the middle of the supported frequency range in each operating band;

-	TRFBW: maximum RF bandwidth located at the top of the supported frequency range in each operating band.

For BS capable of dual-band operation, unless otherwise stated, the test shall be performed at BRFBW_T’RFBW and B’RFBW_TRFBW defined as following:

BRFBW_ T’RFBW: the RF bandwidths located at the bottom of the supported frequency range in the lower operating band and at the highest possible simultaneous frequency position, within the maximum radio bandwidth, in the upper operating band.

B’RFBW_TRFBW: the RF bandwidths located at the top of the supported frequency range in the upper operating band and at the lowest possible simultaneous frequency position, within the maximum radio bandwidth, in the lower operating band.

NOTE: BRFBW_T’RFBW = B’RFBW_TRFBW = BRFBW_TRFBW when the declared maximum radio bandwidth spans both operating bands. BRFBW_TRFBW means the RF bandwidths are located at the bottom of the supported frequency range in the lower operating band and at the top of the supported frequency range in the upper operating band.

When a test is performed by a test laboratory, the position of BRFBW, MRFBW and TRFBW in each supported operating band, as well as the position of BRFBW_T’RFBW and B’RFBW_TRFBW in the supported operating band combinations, shall be specified by the laboratory. The laboratory may consult with operators, the manufacturer or other bodies.

### 4.7.2 	Aggregated channel bandwidth position for Contiguous CA occupied bandwidth testing

Occupied bandwidth test in this TS is performed with the aggregated channel bandwidth and sub-block bandwidths located at the bottom, middle and top of the supported frequency range in the operating band. These are denoted as BBW Channel CA(bottom), MBW Channel CA (middle) and TBW Channel CA (top) for contiguous spectrum operation.

Unless otherwise stated, the test for contiguous spectrum operation shall be performed at BBW Channel CA, MBW Channel CA and TBW Channel CA defined as following:

-	BBW Channel CA: 	aggregated channel bandwidth located at the bottom of the supported frequency range in each operating band;

-	MBW Channel CA: aggregated channel bandwidth located close in the middle of the supported frequency range in each operating band, with the center frequency of each component carrier aligned to the channel raster;

-	TBW Channel CA: 	aggregated channel bandwidth located at the top of the supported frequency range in each operating band.

When a test is performed by a test laboratory, the position of BBW Channel CA, MBW Channel CA and TBW Channel CA for contiguous spectrum operation in the operating band shall be specified by the laboratory. The laboratory may consult with operators, the manufacturer or other bodies.

<Next changed section>

## 4.10	Test configurations for multi-carrier and/or CA operation

The test configurations shall be constructed using the methods defined below, subject to the parameters declared by the manufacturer for the supported RF configurations as listed in subclause 4.6.8. The test configurations to use for conformance testing are defined for each supported RF configuration in subclause 4.11.

The applicable test models for generation of the carrier transmit test signal are defined in subclause 6.1.1.

### 4.10.1	ETC1: Contiguous spectrum operation

The purpose of test configuration ETC1 is to test all BS requirements excluding CA occupied bandwidth.

For ETC1 used in receiver tests only the two outermost carriers within each supported operating band need to be generated by the test equipment.

#### 4.10.1.1	ETC1 generation

ETC1 shall be constructed on a per band basis using the following method:

-	Declared maximum supported RF bandwidth for contiguous spectrum operation shall be used;

-	Select the narrowest supported carrier and place it adjacent to the lower edge of the RF bandwidth. Place a 5 MHz carrier adjacent to the upper edge of the RF bandwidth.

-	For transmitter tests, select as many 5 MHz carriers that the BS supports within a band and fit in the rest of the declared maximum supported RF bandwidth. Place the carriers adjacent to each other starting from the upper edge of the RF bandwidth. The nominal carrier spacing defined in clause 5.7 shall apply;

-	If 5 MHz carriers are not supported by the BS the narrowest supported channel BW shall be selected instead.

The test configuration should be constructed on a per band basis for all component carriers of the inter-band CA bands declared to be supported by the BS and are transmitted using the same antenna port. All configured component carriers are transmitted simultaneously in the tests where the transmitter should be on.

#### 4.10.1.2	ETC1 power allocation

For a BS declared to support MC operation,

Set the power of each carrier to the same level so that the sum of the carrier powers equals the rated total output power according to the manufacturer’s declaration in sub clause 4.6.8.

For a BS declared to support only CA operation,

Set the power spectral density of each carrier to the same level so that the sum of the carrier powers equals the rated total output power according to the manufacturer’s declaration in sub clause 4.6.8.

### 4.10.2	ETC2: Contiguous CA occupied bandwidth

ETC2 in this clause is used to test CA occupied bandwidth.

#### 4.10.2.1	ETC2 generation

The CA specific test configuration should be constructed on a per band basis using the following method:

-	All component carrier combinations supported by the BS, which have different sum of channel bandwidth of component carrier, shall be tested. For all component carrier combinations which have the same sum of channel bandwidth of component carriers, only one of the component carrier combinations shall be tested.

-	Of all component carrier combinations which have same sum of channel bandwidth of component carrier, select those with the narrowest carrier at the lower edge.

-	Of the combinations selected in the previous step, select one with the narrowest carrier at the upper edge.

-	If there are multiple combinations fulfilling previous steps, select the one with the smallest number of component carrier.

-	If there are multiple combinations fulfilling previous steps, select the one with the widest carrier being adjacent to the lowest carrier.

-	If there are multiple combinations fulfilling previous steps, select the one with the widest carrier being adjacent to the highest carrier

-	If there are multiple combinations fulfilling previous steps, select the one with the widest carrier being adjacent to the carrier which has been selected in the previous step.

-	If there are multiple combinations fulfilling previous steps, repeat the previous step until there is only one combination left.

-	The nominal carrier spacing defined in clause 5.7.1A shall apply.

#### 4.10.2.2	ETC2 power allocation

Set the power spectral density of each carrier to be the same level so that the sum of the carrier powers equals the rated total output power for E-UTRA according to the manufacturer’s declaration in sub clause 4.6.8.

### 4.10.3	ETC3: Non-contiguous spectrum operation

The purpose of ETC3 is to test all BS requirements excluding CA occupied bandwidth.

For ETC3 used in receiver tests, outermost carriers for each sub-block need to be generated by the test equipment.

#### 4.10.3.1	ETC3 generation

ETC3 is constructed on a per band basis using the following method:

-	The RF bandwidth shall be the maximum supported RF bandwidth for non-contiguous spectrum operation. The RF bandwidth consists of one sub-block gap and two sub-blocks located at the edges of the declared maximum supported RF bandwidth.

-	For transmitter tests, place a 5MHz carrier adjacent to the upper edge of the RF bandwidth and a 5MHz carrier adjacent to the lower edge of the RF bandwidth. If 5 MHz carriers are not supported by the BS, the narrowest supported channel BW shall be selected instead.

-	For receiver tests, place a 5MHz carrier adjacent to the upper edge of the RF bandwidth and a 5MHz carrier adjacent to the lower edge of the RF bandwidth. If 5 MHz E-UTRA carriers are not supported by the BS, the narrowest supported channel BW shall be selected instead.

-	For single-band operation receiver tests, if the remaining gap is at least 15 MHz plus two times the channel BW used in the previous step and the BS supports at least 4 carriers, place a carrier of this BW adjacent to each already placed carrier for each sub-block. The nominal carrier spacing defined in subclause 5.7 shall apply.

-	The sub-block edges adjacent to the sub-block gap shall be determined using the specified FOffset for the carrier adjacent to the sub-block gap.

#### 4.10.3.2	ETC3 power allocation

Set the power of each carrier to the same level so that the sum of the carrier powers equals the rated total output power according to the manufacturer’s declaration in subclause 4.6.8.

#### 4.10.3.24	VOID

### 4.10.4	ETC4: Multi-band test configuration for full carrier allocation

The purpose of ETC4 is to test multi-band operation aspects considering maximum supported number of carriers.

##### 4.10.4.1 	ETC4 generation

ETC4 is based on re-using the existing test configuration applicable per band involved in multi-band operation. It is constructed using the following method:

-	The RF bandwidth of each supported operating band in multi-band operation shall be the declared maximum RF bandwidth of each supported operating band in multi-band operation.

-	The number of carriers of each supported operating band shall be the declared maximum number of supported carriers of each supported operating band in multi-band operation. Carriers shall first be placed at the outermost edges of the declared maximum radio bandwidth. Additional carriers shall next be placed at the edges of the RF bandwidths, if possible.

-	The allocated RF bandwidth of the outermost bands shall be located at the outermost edges of the declared maximum radio bandwidth.

-	Each concerned band shall be considered as an independent band and the corresponding ETC1 shall be generated in each band. The mirror image of the single-band test configuration shall be used in the highest band being tested for the BS to ensure a narrowband carrier being placed at both edges of the BS maximum radio bandwidth.

-	If a multi-band BS supports only 3 carriers, two carriers shall be placed in one band according to ETC1 while the remaining carrier shall be placed at the edge of the maximum radio bandwidth in the other band.

-	If the sum of the maximum RF bandwidth of each supported operating bands is larger than the declared total RF bandwidth of transmitter and receiver for the declared band combinations of the BS, repeat the steps above for test configurations where the RF bandwidth of one of the operating band shall be reduced so that the total RF bandwidth of transmitter and receiver is not exceeded and vice versa.

-	If the sum of the maximum number of supported carrier of each supported operating bands in multi-band operation is larger than the declared total number of supported carriers for the declared band combinations of the BS, repeat the steps above for test configurations where in each test configuration the number of carriers of one of the operating band shall be reduced so that the total number of supported carriers is not be exceeded and vice versa.

##### 4.10.4.2 	ETC4 power allocation

Unless otherwise stated, set the power of each carrier in all supported operating bands to the same power so that the sum of the carrier powers equals the total output power according to the manufacturer’s declaration.

If the allocated power of a supported operating band(s) exceeds the declared rated total output power of the operating band(s) in multi-band operation, the exceeded part shall, if possible, be reallocated into the other band(s). If the power allocated for a carrier exceeds the rated output power declared for that carrier, the exceeded power shall, if possible, be reallocated into the other carriers.

### 4.10.5	ETC5: Multi-band test configuration with high PSD per carrier

The purpose of ETC5 is to test multi-band operation aspects considering higher PSD cases with reduced number of carriers.

#### 4.10.5.1	ETC5 generation

ETC5 is based on re-using the existing test configuration applicable per band involved in multi-band operation. It is constructed using the following method:

-	The RF bandwidth of each supported operating band in multi-band operation shall be the declared maximum RF bandwidth of each supported operating band in multi-band operation.

-	The allocated RF bandwidth of the outermost bands shall be located at the outermost edges of the declared maximum radio bandwidth.

-	The maximum number of carriers is limited to two per band.  Carriers shall first be placed at the outermost edges of the declared maximum radio bandwidth. Additional carriers shall next be placed at the edges of the RF bandwidths, if possible.

-	Narrowest supported E-UTRA channel bandwidth shall be used in the test configuration.

-	Each concerned band shall be considered as an independent band and the corresponding ETC3 shall be generated in each band. The mirror image of the single-band test configuration shall be used in the highest band being tested for the BS to ensure a narrowband carrier being placed at both edges of the BS maximum radio bandwidth.

-	If the sum of the maximum RF bandwidth of each supported operating bands is larger than the declared total RF bandwidth of transmitter and receiver for the declared band combinations of the BS, repeat the steps above for test configurations where the RF bandwidth of one of the operating band shall be reduced so that the total RF bandwidth of transmitter and receiver is not exceeded and vice versa.

#### 4.10.5.2	ETC5 power allocation

Unless otherwise stated, set the power of each carrier in all supported operating bands to the same power so that the sum of the carrier powers equals the total output power according to the manufacturer’s declaration.

If the allocated power of a supported operating band(s) exceeds the declared rated total output power of the operating band(s) in multi-band operation, the exceeded part shall, if possible, be reallocated into the other band(s). If the power allocated for a carrier exceeds the rated output power declared for that carrier, the exceeded power shall, if possible, be reallocated into the other carriers.

## 4.11	Applicability of test configurations

The present clause defines for each RF test requirement the set of mandatory test configurations which shall be used for demonstrating conformance. The applicable test configurations are specified in the tables below for each the supported RF configuration, which shall be declared according to subclause 4.6.8. The generation and power allocation for each test configuration is defined in subclause 4.10.

For a BS declared to be capable of single carrier operation only, a single carrier (SC) shall be used for testing.

For a BS declared to be capable of multi-carrier and/or CA operation in contiguous spectrum operation only, the test configurations in Table 4.11-1 shall be used for testing.

# Table 4.11-1: Test configurations for a BS capable of multi-carrier and/or CA operation in contiguous spectrum only

| BS test case | Contiguous spectrum capable BS |
| --- | --- |
| 6.2 Base station output power | ETC1 |
| 6.3 Output power dynamics | SC |
| 6.4 Transmit ON/OFF power (only applied for E-UTRA TDD BS) | ETC1 |
| 6.5 Transmitted signal quality | - |
| 6.5.1 Frequency error | ETC1 |
| 6.5.2 Error Vector Magnitude | ETC1 |
| 6.5.3 Time alignment error | ETC1 |
| 6.5.4 DL RS power | SC |
| 6.6 Unwanted emissions | - |
| 6.6.1 Occupied bandwidth | SC, ETC2 (Note) |
| 6.6.2 Adjacent Channel Leakage power Ratio (ACLR) | ETC1 |
| 6.6.3 Operating band unwanted emissions | ETC1 |
| 6.6.4 Transmitter spurious emissions | ETC1 |
| 6.7 Transmitter intermodulation | ETC1 |
| 7.2 Reference sensitivity level | SC |
| 7.3 Dynamic range | SC |
| 7.4 In-channel selectivity | SC |
| 7.5 Adjacent Channel Selectivity(ACS) and narrow-band blocking | ETC1 |
| 7.6 Blocking | ETC1 |
| 7.7 Receiver spurious emissions | ETC1 |
| 7.8 Receiver intermodulation | ETC1 |
| Note: ETC2 is only applicable when contiguous CA is supported. |  |

For a BS declared to be capable of multi-carrier and/or CA operation in contiguous and non-contiguous spectrum and where the parameters in the manufacture’s declaration according to subclause 4.6.8 are identical for contiguous (C) and non-contiguous (NC) spectrum operation, the test configurations in the second column of Table 4.11-2 shall be used for testing.

For a BS declared to be capable of multi-carrier and/or CA operation in contiguous and non-contiguous spectrum and where the parameters in the manufacture’s declaration according to subclause 4.6.8 are not identical for contiguous and non-contiguous spectrum operation, the test configurations in the third column of Table 4.11-2 shall be used for testing.

# Table 4.11-2: Test configuration for a BS capable of multi-carrier and/or CA operation in both contiguous and non-contiguous spectrum

| BS test case | C and NC capable BS with identical parameters | C and NC capable BS with different parameters |
| --- | --- | --- |
| 6.2 Base station output power | ETC1 | ETC1, ETC3 |
| 6.3 Output power dynamics | SC | SC |
| 6.4 Transmit ON/OFF power (only applied for E-UTRA TDD BS) | ETC1 | ETC1, ETC3 |
| 6.5 Transmitted signal quality | - | - |
| 6.5.1 Frequency error | ETC1 | ETC1, ETC3 |
| 6.5.2 Error Vector Magnitude | ETC1 | ETC1, ETC3 |
| 6.5.3 Time alignment error | ETC1 | ETC1, ETC3 |
| 6.5.4 DL RS power | SC | SC |
| 6.6 Unwanted emissions | - | - |
| 6.6.1 Occupied bandwidth | SC, ETC2 (Note) | SC, ETC2 (Note) |
| 6.6.2 Adjacent Channel Leakage power Ratio (ACLR) | ETC1, ETC3 | ETC1, ETC3 |
| 6.6.2.2 Cumulative ACLR requirement in non-contiguous spectrum | ETC3 | ETC3 |
| 6.6.3 Operating band unwanted emissions | ETC1, ETC3 | ETC1, ETC3 |
| 6.6.4 Transmitter spurious emissions | ETC3 | ETC1, ETC3 |
| 6.7 Transmitter intermodulation | Same TC as used in 6.6 | Same TC as used in 6.6 |
| 7.2 Reference sensitivity level | SC | SC |
| 7.3 Dynamic range | SC | SC |
| 7.4 In-channel selectivity | SC | SC |
| 7.5 Adjacent Channel Selectivity(ACS) and narrow-band blocking | ETC3 | ETC1, ETC3 |
| 7.6 Blocking | ETC3 | ETC1, ETC3 |
| 7.7 Receiver spurious emissions | ETC3 | ETC1, ETC3 |
| 7.8 Receiver intermodulation | ETC3 | ETC1, ETC3 |
| Note: ETC2 is only applicable when contiguous CA is supported. |  |  |

### For a BS declared to be capable of multi-band operation, the test configuration in Table 4.11-3 shall be used for testing. In the case where multiple bands are mapped on common antenna connector, the test configuration in the second column of Table 4.11-3 shall be used. In the case where multiple bands are mapped on separate antenna connectors, the test configuration in the third column of Table 4.11-3 shall be used.

# Table 4.11-3: Test configuration for a BS capable of multi-band operation

| BS test case | Test configuration |  |
| --- | --- | --- |
|  | Common antenna connector | Separate antenna connector |
| 6.2 Base station output power | ETC1/3 (Note 1), ETC4 | ETC1/3 (Note 1), ETC4 |
| 6.3 Output power dynamics |  |  |
| 6.3.1 RE Power control dynamic range | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| 6.3.2 Total power dynamic range | SC | SC |
| 6.4 Transmit ON/OFF power (only applied for E-UTRA TDD BS) | ETC4 | ETC4 |
| 6.5 Transmitted signal quality |  |  |
| 6.5.1 Frequency error | ETC1/3 (Note 1), ETC4 | ETC1/3 (Note 1), ETC4 |
| 6.5.2 Error Vector Magnitude | ETC1/3 (Note 1), ETC4 | ETC1/3 (Note 1), ETC4 |
| 6.5.3 Time alignment error | ETC1/3 (Note 1), ETC5 (Note 2) | ETC1/3 (Note 1), ETC5 (Note 2) |
| 6.5.4 DL RS power | SC | SC |
| 6.6 Unwanted emissions |  |  |
| 6.6.1 Occupied bandwidth | SC, ETC2 (Note 3) | SC, ETC2 (Note 3) |
| 6.6.2 Adjacent Channel Leakage power Ratio (ACLR) | ETC1/3 (Note 1), ETC5 (Note 4) | ETC1/3 (Note 1, 5), ETC5 (Note 4, 5) |
| 6.6.2.6 Cumulative ACLR requirement in non-contiguous spectrum | ETC3 (Note 1), ETC5 (Note 4) | ETC3 (Note 1, 5) |
| 6.6.3 Operating band unwanted emissions | ETC1/3 (Note 1), ETC5 | ETC1/3 (Note 1, 5), ETC5 (Note 5) |
| 6.6.4 Transmitter spurious emissions | ETC1/3 (Note 1), ETC5 | ETC1/3 (Note 1, 5), ETC5 (Note 5) |
| 6.7 Transmitter intermodulation | ETC1/3 (Note 1) | ETC1/3 (Note 1, 5) |
| 7.2 Reference sensitivity level | SC | SC |
| 7.3 Dynamic range | SC | SC |
| 7.4 In-channel selectivity | SC | SC |
| 7.5 Adjacent Channel Selectivity(ACS) and narrow-band blocking | ETC5 | ETC1/3 (Note 1), ETC5 (Note 6) |
| 7.6 Blocking | ETC5 | ETC1/3 (Note 1), ETC5 (Note 6) |
| 7.7 Receiver spurious emissions | ETC1/3 (Note 1), ETC5 | ETC1/3 (Note 1, 5), ETC5 (Note 5) |
| 7.8 Receiver intermodulation | ETC5 | ETC1/3 (Note 1), ETC5 (Note 6) |
| Note 1: ETC1 and/or ETC3 shall be applied in each supported operating band according to Tables 4.11-1 and 4.11-2. Note 2: ETC5 is only applicable when inter-band CA is supported. Note 3: ETC2 is only applicable when contiguous CA is supported. Note 4: ETC5 may be applied for Inter RF bandwidth gap only. Note 5: Single-band requirement apply to each antenna connector for both multi-band operation test and single-band operation test. For single-band operation test, other antenna connector(s) is (are) terminated. Note 6: ETC5 is only applicable for multi-band receiver. |  |  |

## 4.12	Requirements for BS capable of multi-band operation

For BS capable of multi-band operation, the RF requirements in clause 6 and 7 apply for each supported operating band unless otherwise stated. For some requirements it is explicitly stated that specific additions or exclusions to the requirement apply for BS capable of multi-band operation.

For BS capable of multi-band operation, various structures in terms of combinations of different transmitter and receiver implementations (multi-band or single band) with mapping of transceivers to one or more antenna port(s) in different ways are possible. In the case where multiple bands are mapped on separate antenna connectors, the following applies:

-	Single-band ACLR, operating band unwanted emissions, transmitter spurious emissions, transmitter intermodulation and receiver spurious emissions requirements apply to each antenna connector.

-	If the BS is configured for single-band operation, single-band requirements shall apply to the antenna connector configured for single-band operation and no exclusions or provisions for multi-band capable BS are applicable. Single-band requirements are tested separately at the antenna connector configured for single-band operation, with all other antenna connectors terminated.

For a BS capable of multi-band operation supporting bands for TDD, the RF requirements in the present specification assume synchronized operation, where no simultaneous uplink and downlink occur between the supported operating bands.

<End of change>