3GPP TSG-RAN WG4 Meeting #70	R4-140577

Prague, Czech, February 10th – 14th, 2014

Agenda item:	7.28

Source: 	Intel Corporation

Title: 	Higher order IMD and harmonic table for UE self-desensitization analysis

Document for:	Approval

## 1	Introduction

In the RAN4-UE-RF-69AH meeting, it was endorsed to include IMD5 into 2UL inter-band CA Class A4 consideration [1]. The meantime, other higher order IMDs were discussed regarding UE self-desensitization concern. In this contribution, further study is discussed for IMD4, IMD6 and IMD7 analysis.

## 2	Discussion

2.1 IMD and harmonic analysis table

When two PAs are transmitting simultaneously, Harmonics and intermodulation products could fall into UE own DL spectrum and desensitize receiver performance significantly, not only from IMD2 and IMD3, but also other orders of IMD. Previous study showed that higher order IMDs could impact on UE self- desensitization significantly [1]. In this contribution, a further analysis is discussed.

A table is established regarding IMD4, IMD6 and IMD7. It is realized that IMD products with the sum of frequency components would not cause self-desensitization, such as I 2 x fx + 2 x fy I or I 5 x fx +1x fy I for Band x and Band y. Therefore IMD products with the difference of frequency components are listed in Table 1, such as I 2 x fx - 2 x fy I and I 5 x fx - 1x fy I. This table also includes harmonics from 4th order to complete the table. 6th and 7th order harmonics are not included.

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) |  |  |  |  |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ |  |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 6th order IMD products | ¦5*fx_low –1* fy_high¦ | ¦5*fx_high – 1*fy_low¦ | ¦5*fy_low – 1*fx_high¦ | ¦5*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 6th order IMD products | ¦4*fx_low - 2* fy_high¦ | ¦4*fx_high - 2* fy_low¦ | ¦4*fy_low - 2*fx_high¦ | ¦4*fy_high - 2*fx_low¦ |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 6th order IMD products | ¦3*fx_low –3* fy_high¦ |  | ¦3*fx_high – 3*fy_low¦ |  |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 7th order IMD products | ¦6*fx_low –1* fy_high¦ | ¦6*fx_high – 1*fy_low¦ | ¦6*fy_low – 1*fx_high¦ | ¦6*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 7th order IMD products | ¦5*fx_low - 2* fy_high¦ | ¦5*fx_high - 2* fy_low¦ | ¦5*fy_low - 2*fx_high¦ | ¦5*fy_high - 2*fx_low¦ |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 7th order IMD products | ¦4*fx_low –3* fy_high¦ | ¦4*fx_high – 3*fy_low¦ | ¦4*fy_low – 3*fx_high¦ | ¦4*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) |  |  |  |  |

Table 1: Band x and Band y UL higher order harmonics and IMD products

2.2 2UL inter-band CA spectrum analysis

Table 2 is used for existing 2UL inter-band CA combination. Table 2 shows the self-desens affected band combination from IMD4, IMD6 and IMD7.

| Class |  | Band Combination | DL impacted by IMDs |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | IMD4 | IMD6 | IMD7 |
| Dual-uplink Inter-band CA | A1 | B1+B5 | Yes |  |  |
|  |  | B1+B19 | Yes |  |  |
|  |  | B3+B20 | Yes |  | Yes |
|  |  | B7 +B28 |  |  |  |
|  | A2 | B3+B8 | Yes |  | Yes |
|  |  | B4+B12 | Yes |  |  |
|  |  | B4+B17 |  |  |  |
|  | A3 | B1+B7 |  | Yes |  |
|  |  | B3+B7 | Yes | Yes |  |
|  |  | B4+B7 |  |  |  |
|  |  | B5+B12 |  |  |  |
|  |  | B5+B17 |  |  |  |
|  | A4 | B2+B4 |  |  | Yes |
|  |  | B3+B5 | Yes |  |  |
|  |  | B3+B19 | Yes |  | Yes |
|  |  | B3+B26 | Yes |  |  |
|  |  | B5+B7 |  |  |  |
|  |  | B7+B20 |  |  | Yes |
|  | A5 | B1+B21 |  | Yes |  |
|  |  | B19+B21 |  |  |  |

Table 2: Higher order IMD impact on self-downlink

## 3	Conclusion

This contribution proposes to include a table analysing higher order harmonic and intermodulation analysis for 2UL inter-band CA combinations up to 7th order. Also, a summary of impacted band combinations is presented from existing 2UL inter-band CA combinations. An associated text proposal is attached.

## 4	References

- RAN4-UE-RF-AH69, “Consideration of higher order IMD for 2UL Inter-Band CA from UE Perspective”, Intel Corporation.

Text proposal for TR 36.860

********************** Start of text proposal to TR 36.860 Chapter 5.2.4.3 ***********************

## 5.2.4.3 Impact to DL performance

Cross-modulation is always close to own uplink. Thus the cross-modulation power can cause performance degradation to own downlink for the CA bands that have very narrow TX-RX gap. In these cases the cross-modulation could reach on top of the downlink or very close to the downlink. More work is needed to fully evaluate impact to DL performance.

OBSERVATION 4: More work is needed to fully evaluate the impact to DL performance

When two PAs are transmitting simultaneously, Harmonics and intermodulation products could fall into UE own DL spectrum and desensitize receiver performance significantly, not only from IMD2 and IMD3, but also other orders of IMD.

A table is established regarding IMD4, IMD6 and IMD7. It is realized that IMD products with the sum of frequency components would not cause self-desensitization, such as I 2 x fx + 2 x fy I or I 5 x fx +1x fy I for Band x and Band y. Therefore IMD products with the difference of frequency components are listed in Table 1, such as I 2 x fx - 2 x fy I and I 5 x fx - 1x fy I. This table also includes harmonics from 4th order to complete the table. 6th and 7th order harmonics are not included.

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) |  |  |  |  |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ |  |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 6th order IMD products | ¦5*fx_low –1* fy_high¦ | ¦5*fx_high – 1*fy_low¦ | ¦5*fy_low – 1*fx_high¦ | ¦5*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 6th order IMD products | ¦4*fx_low - 2* fy_high¦ | ¦4*fx_high - 2* fy_low¦ | ¦4*fy_low - 2*fx_high¦ | ¦4*fy_high - 2*fx_low¦ |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 6th order IMD products | ¦3*fx_low –3* fy_high¦ |  | ¦3*fx_high – 3*fy_low¦ |  |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 7th order IMD products | ¦6*fx_low –1* fy_high¦ | ¦6*fx_high – 1*fy_low¦ | ¦6*fy_low – 1*fx_high¦ | ¦6*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 7th order IMD products | ¦5*fx_low - 2* fy_high¦ | ¦5*fx_high - 2* fy_low¦ | ¦5*fy_low - 2*fx_high¦ | ¦5*fy_high - 2*fx_low¦ |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 7th order IMD products | ¦4*fx_low –3* fy_high¦ | ¦4*fx_high – 3*fy_low¦ | ¦4*fy_low – 3*fx_high¦ | ¦4*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) |  |  |  |  |

Table 5.2.4.3-1: Band x and Band y UL higher order harmonics and IMD products

Table 5.2.4.3-2 shows the self-desens affected band combination from IMD4, IMD6 and IMD7, based on available 2UL inter-band combination from RAN#62 plenary.

| Class |  | Band Combination | DL impacted by IMDs |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | IMD4 | IMD6 | IMD7 |
| Dual-uplink Inter-band CA | A1 | B1+B5 | Yes |  |  |
|  |  | B1+B19 | Yes |  |  |
|  |  | B3+B20 | Yes |  | Yes |
|  |  | B7 +B28 |  |  |  |
|  | A2 | B3+B8 | Yes |  | Yes |
|  |  | B4+B12 | Yes |  |  |
|  |  | B4+B17 |  |  |  |
|  | A3 | B1+B7 |  | Yes |  |
|  |  | B3+B7 | Yes | Yes |  |
|  |  | B4+B7 |  |  |  |
|  |  | B5+B12 |  |  |  |
|  |  | B5+B17 |  |  |  |
|  | A4 | B2+B4 |  |  | Yes |
|  |  | B3+B5 | Yes |  |  |
|  |  | B3+B19 | Yes |  | Yes |
|  |  | B3+B26 | Yes |  |  |
|  |  | B5+B7 |  |  |  |
|  |  | B7+B20 |  |  | Yes |
|  | A5 | B1+B21 |  | Yes |  |
|  |  | B19+B21 |  |  |  |

Table 5.2.4.3-2: Higher order IMD impact on self-downlink

### 5.2.5		Transmit power

********************** End of text proposal ***********************