TSG-RAN 4 #70	 R4-140863

Prague, Czech Republic,  February 10- 14, 2014

Source:	Sprint

Title:	TP for TR 36.833-5-41:  Blocking Min Requirements

Agenda item:	7.16

Document for:	Approval

# Introduction

LTE Advanced intra-band contiguous Carrier Aggregation in Band 41 for 3DL work item was approved at TSG RAN #61 [1]. This contribution discusses the impact of introducing 3-CC DL intra-band contiguous CA in Band 41 on Band 41 Blocking Minimum Requirements, and proposes text for the Technical Report 36.833-5-41 for WI LTE_CA_C_B41_3DL. [2].

# 2. Discussion

Based on the in-band blocking discussion contained in R4-140166,‘View on receiver requirements for CA Bandwidth class D’, from Samsung, it is proposed to adopt the in-band blocking requirements derived in [3] as shown in the text proposal below.

# 3. Summary

It is proposed to adopt the text proposal below for inclusion into TR 36.833-5-41.

# 4. References

- RP-131244, New WID – LTE Advanced intra-band contiguous Carrier Aggregation in Band 41 for 3DL, RAN#66, August 2013

- 36.833-5-41 v0.2.0 Technical Report LTE Advanced Carrier Aggregation in Band 41 for 3DL

- R4-140166 View on receiver requirements for CA Bandwidth class D, Samsung

# Text Proposal

-- Start of text proposal --

Table 7.6.1.1A-1: In band blocking parameters

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Power per CC in Aggregated Transmission Bandwidth Configuration | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 | 12 | 13.8 |  |  |
| BWInterferer | MHz | 5 | 5 | 5 |  |  |
| FIoffset, case 1 | MHz | 7.5 | 7.5 | 7.5 |  |  |
| FIoffset, case 2 | MHz | 12.5 | 12.5 | 12.5 |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L or PCMAX_L_CA as defined in subclause 6.2.5A NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |  |  |  |

Table 7.6.1.1A-2: In-band blocking

| CA configuration |  | Parameter | Unit | Case 1 | Case 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PInterferer | dBm | -56 | -44 |  |
|  |  | FInterferer | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |  |
| CA_1C, CA_3C, CA_7C, CA_23B, CA_27B, CA_38C, CA_40C, CA_41C, CA_41D |  | FInterferer (Range) | MHz | (Note 2) | FDL_low – 15 to FDL_high + 15 |  |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -Foffset - FIoffset, case 1 and b. the carrier frequency +Foffset + FIoffset, case 1 NOTE 3: Foffset is the frequency offset from the center frequency of the adjacent CC being tested to the edge of aggregated channel bandwidth. NOTE 4: The Finterferer (offset) is relative to the center frequency of the adjacent CC being tested and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

-- End of text proposal --

form change history:

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"