TSG-RAN 4 #70	 R4-141173

Prague, Czech Republic,  February 10- 14, 2014

Source:	Sprint

Title:	TP for TR 36.833-5-41:  Spurious Response

Agenda item:	7.16

Document for:	Approval

# Introduction

LTE Advanced intra-band contiguous Carrier Aggregation in Band 41 for 3DL work item was approved at TSG RAN #61 [1]. This contribution discusses the impact of introducing 3-CC DL intra-band contiguous CA in Band 41 on Band 41 spurious response, and proposes text for the Technical Report 36.833-5-41 for WI LTE_CA_C_B41_3DL. [2].

# Discussion

There is no aggregated bandwidth dependency for spurious response.  Section 7.7.1A, Minimum Requirements for CA, list CA bandwidth class B and C as having the same requirements.  It is proposed to extend these requirements to CA bandwidth Class D with no changes from Class C.

# 3. Summary

It is proposed to adopt the text proposal below for inclusion into TR 36.833-5-41.

# 4. References

- RP-131244, New WID – LTE Advanced intra-band contiguous Carrier Aggregation in Band 41 for 3DL, RAN#66, August 2013

- 36.833-5-41 v0.2.0 Technical Report LTE Advanced Carrier Aggregation in Band 41 for 3DL

# Text Proposal

-- Start of text proposal --

## 6.1	Required changes to TS36.101

Required changes in UE RF specification TS 36.101 are shown in Table 6.1-1.

Table 6.1-1: Required changes in TS 36.101

| Section | Requirement | Required changes in TS 36.101 |
| --- | --- | --- |
| 5.6A | Channel BW for CA | BW class D not defined in Table 5.6A-1. Need to agree on proper definition of Class D |
| 5.6A.1 | Channel bandwidths per operating band for CA | info for CA-B41D not included in Table 5.6A.1-1. Table will need to be modified (or totally different table) to accommodate 3 channels. |
| 5.7.1A | Channel spacing for CA | only defined for Class C .. Need to define for D. |
| Table 7.3.1A-1 | REFSENS | Need to define for CA_41D |
| 7.4.1A | Max input level Minimum requirements for CA | Need to define for CA_41D |
| 7.5.1A | ACS Min requirements for CA | Need to define for CA_41D |
| 7.6.1.1A | Blocking Min requirements for CA | Need to define for CA_41D |
| 7.6.2 | Out-of-band blocking | Need to define for CA_41D |
| 7.6.3 | Narrow band blocking | Need to define for CA_41D |
| 7.7.1 | Spurious response | Need to define for CA_41D |
| 7.8 | Intermods | Need to define for CA_41D |
| 7.10 | Rx Image | Need to define for CA_41D |

Table 7.7.1A-1: Spurious response parameters

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Power per CC in Aggregated Transmission Bandwidth Configuration | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 | 9 | 9 |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L or PCMAX_L_CA as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |

-- End of text proposal --

form change history:

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"