TSG-RAN 4 #70	 R4-141255

Prague, Czech Republic,  February 10- 14, 2014

Source:	Sprint

Title:	TP for TR 36.833-5-41:  Blocking Min Requirements

Agenda item:	7.16

Document for:	Approval

# Introduction

LTE Advanced intra-band contiguous Carrier Aggregation in Band 41 for 3DL work item was approved at TSG RAN #61 [1]. This contribution discusses the impact of introducing 3-CC DL intra-band contiguous CA in Band 41 on Band 41 Blocking Minimum Requirements, and proposes text for the Technical Report 36.833-5-41 for WI LTE_CA_C_B41_3DL. [2].

# 2. Discussion

Based on the in-band blocking discussion contained in R4-140166,‘View on receiver requirements for CA Bandwidth class D’, from Samsung, it is proposed to adopt the in-band blocking requirements derived in [3] as shown in the text proposal below.  To account for the added difficulty of the baseband filter impact on in-band blocking when the aggregated bandwidth is wider, the in-band blocking requirements are scaled for the number of component carriers.  The relaxation can be revisited if the approach of scaling up the wanted CC power according to increased BW is shown to be not justified.

# 3. Summary

It is proposed to adopt the text proposal below for inclusion into TR 36.833-5-41.

# 4. References

- RP-131244, New WID – LTE Advanced intra-band contiguous Carrier Aggregation in Band 41 for 3DL, RAN#66, August 2013

- 36.833-5-41 v0.2.0 Technical Report LTE Advanced Carrier Aggregation in Band 41 for 3DL

- R4-140166 View on receiver requirements for CA Bandwidth class D, Samsung

# Text Proposal

-- Start of text proposal --

To account for the added difficulty of the baseband filter impact on in-band blocking when the aggregated bandwidth is wider, the in-band blocking requirements are scaled for the number of component carriers.  The relaxation can be revisited if the approach of scaling up the wanted CC power according to increased BW is shown to be not justified.

Table 7.6.1.1A-1: In band blocking parameters

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Power per CC in Aggregated Transmission Bandwidth Configuration | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 | 12 | 13.8 |  |  |
| BWInterferer | MHz | 5 | 5 | 5 |  |  |
| FIoffset, case 1 | MHz | 7.5 | 7.5 | 7.5 |  |  |
| FIoffset, case 2 | MHz | 12.5 | 12.5 | 12.5 |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L or PCMAX_L_CA as defined in subclause 6.2.5A NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |  |  |  |

Table 7.6.1.1A-2: In-band blocking

| CA configuration |  | Parameter | Unit | Case 1 | Case 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PInterferer | dBm | -56 | -44 |  |
|  |  | FInterferer | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |  |
| CA_1C, CA_3C, CA_7C, CA_23B, CA_27B, CA_38C, CA_40C, CA_41C, CA_41D |  | FInterferer (Range) | MHz | (Note 2) | FDL_low – 15 to FDL_high + 15 |  |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -Foffset - FIoffset, case 1 and b. the carrier frequency +Foffset + FIoffset, case 1 NOTE 3: Foffset is the frequency offset from the center frequency of the adjacent CC being tested to the edge of aggregated channel bandwidth. NOTE 4: The Finterferer (offset) is relative to the center frequency of the adjacent CC being tested and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

-- End of text proposal --

form change history:

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"