3GPP TSG-RAN WG4 Meeting #70	R4-141204

Prague, Czech, February 10th – 14th, 2014

Agenda item:	7.32

Source: 	Intel Corporation, MediaTek Inc., Qualcomm Incorporated, Broadcom          Corporation, Nokia Corporation

Title: 	IMD5 analysis in Class A4 for 2UL inter-band CA

Document for:	Approval

## 1	Introduction

In the RAN4-UE-RF-69AH meeting, it was endorsed to include IMD5 into Class A4 consideration for 2UL inter-band CA [1]. This contribution provides a table for IMD5 analysis.

## 2	Discussion

When two PAs are transmitting simultaneously, intermodulation products could fall into UE own DL spectrum and desensitize receiver performance significantly, not only from IMD2 and IMD3, but also other orders of IMD. IMD5 needs to be considered for UE self-desensitization in 2UL inter-band CA cases.[1] Therefore a table is established to analyse IMD5 as shown in Table 1. It is also realized that IMD products with the sum of frequency components would not cause self-desensitization, such as I 3 x fx + 2 x fy I for Band x and Band y. Therefore only IMD products with the difference of frequency components are included, such as I 3 x fx - 2 x fy I.

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) |  |  |  |  |

Table 1: Band x and Band y UL IMD5 products

Proposal1: IMD5 shall be considered for self-desensitization analysis in 2UL inter-band CA. The self-desensitization due to IMD5 of each 2UL CA combination shall be investigated based on case-by-case study. The class allocation of each 2UL CA combination shall not be modified by this proposal.

Proposal2: IMD5 analysis table is provided to be included for self-desensitization analysis in 2UL inter-band CA.

## 3	Conclusion

This contribution presents the 5th order intermodulation analysis for Class A4 band combinations.

Proposal1: IMD5 shall be considered for self-desensitization analysis in 2UL inter-band CA. The self-desensitization due to IMD5 of each 2UL CA combination shall be investigated based on case-by-case study. The class allocation of each 2UL CA combination shall not be modified by this proposal.

Proposal2: IMD5 analysis table is provided to be included for self-desensitization analysis in 2UL inter-band CA.

## 4	References

- R4-69AH-0030, “Consideration of higher order IMD for 2UL Inter-Band CA from UE Perspective”, Intel Corporation. RAN4-UE-RF-AH69, Austin, US.