3GPP TSG-RAN4 Meeting #71 	R4-142692

# Seoul, Korea, May 19 - 23, 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 2367 | rev | - | Current version: | 10.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial Correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Alcatel-Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10 |  |  |  |  |  |  | Date: |  | 2014-05-05 |
| Category: | D |  |  |  |  |  |  | Release: |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | The unit for RSRQ is dB. However, the unit for RSRQ is set incorrectly to dBm in the tables: Table 9.1.5.1-1, Table 9.1.5.2-1, Table 9.1.6.1-1, Table 9.1.6.2-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the RSRQ unit to dB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Isolated editorial correction |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Sections 9.1.5, and 9.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of first changes>

### 9.1.5	Intra-frequency RSRQ Accuracy Requirements

#### 9.1.5.1	Absolute RSRQ Accuracy

The requirements for absolute accuracy of RSRQ in this section apply to a cell on the same frequency as that of the serving cell.

The accuracy requirements in Table 9.1.5.1-1 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 Section 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.1 for a corresponding Band

Table 9.1.5.1-1: RSRQ Intra frequency absolute accuracy

| Parameter | Unit | Accuracy [dB] |  | Conditions1,2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Normal condition | Extreme condition | Bands 1, 4, 6, 10, 11, 18, 19, 21, 23, 24, 33, 34, 35, 36, 37, 38, 39, 40 | Bands 2, 5, 7, 41 | Band 25 | Bands 3, 8, 12, 13, 14, 17, 20, 22 | Bands 9, 42, 43 |
|  |  |  |  | Io | Io | Io | Io | Io |
| RSRQ when RSRP Ês/Iot ≥ -3 dB | dB | 2.5 | 4 | -121dBm/15kHz … -50dBm/ BWChannel | -119dBm/15kHz … -50dBm/ BWChannel | -117.5dBm/15kHz … -50dBm/ BWChannel | -118dBm/15kHz … -50dBm/ BWChannel | -120dBm/15kHz … -50dBm/ BWChannel |
| RSRQ when RSRP Ês/Iot ≥ -6 dB | dB | 3.5 | 4 | -121dBm/15kHz … -50dBm/ BWChannel | -119dBm/15kHz … -50dBm/ BWChannel | -117.5dBm/15kHz … -50dBm/ BWChannel | -118dBm/15kHz … -50dBm/ BWChannel | -120dBm/15kHz … -50dBm/ BWChannel |
| Note 1: Io is assumed to have constant EPRE across the bandwidth. Note 2: The minimum condition level is increased by ∆>0, when applicable, as described in Section B.4.2. |  |  |  |  |  |  |  |  |

#### 9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction

The requirements for absolute accuracy of RSRQ in this section shall apply to a cell on the same frequency as that of the serving cell when a time domain measurement resource restriction pattern for performing RSRQ measurements of this cell is configured by higher layers [2].

The accuracy requirements in Table 9.1.5.2-1 are valid under the following conditions:

Cell specific reference signals in the measured cell are transmitted either from one, two or four antenna ports,

Conditions defined in 36.101 Section 7.3 for reference sensitivity are fulfilled,

RSRP|dBm according to Annex B.3.9 for a corresponding Band,

The time domain measurement resource restriction pattern configured for the measured cell indicates at least one subframe per radio frame for performing the RSRQ measurement,

The RSRQ measurement is not performed in any subframe other than those indicated by the time domain measurement resource restriction pattern configured for the measured cell,

Four symbols containing CRS are available in all subframes indicated by the time domain measurement resource restriction pattern.

Table 9.1.5.2-1: RSRQ Intra frequency absolute accuracy under time domain measurement resource restriction

| Parameter | Unit | Accuracy [dB] |  | Conditions1,2,3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Normal condition | Extreme condition | Bands 1, 4, 6, 10, 11, 18, 19, 21, 24, 33, 34, 35, 36, 37, 38, 39, 40 | Bands 2, 5, 7, 41 | Bands 3, 8, 12, 13, 14, 17, 20, 22 | Bands 9, 42, 43 |
|  |  |  |  | Io | Io | Io | Io |
| RSRQ when RSRP Ês/Iot ≥ -2 dB | dB | 2.5 | 4 | -121dBm/15kHz … -50dBm/ BWChannel | -119dBm/15kHz … -50dBm/ BWChannel | -118dBm/15kHz … -50dBm/ BWChannel | -120dBm/15kHz … -50dBm/ BWChannel |
| RSRQ when RSRP Ês/Iot ≥ -4 dB | dB | 3.5 | 4 | -121dBm/15kHz … -50dBm/ BWChannel | -119dBm/15kHz … -50dBm/ BWChannel | -118dBm/15kHz … -50dBm/ BWChannel | -120dBm/15kHz … -50dBm/ BWChannel |
| Note 1: When in dBm/15kHz, the minimum Io condition is expressed as the average Io per RE over all REs in that symbol. Note 2: Io is defined in subframes indicated by the time domain measurement resource restriction pattern configured for performing RSRQ measurements of this cell. The Io range defined by the minimum and the maximum Io levels applies to CRS and non-CRS symbols. Io may be different in different symbols within a subframe. Note 3: The minimum condition level is increased by ∆>0, when applicable, as described in Section B.4.2. |  |  |  |  |  |  |  |

### 9.1.6	Inter-frequency RSRQ Accuracy Requirements

#### 9.1.6.1	Absolute RSRQ Accuracy

The requirements for absolute accuracy of RSRQ in this section apply to a cell that has different carrier frequency from the serving cell.

The accuracy requirements in Table 9.1.6.1-1 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 Section 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.3 for a corresponding Band

Table 9.1.6.1-1: RSRQ Inter frequency absolute accuracy

| Parameter | Unit | Accuracy [dB] |  | Conditions1,2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Normal condition | Extreme condition | Bands 1, 4, 6, 10, 11, 18, 19, 21, 23, 24, 33, 34, 35, 36, 37, 38, 39, 40 | Bands 2, 5, 7, 41 | Band 25 | Bands 3, 8, 12, 13, 14, 17, 20, 22 | Bands 9, 42, 43 |
|  |  |  |  | Io | Io | Io | Io | Io |
| RSRQ when RSRP Ês/Iot > -3 dB | dB | 2.5 | 4 | -121dBm/15kHz … -50dBm/ BWChannel | -119dBm/15kHz … -50dBm/ BWChannel | -117.5dBm/15kHz … -50dBm/ BWChannel | -118dBm/15kHz … -50dBm/ BWChannel | -120dBm/15kHz … -50dBm/ BWChannel |
| RSRQ when RSRP Ês/Iot ≥ -6 dB | dB | 3.5 | 4 | -121dBm/15kHz … -50dBm/ BWChannel | -119dBm/15kHz … -50dBm/ BWChannel | -117.5dBm/15kHz … -50dBm/ BWChannel | -118dBm/15kHz … -50dBm/ BWChannel | -120dBm/15kHz … -50dBm/ BWChannel |
| Note 1: Io is assumed to have constant EPRE across the bandwidth. Note 2: The minimum condition level is increased by ∆>0, when applicable, as described in Section B.4.2. |  |  |  |  |  |  |  |  |

#### 9.1.6.2	Relative Accuracy of RSRQ

The relative accuracy of RSRQ in inter frequency case is defined as the RSRQ measured from one cell compared to the RSRQ measured from another cell on a different frequency.

The accuracy requirements in Table 9.1.6.2-1 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 Section 7.3 for reference sensitivity are fulfilled.

RSRP1,2|dBm according to Annex B.3.4 for a corresponding Band

| Channel 1_Io -Channel 2_Io |  20 dB

Table 9.1.6.2-1: RSRQ Inter frequency relative accuracy

| Parameter | Unit | Accuracy [dB] |  | Conditions1,3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Normal condition | Extreme condition | RSRQ is on Bands 1, 4, 6, 10, 11, 18, 19, 21, 23, 24, 33, 34, 35, 36, 37, 38, 39, 40 | RSRQ is on Bands 2, 5, 7, 41 | RSRQ is on Band 25 | RSRQ is on Bands 3, 8, 12, 13, 14, 17, 20, 22 | RSRQ is on Bands 9,42, 43 |
|  |  |  |  | Io | Io | Io | Io | Io |
| RSRQ when RSRP Ês/Iot > -3 dB | dB | 3 | 4 | -121dBm/15kHz … -50dBm/ BWChannel | -119dBm/15kHz … -50dBm/ BWChannel | -117.5dBm/15kHz … -50dBm/ BWChannel | -118dBm/15kHz … -50dBm/ BWChannel | -120dBm/15kHz … -50dBm/ BWChannel |
| RSRQ when RSRP Ês/Iot ≥ -6 dB | dB | 4 | 4 | -121dBm/15kHz … -50dBm/ BWChannel | -119dBm/15kHz … -50dBm/ BWChannel | -117.5dBm/15kHz … -50dBm/ BWChannel | -118dBm/15kHz … -50dBm/ BWChannel | -120dBm/15kHz … -50dBm/ BWChannel |
| Note 1: Io is assumed to have constant EPRE across the bandwidth. Note 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells.to which the requirement applies. Note 3: The minimum condition level is increased by ∆>0, when applicable, as described in Section B.4.2. |  |  |  |  |  |  |  |  |

<End of changes>