3GPP TSG-RAN WG4 Meeting #71	 R4-142782

Seoul, Korea, May 19 - 23, 2014

Agenda item:	9.4.1

Source:		Alcatel-Lucent

Title:	Indoor and Vertical Positioning

Document for:	Discussion

# 1	Introduction

The current FCC Phase II E911 rules only require wireless service providers to provide the horizontal location (e.g., the latitude and longitude) of the E911 mobile caller. Also, the location accuracy requirements apply only for outdoor callers. Recently, FCC proposed the new E911 rules [1], in which it requires the carrier providers to locate the E911 callers indoors and provide both horizontal and vertical location. The required accuracy is horizontally within 50 meters (67% of calls) and vertically within in 3 meters (67% of calls).

In this paper, we make an initial discussion on the indoor and vertical positioning for meeting the FCC new proposal on the UE positioning performance requirements.

# Discussion

  - Horizontal Positioning Indoor

The current FCC Phase II E911 rules requires wireless providers to provide horizontal location, but the accuracy requirements apply only for outdoor wireless 911 calls. The new FCC proposal changes to the E911 rules to include location accuracy in challenging indoor environments such as large multi-story buildings, where first responders are often unable to determine the floor or even the building where the 911 call originated. Determining the location of indoor wireless callers is much more challenging than determining an outdoor location.

For outdoor environments, GNSS may provide very accurate horizontal location. However, GNSS signals are normally too weak indoors for reliable positioning, especially regular GNSS receivers. Advanced GNSS receivers, which have 10-20dB better sensitivity than regular GNSS receivers, may  work for some indoor scenarios. However, it is still no guarantee that A-GNSS receivers can see enough satellites for providing location under all indoor conditions.

E-CID and OTDOA determines a UE’s position based on the measurements of wireless RF signals. Thus, in theory E-CID and OTDOA should work under the RF conditions that a caller can make calls, including indoor environments. The current performance of E-CID and OTDOA, however, is for away from the requirements proposed by the new FCC rule. For example, the intra-frequency RSTD accuracy is defined as +/- 15Ts for carrier bandwidth smaller than 5MHz under AWGN channel. Consider 1Ts (=1/30720 ms) is about 10m in distance, 15Ts itself is already about 150m, which is much larger than the required 50m horizontal accuracy. For 10MHz carrier bandwidth, the intra-frequency accuracy is defined as +/- 5Ts under AWGN channel. The accuracy is still not good enough to meet the new FCC requirements for horizontal accuracy, due to the impacts of RF signal fading, multipath and other geometric factors.

In recent effort for the SI Positioning enhancements for E-UTRA [2-3], it was shown that the performance for narrow bandwidth (<5MHz) and wide bandwidth (>10MHz) should/could be improved. In addition, the current RSTD performance requirements were defined by adding large implementation margin (~ 4Ts) on the simulation results. The implementation margin will need to be reduced in order to use OTDOA to meet the new FCC’s proposal.

Considering the fact that advanced A-GNSS receivers may still be able to receive the signals indoors from a few GNSS satellites, a hybrid system (A-GNSS + Enhanced OTDOA) may be a good solution for meeting the new FCC rules for horizontal location.

Observation 1: In order to meet the new FCC proposal for E911 horizontal positioning accuracy indoor, the OTDOA performance needs to be enhanced. The RSTD measurement accuracy for both narrow bandwidth (<5MHz) and wide bandwidth (>10MHz) need to be redefined. Furthermore, the implementation margin needs to be reduced for the RSTD performance requirements. A hybrid system (A-GNSS + Enhanced OTDOA) may be a penitential solution for meeting the new FCC rules for horizontal location.

  - Vertical Positioning

The current FCC Phase II E911 rules do not have requirement for vertical positioning, regardless indoor or outdoor. Also, 3GPP positioning performance requirements have been focusing so far on horizontal positioning only. The new FCC proposal not only adds the requirements to vertical positioning indoor, also requires the accuracy within 3 meters for 67% of calls. Needless to say, providing the vertical location with such accuracy indoor would be a very challenging task.

2.2.1 GNSS

For outdoor scenarios with good reception of GNSS signals, GNSS receivers should be able to provide the vertical location together with horizontal location. However, it is well known that the accuracy of GNSS vertical location is worse than the horizontal location due to the nature of geometrical distribution of GNSS satellites. A simple explanation is that horizontal positioning is based on the horizontal projection of range measurements from multiple GNSS satellites, surrounding the GNSS receiver in azimuth angles from 0 to 360 degrees. But, for vertical positioning, the vertical projections of range measurements from multiple GNSS satellites relative to the GNSS receiver can only be with elevation larger than 0 degree.

For indoor scenarios, GNSS signals are generally too weak to even provide reliable horizontal location for regular GNSS receivers. Even for high-performance A-GNSS receivers, it is unlikely that GNSS can provide 3meter vertical location accuracy indoor.

Observation 2: It is unlikely for GNSS to provide the 3meter vertical location accuracy indoor.

2.2.2 OTDOA

As mentioned before, OTDOA works based on wireless signals from the cells surrounding the UE. Thus, OTDOA should be able to provide horizontal positioning both indoor and outdoor as long as the UE can make the calls. However, OTDOA in general is not suitable for vertical positioning. In theory, the RSTD measurements are the 3-dimensional distance differences from the UE to the cells. However, it is very difficult to accurately separate the vertical location differences and the horizontal location distance differences from the 3D distance difference measurements. Unlike GNSS satellites, which are distributed in different elevations, the neighboring cells of the UE are normally distributed in nearly the same altitude (except cells inside multi-story building, which may not be able to support OTDOA due to lack of synchronous timing source or use of distribute antenna systems or both.). Thus, in general it is difficult for OTDOA to get vertical location. In addition, the minimum measurement error of RSTD is +/-5Ts (~50m), which make basically impossible to use OTDOA to meet the FCC proposed vertical accuracy of 3meters.

Observation 3: It is not feasible for OTDOA to provide the 3meter vertical location accuracy indoor.

2.2.3 E-CID

For small cell deployment in multi-story buildings, the coverage of a cell is normally very small and the UE is likely to be served by the small cells in the same floor or an adjacent floor. Thus, if the vertical coordinates for the indoor small cells are known, the UE’s vertical position might be determined accurately by the RF signal strengths and the coordinates of the serving cell and neighboring cells.

Observation 4: For small cell deployment in multi-story building, small cell coordinates + RF signal strengths could be very useful for determining UE’s vertical position.

2.2.4 Vertical measurements from Barometer measurements

Barometer measurements are shown to be very effective for improving the accuracy and availability of the altitude solution [4-7]. With the development of Micro-Electro-Mechanical Systems (MEMS) technology, more and more smart phones have installed barometers. The pressure measured by barometers can be converted to altitude information, which is called pressure altitude. It is also well known that pressure sensors can be very sensitive to vertical position changes to sub-meter level. Therefore, the barometer measurements may provide a good potential to meter level accuracy in altitude.

The problem with the barometer the pressure measurements is that the measurements are very sensitive to the change of the local air pressure, which make the pressure altitude unreliable if the measurement errors cannot be removed properly.

One effective approach to make pressure altitude reliable is through the differential operation, in which the pressure altitude measured by a UE is compared with the pressure altitude from a reference barometer, which is installed at a known altitude. By the use of the differential operation, the measurement errors that have impact on both the UE and the reference barometers will be removed. Thus, accurate UE altitude information may be obtained. However, some pressure disturbances and temperature difference local to UE barometer and reference barometer may still remain.

In order to use barometer measurements for vertical positioning, it would need to install the barometers in both UE and in the network (may even in the small cells). Also, proper algorithm needs to be implemented in the location server to calibrate and track the pressure altitude error based on the measurement report (air pressure, local temperature, etc.). Obviously, more investigation is needed to evaluate the effectiveness of the vertical location based on the barometer measurements.

Observation 5: Differential barometer measurements, which requires both UE and network to provide the barometer measurements (maybe also other parameters, such as local temperature), may be a feasible approach to provide accurate vertical location. More investigation is needed to investigate the effectiveness of this approach.

# 3	Conclusion

In this paper, we present the initial discussion on indoor and vertical location for FCC’s third further notice of proposed rulemaking for wireless E911 location accuracy requirement. Our observations are:

Observation 1: In order to meet the new FCC proposal for E911 horizontal positioning accuracy indoor, the OTDOA performance needs to be enhanced. The RSTD measurement accuracy for both narrow bandwidth (<5MHz) and wide bandwidth (>10MHz) need to be redefined. Furthermore, the implementation margin needs to be reduced for the RSTD performance requirements. A hybrid system (A-GNSS + Enhanced OTDOA) may be a possible solution for meeting the new FCC rules for horizontal location.

Observation 2: It is unlikely for GNSS only to provide the 3meter vertical location accuracy indoor.

Observation 3: It is not feasible for OTDOA to provide the 3meter vertical location accuracy indoor.

Observation 4: For small cell deployment in multi-story building, small cell coordinates + RF signal strengths could be very useful for UE’s vertical position.

Observation 5: Differential barometer measurements, which requires both UE and network to provide the barometer measurements (maybe also other parameters, such as local temperature), may be a feasible approach to provide accurate vertical location. More investigation is needed to investigate the effectiveness of this approach.

Finally, we believe a hybrid positioning system, e.g., (A-GNSS + Enhanced OTDOA + barometer measurements+E-CID+RF signal strengths), may be required in order to provide accurate horizontal and vertical positioning for the FCC proposed new E911 rules.

# 4. References

- FCC, Wireless E911 Location Accuracy Requirement, THIRD FURTHER NOTICE OF PROPOSED RULEMAKING, PS Docket No. 07-114, February 21, 2014

- RP-130680, New SI proposal: Positioning enhancements for E-UTRA, Huawei, HiSilicon

- 3GPP Draft TR 36.855 Feasibility of positioning enhancements for E-UTRA

- M. Asbury, et al, “Probability distributions that are important when assessing barometric aiding to GPS,” in Proc. of the 3rd International Technical Meeting of ION GNSS, Sept. 1990, pp. 445–453.

- J. McLellan, et al, “GPS/barometry height-aided positioning system,” in Proc. of IEEE/ION PLANS, Apr. 1994, pp. 369–375.

- J. Kappi and K. Alanen, “Pressure altitude enhanced AGNSS hybrid receiver for a mobile terminal,” in Proc. of the International Technical Meeting of ION GNSS, Sept. 2005, pp. 1991–1997.

- CSRIC WG3, Indoor Location Test Bed Report, March 14, 2013