3GPP TSG-RAN WG4 RAN4 #71                                    R4 -143191

Seoul, Korea, 19th – 23rd May, 2014

Agenda item:	5.4

Source: 	Samsung

Title: 			  Simulation results for FeICIC RI test 1

Document for:	   Discussion

1	Introduction

In last meeting, it was agreed to revise current FeICIC RI test point 1 to keep this test case as receiver agonistic.

In this contribution, we provide simulation results for this modified test case.

- Analysis

Simulation assumption

As agreed in the CR, for Test 1, antenna correlation has been modified as high correlation.

Table 9.5.4.1-2 Minimum requirement (FDD)

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| for Cell 1 (dB) | 4 | 20 | 20 |
| for Cell 1 (dB[mW/15kHz]) | -94 | -78 | -78 |
| Antenna correlation | High for Cell 1, low for Cell 2 and Cell 3 | Low for Cell 1, Cell 2 and Cell 3 | High for Cell 1, low for Cell 2 and Cell 3 |
| 1 | N/A | 1.05 | 0.9 |
| 2 | TBD | N/A | N/A |
| UE Category | 2-8 | 2-8 | 2-8 |

Simulation result

Based on the updated test set-up, throughput ratio for gamma2 with MRC receiver and ML receiver has been summarized in table 1 below:

Figure1 TP-Ratio (Gamm2) Vs. SNR

Table 1: Throughput Ratio vs. SNR

| Receivers | SNR (dB) | γ2 |
| --- | --- | --- |
| MMSE | 3.00 | 1.46 |
|  | 4.00 | 1.48 |
|  | 5.00 | 1.49 |
| ML | 3.00 | 1.37 |
|  | 4.00 | 1.39 |
|  | 5.00 | 1.36 |

Based on such simulation results, we proposed to:

Defining requirements for updated test 1 as gamma2>1.05

- Conclusion

In this contribution, we provide simulation results for FeICIC RI test point 1. Based on simulation results, such proposal was given:

Defining requirements for updated test 1 as gamma2>1.05

- Reference

- R4-142337, “CR of modification on FeICIC rank testing (Rel-11)”, Huawei