3GPP TSG-RAN WG4 Meeting #71                                                                       R4-143210

# Seoul, Korea, 19 – 23 May, 2014

Source:	LG Electronics

Title:	Simulation results for interference cancellation under SU-MIMO

Agenda item:	7.12.2

Document for:	Discussion

Introduction

In last RAN4 meeting, the WF for scenario and simulation assumption was agreed [1]. In this contribution, we provide link level performance for SU-MIMO under 36.101 single cell multi-layer spatial multiplexing FRC test setups.

Link level performance

Simulation assumptions are as following table.

Table 2-1 Simulation assumption

| TM | Antenna configuration | Fading channel | Mod | Test setup reference in 36.101 |
| --- | --- | --- | --- | --- |
| TM3 | 2x2 Medium | EVA 70 | 16QAM | 8.2.1.3.1 |
|  | 4x2 Medium | EVA 70 | 16QAM | 8.2.1.3.2 |
| TM4 | 2x2 Medium | EPA 5 | 64QAM | 8.2.1.4.2 |
|  |  | ETU 70 | 16QAM | 8.2.1.4.2 |
|  | 4x2 Medium | EPA 5 | 64QAM | 8.2.1.4.3 |

The performances for R-ML and MMSE receivers are evaluated under medium antenna correlation based on 36.101 single cell multi-layer test setups. The SNR gains for R-ML receiver are summarized in Table 2-2.

Table 2-2 SNR gain (dB) at 70%-throughput for R-ML receiver

| TM | Test setup reference in 36.101 | SNR gain |
| --- | --- | --- |
| TM3 | 8.2.1.3.1 | 2.8 |
|  | 8.2.1.3.2 | 4.8 |
| TM4 | 8.2.1.4.2 | 1.6 |
|  | 8.2.1.4.2 | 2.6 |
|  | 8.2.1.4.3 | 1.2 |

Based on simulation results, R-ML receiver for TM3 and TM4 under medium antenna correlation has reasonable performance gain about 1.2~2.8dB in comparison with baseline MMSE receiver.

- Observation1: R-ML receiver provides reasonable performance gain under medium antenna correlation.

- Observation2: Test setup of 36.101 single cell multi-layer spatial multiplexing FRC can be reused for the performance verification of SU-MIMO advanced receiver.

Conclusion

In this contribution, we provide link level simulation results for interference cancellation in SU-MIMO, and our observations are as follows:

- Observation1: R-ML receiver provides reasonable performance gain under medium antenna correlation.

- Observation2: Test setup of 36.101 single cell multi-layer spatial multiplexing FRC can be reused for the performance verification of SU-MIMO advanced receiver.

Reference

- R4-142376, “Way Forward on SU-MIMO,” Huawei, HiSilicon, Ericsson, NVIDIA, Qualcomm, Samsung, Intel, NTT DOCOMO, MediaTek.

Appendix

Figure 5-1~Figure 5-5 show throughput performance for R-ML and MMSE receivers under Table 2-1 simulation assumption.

Figure 5-1 Throughput forTM3, EVA70, 2Tx

Figure 5-2 Throughput forTM3, EVA70, 4Tx

Figure 5-3 Throughput forTM4, EPA5, 2Tx

Figure 5-4 Throughput forTM4, ETU70, 2Tx

Figure 5-5 Throughput forTM4, EPA5, 4Tx