3GPP TSG RAN WG4 Meeting #71	        R4-143220

# Seoul, Republic of Korea, 19th  to 23th, May 2014

- Agenda Item:	7.36

- Source:	LG Electronics

- Title:	TP for high order IMD impacts for 2ULs CA_1A-5A UE

Document for:	Approval

- Introduction

In RAN4 #70 meetings, RAN4 agreed that IMD4 shall be considered for self-desensitization analysis in 2ULs inter-band CA UE case-by-case [1][2], which shall not modify the inter-band CA classes.

In this contribution, we revised our previous measurements [4] of IMD4 level for 2ULs CA_1A-5A UE as a general aspect based on the worst case scenario to define MSD levels and provide our view on this result.

- Measurement on the IMD4 levels for 2ULs CA_1A-5A

Table 1 shows the frequency range of IMD4 products in CA_1A-5A combinations.

Table 1: Band 1 and Band 5 UL IMD4 product

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1920 | 1980 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 492 - 627 |  | 4911 - 5116 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ |  |
| IMD frequency limits (MHz) | 2142 – 2312 |  |  |  |

In the previous analysis in last RAN4 meeting, we consider victim signal is full RB allocation, but we knew that the worst case scenario is that the RB allocation in both victim and aggressor band has allocated just 1RB in each band.

To analyze the IMD4 products at PA output, we assumed the typical RF components values in CA_1A-5A as follows

  - CA Channel BW : 10MHz  + 10MHz

  - RB Allocation for worst scenario : Victim band (1 RB), Aggressor band (1 RB)

  - Aggregated maximum output power : 25dBm with 2dB tolerance

  - Tx/Rx RF component IL : 3.5dB

    - 3.5dB = sum(ANT SW: 0.5dB,  Diplexer :0.5dB, Duplexer: 2.5dB)

  - FE coupling loss : 30~70dB

  - Antenna loss :10dB

  - Diplexer Characteristics: 20dB for Band 1- Band 5 isolation

  - Duplexer Characteristics : Tx-Rx isolation: 50dB, Tx-Ant isolation : 35 dB

For example, in the 2UL inter-band CA_1A-5A, the leakage from Band 5 into Band 1 can be modelled with two paths which are depicted in Figure 1. The first path (blue line) is general interference signal from band 5 transmission, which level is estimated to be about -34dBm considering general duplexer, diplexer and antenna loss. And the second path (yellow line) is generated by cross coupling effect through PCB which is quite dependent on the UE implementation.

Figure1. Example of input signal level to generate the IMD4 at PA components

This two leakages are described as Eq. 1 and Eq. 2 :

- The leakage signal level at “P” :

S1 (Blue line) = PA output power of Band 5 - Tx RF component loss of Band 5 –Rx RF component loss of Band 1 = -34dBm/CBW 						(Eq. 1)

S2 (Yellow Line) = PA output power of Band 5 – Coupling loss (30~70dB) = 25-(30~70) = -5  ~ -45 dBm/CBW 											(Eq. 2)

In Figure 1, the “P” is the point where IMD4 can be generated by two different signal paths (blue path and yellow path). But the input signal level by coupling loss in PA (yellow path) is more critical than the general interference signal by reversed RF path (blue path). That is to say, assuming coupling loss of 50 dB to evaluate the IMD4 signal level in PA, the input signal level of Band 1 PA is about -25dBm which is 9dB higher than the general interference signal level of -34dBm by reversed RF path in blue line.

Figure2. IMD measurement setup by coupling loss at PA output in Case1

So, we can measure the IMD4 level by coupling loss as shown in Figure 2 [2]. The measurement results are shown in Table 2 and 3 according two test cases.

- Case 1: Band 1 victim and Band 5 aggressor

- Case 2: Band 1 aggressor and Band 5 victim  (No directly impact to B5 DL band)

Table 2: IMD4 measurements results of 2ULs CA_1A-5A UE in Case1

| Band 1 (MHz) | Band 5 (MHz) | Coupling Loss | B5 (aggressor) Power | B1 (victim) Power | Pout | Level at 607MHz (IM4) | Level at 2162 MHz (IM4) | Level at 1318 MHz (IM5) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1925 | 844 | 40dB | -15dBm | 0 dBm | 25dBm | -26.3 | -8.9 | -22.7 |
| 1925 | 844 | 45dB | -20dBm | 0 dBm | 25dBm | -41.8 | -19.2 | -40.5 |
| 1925 | 844 | 50dB | -25dBm | 0 dBm | 25dBm | -58.5 | -29.3 | -56.3 |
| 1925 | 844 | 55dB | -30dBm | 0 dBm | 25dBm | - | -40.2 |  |
| 1925 | 844 | 60dB | -35dBm | 0 dBm | 25dBm | - | -48.9 |  |

Table 3: IMD4 measurements results of 2ULs CA_1A-5A UE in Case2

| Band 1 (MHz) | Band 5 (MHz) | Coupling loss | B5 (Victim) Power | B1 (aggressor) Power | Pout | Level at 607MHz (IM4) | Level at 2162 MHz (IM4) | Level at 1318 MHz (IM5) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1925 | 844 | 40dB | 0 dBm | -15 dBm | 25dBm | -15.7 | -57.9 | -44.2 |
| 1925 | 844 | 45dB | 0 dBm | -20 dBm | 25dBm | -20.5 | -64.1 | -54.1 |
| 1925 | 844 | 50dB | 0 dBm | -25 dBm | 25dBm | -25.2 | - | -62.0 |
| 1925 | 844 | 55dB | 0 dBm | -30 dBm | 25dBm | -30.7 | - |  |
| 1925 | 844 | 60dB | 0 dBm | -35 dBm | 25dBm | -35.2 | - |  |

From Table 2, we can see that the IMD4 level has quite an impact on the Band 1 DL frequency range (8th column) considering coupling loss of equal or less than 50 dB. From Table 3, the most measured IMD4 levels (7th column) are higher than those of Case1. But the frequency range of IMD4 in Case2 is not directly related to the own DL frequency of band 1 and band 5. And also the measured IMD4 level by |3*fx- 1*fy| is a bit higher than IMD4 level by |2*fx-2*fy|, but the difference is small.

We can analyze impact on the REFSENS with this measurement. To do this, we consider the B1 duplexer attenuation to be 50dB. Then the estimated REFSENS degradation is:

- Desense level for 2ULs UE = Conventional thermal noise level – (Thermal noise + Interference level of IMD4 products)

= -104dBm – (10^(-10.4) +10^( -7.93)) = -104dBm – (-79.285 dBm)

= -24.71dB													(Eq. 3)

From the IMD4 measurements and REFSENS analyses, we can summarize the self desensitisation levels in Table 4 according to the coupling loss for Case 1.

Table 4: Self-desensitization in Band 1 by coupling loss for 2ULs CA_1A-5A UE

| Coupling factor (dB) | Desensitization (dB) |
| --- | --- |
| 40 | 45.10 |
| 45 | 34.80 |
| 50 | 24.71 |
| 55 | 13.98 |
| 60 | 6.27 |

So we have the following observation and proposal.

Observation1: For general aspect of CA_1A-5A UE, when considering 50dB coupling loss in RF design, IMD4 impact is not negligible on REFSENS of Band 1.

But, from the discussion paper [3], IMD4 from B1+B5 would not be falling into Band 1 DL spectrum when we assume maximum 20MHz channel bandwidth per each band and fixed Tx-Rx separation. So RAN4 needs not define any MSD level for 2ULs CA_1A-5A UE.

Observation2: For the CA_1A-5A UE, RAN4 needs not consider the MSD level for Band 1 assuming maximum 20MHz channel bandwidth per each bands and fixed Tx-Rx separation.

- Conclusions

In this contribution we measured the IMD4 level on PA output to analyze the impact the self desense problems. The followings are the observations in this paper.

Observation1: For general aspect of CA_1A-5A UE, when considering 50dB coupling loss in RF design, IMD4 impact is not negligible on REFSENS of Band 1.

Observation2: For the CA_1A-5A UE, RAN4 needs not consider the MSD level for Band 1 assuming maximum 20MHz channel bandwidth per each bands and fixed Tx-Rx separation.

The corresponding text proposal for TR36.860 is provided below.

- Reference

- R4-141259, “Higher order IMD and harmonic table for UE self-desensitization analysis,” Intel

- R4-140605, “More on IMD measurements for 2UL inter-band CA,” Media Tek Inc.

- R4-141544, “Spectrum study on intermodulation analysis for 2 UL CA,” Intel

- R4-142096, “Measurements results of IMD4 level for 2ULs CA_1A-5A UE,” LG Electronics

- TR 36.860 v0.8.0

- TS 36.101 v12.3.0

Text proposal for TR 36.860

********************* Start of text proposal to TR 36.860 Chapter 6.2.1.1.2 *****************

        - Co-existence studies for CA_1A-5A with 2ULs

For the analysis of 2UL inter-band CA 1A-5A UE, we should study and analyze the inter-modulation products and harmonics products when both Band 1 and Band 5 with 2ULs are transmitting simultaneously.

The combination of Band 1 and Band 5 has harmonics frequencies that are far away from the components receive and transmit frequencies, in the UL and in the DL of the CA combination, as shown in table 6.2.1.1.2-1. So there was no harmonic issue except Band 41. In the conventional SE for UE-to-UE coexistence requirements, the 3rd harmonics of Band 5 UE already fall into B41 DL band, but exceptions for SE measurements are permitted if there is at least one individual RB within the transmission bandwidth for which the 2nd, 3rd or 4th harmonic totally or partially overlaps the measurement bandwidth (MBW) [3].

Also, the inter-modulation products for CA_1A-5A UE with 2 ULs as shown in table 6.2.1.1.2-1. None of the IMD products fall into the own CA_1A-5A UE receive bands.  But, 2-tone 3rd IMD products may have impact on the Band 22 DL, Band 42 DL and 43 DL bands. But Band 22, Band 42 and Band 43 are far away from Band1 and Band 5, so we can assume that the UE front-end filter can effectively attenuate these IMD products.

Table 6.2.1.1.2-1: 2 UL B1 + B5 harmonic products and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 824 | 849 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) | 3840 to 3960 |  |  | 1648 to 1698 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low |  | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 to 5940 |  |  | 2472 to 2547 |  |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1071 to 1156 |  |  | 2744 to 2829 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 2991 to 3136 |  |  | 222 to 332 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4664 to 4809 |  |  | 3568 to 3678 |  |  |

For the connectivity radios (WLAN, BT etc. in ISM bands) and positioning systems (GPS, Galileo, Glonass, Compass, etc), RAN4 analyzed the impact of the IMD/Harmonic from dual-uplink CA transmission in Table 6.2.1.1.2-2. From the analysis table, WLAN system has some harmonics problems in 5GHz and 2.4GHz by 3rd harmonics from each Band 1 and Band5, respectively.

Table 6.2.1.1.2-2: The IMD/Harmonic analysis to protect ISM bands and positioning systems

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | Band5 3rd harmonics |
|  | 2400 | - | 2494 | Yes | Asia | Band5 3rd harmonics |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Band1 3rd harmonics |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | Band1 3rd harmonics |

        - Analysis of IMD4 products

The IMD4 products for 2ULs CA_1A-5A UE are shown in table 6.2.1.1.3-1. The two-tone IMD4 products by |2*fx_low –2* fy_high| to |2*fx_high – 2*fy_low| fall into the own CA_1A-5A UE receive bands. And also 2-tone IMD4 products may have impact on the Band 4 DL, Band 10 DL and 23 DL bands. But Band 4, Band 10 and Band 23 are not deployed in the same region with CA_1A-5A UE.

Table 6.2.1.1.3-1: Band 1 and Band 5 UL IMD4 product

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1920 | 1980 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 492 - 627 |  | 4911 - 5116 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ |  |
| IMD frequency limits (MHz) | 2142 – 2312 |  |  |  |

To analyze the IMD4 products on PA components, we assume the typical values of RF components as follows

  - CA Channel BW : 10MHz  + 10MHz

  - RB Allocation for worst scenario: Victim band (1 RB), Aggressor band (1 RB)

  - Aggregated maximum output power : 25dBm with 2dB tolerance

  - Tx/Rx RF component IL : 3.5dB

    - 3.5dB = sum(ANT SW: 0.5dB,  Diplexer :0.5dB, Duplexer: 2.5dB)

  - FE coupling loss : 30~70dB

  - Antenna loss :10dB

  - Diplexer Characteristics: 20dB for Band 1- Band 5 isolation

  - Duplexer Characteristics: Tx-Rx isolation: 50dB, Tx-Ant isolation : 35 dB

For example, in the 2UL inter-band CA_1A-5A, the leakage from Band 5 into Band 1 can be modelled with two paths which are depicted in Figure 6.2.1.1.3-1. The first path (blue line) is general interference signal from band 5 transmission, which level is estimated to be about -34dBm considering general duplexer, diplexer and antenna loss. And the second path (yellow line) is generated by cross coupling effect through PCB which is quite dependent on the UE implementation.

Figure 6.2.1.1.3-1 Example of input signal level to generate the IMD4 at PA components

This two leakages are described as Eq. 6.2.1.1.3-1 and Eq. 6.2.1.1.3-2 :

- The leakage signal level at “P” :

S1 (Blue line) = PA output power of Band 5 - Tx RF component loss of Band 5 –Rx RF component loss of Band 1 = -34dBm/CBW 					(Eq. 6.2.1.1.3-1)

S2 (Yellow Line) = PA output power of Band 5 – Coupling loss (30~70dB) = 25-(30~70) = -5  ~ -45 dBm/CBW 										(Eq. 6.2.1.1.3-2)

In Figure 6.2.1.1.3-1, the “P” is the point where IMD4 can be generated by two different signal paths (blue path and yellow path). But the input signal level by coupling loss in PA (yellow path) is more critical than the general interference signal by reversed RF path (blue path). That is to say, assuming coupling loss of 50 dB to evaluate the IMD4 signal level in PA, the input signal level of Band 1 PA is about -25dBm which is 9dB higher than the general interference signal level of -34dBm by reversed RF path in blue line.

Figure 6.2.1.1.3-2. IMD measurement setup by coupling loss at PA output in Case1

So, we can measure the IMD4 level by coupling loss as shown in Figure 6.2.1.1.3-2 [2]. The measurement results are shown in Table 6.2.1.1.3-2 and 6.2.1.1.3-3 according two test cases.

- Case 1: Band 1 victim and Band 5 aggressor

- Case 2: Band 1 aggressor and Band 5 victim

Table 6.2.1.1.3-2: IMD4 measurements results of 2ULs CA_1A-5A UE in Case1

| Band 1 (MHz) | Band 5 (MHz) | Coupling Loss | B5 (aggressor) Power | B1 (victim) Power | Pout | Level at 607MHz (IM4) | Level at 2162 MHz (IM4) | Level at 1318 MHz (IM5) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1925 | 844 | 40dB | -15dBm | 0 dBm | 25dBm | -26.3 | -8.9 | -22.7 |
| 1925 | 844 | 45dB | -20dBm | 0 dBm | 25dBm | -41.8 | -19.2 | -40.5 |
| 1925 | 844 | 50dB | -25dBm | 0 dBm | 25dBm | -58.5 | -29.3 | -56.3 |
| 1925 | 844 | 55dB | -30dBm | 0 dBm | 25dBm | - | -40.2 |  |
| 1925 | 844 | 60dB | -35dBm | 0 dBm | 25dBm | - | -48.9 |  |

Table 6.2.1.1.3-3: IMD4 measurements results of 2ULs CA_1A-5A UE in Case2

| Band 1 (MHz) | Band 5 (MHz) | Coupling loss | B5 (Victim) Power | B1 (aggressor) Power | Pout | Level at 607MHz (IM4) | Level at 2162 MHz (IM4) | Level at 1318 MHz (IM5) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1925 | 844 | 40dB | 0 dBm | -15 dBm | 25dBm | -15.7 | -57.9 | -44.2 |
| 1925 | 844 | 45dB | 0 dBm | -20 dBm | 25dBm | -20.5 | -64.1 | -54.1 |
| 1925 | 844 | 50dB | 0 dBm | -25 dBm | 25dBm | -25.2 | - | -62.0 |
| 1925 | 844 | 55dB | 0 dBm | -30 dBm | 25dBm | -30.7 | - |  |
| 1925 | 844 | 60dB | 0 dBm | -35 dBm | 25dBm | -35.2 | - |  |

From Table 6.2.1.1.3-2, we can see that the IMD4 level has quite an impact on the Band 1 DL frequency range (8th column) considering coupling loss of equal or less than 50 dB. From Table 6.2.1.1.3-3, the most measured IMD4 levels (7th column) are higher than those of Case1. But the frequency range of IMD4 in Case2 is not directly related to the own DL frequency of band 1 and band 5. And also the measured IMD4 level by |3*fx- 1*fy| is a bit higher than IMD4 level by |2*fx-2*fy|, but the difference is small.

We can analyze impact on the REFSENS with this measurement. To do this, we consider the B1 duplexer attenuation to be 50dB. Then the estimated REFSENS degradation is:

- Desense level for 2ULs UE = Conventional thermal noise level – (Thermal noise + Interference level of IMD4 products)

= -104dBm – (10^(-10.4) +10^( -7.93)) = -104dBm – (-79.285 dBm)

= -24.71dB									(Eq. 6.2.1.1.3-3)

From the IMD4 measurements and REFSENS analyses, we can summarize the self desensitisation levels in Table 6.2.1.1.3-4 according to the coupling loss for Case 1.

Table 6.2.1.1.3-4: Self-desensitization in Band 1 by coupling loss for 2ULs CA_1A-5A UE

| Coupling factor (dB) | Desensitization (dB) |
| --- | --- |
| 40 | 45.10 |
| 45 | 34.80 |
| 50 | 24.71 |
| 55 | 13.98 |
| 60 | 6.27 |

So we have the following observation.

Observation1: For general aspect of CA_1A-5A UE, when considering 50dB coupling loss in RF design, IMD4 impact is not negligible on REFSENS of Band 1.

But, from the discussion paper [3], IMD4 from B1+B5 would not be falling into Band 1 DL spectrum when we assume maximum 20MHz channel bandwidth per each band and fixed Tx-Rx separation. So RAN4 needs not define any MSD level for 2ULs CA_1A-5A UE.

Observation2: For the CA_1A-5A UE, RAN4 needs not consider the MSD level for Band 1 assuming maximum 20MHz channel bandwidth per each bands and fixed Tx-Rx separation.

******************** End of text proposal to TR 36.860 chapter 6.2.1.1.2 **********************