3GPP TSG-RAN WG4 RAN4 #71                                         R4-143348

Seoul, Korea, 19 May - 23 May, 2014

Agenda item:	7.14.2.1

Source: 	Samsung

Title:			   On blind detection feasibility of parameter combination for CRS-based TM

Document for:	Discussion

# 1	Introduction

In 3GPP RAN4 #70Bis meeting, RAN4 identified and agreed on the following dynamic parameter combinations that could be blindly detected for CRS based TM under the assumption that remaining semi-static parameters, P_A and TM are known, including:

- Presence or absence of interference

- RI/PMI/Modulation

In this paper, we provide our further views and analysis on the feasibility of TM blind detection and the impact of resource allocation granularity when desired signal is CRS based TM mode.

For TM blind detection, when desired signal is CRS based TM mode, e.g. TM4, TM2, UE at least need to detect

- Whether interference is DMRS-based TM mode.

  - If yes, then at least UE can fallback to linear receiver as baseline behavior and guarantee no performance loss compared with Rel-11 MMSE-IRC receiver.

- Whether the interference is TM2, TM3 or TM4, if interference is not DMRS-based TM mode.

  - Note, TM1 and TM5 is not considered as a typical network operation here. Thus, TM1 and TM5 are not included in TM detection set in this paper. For TM6, it is considered as a subset of TM4.

In this contribution, we provide our analysis on these two aspects above.

# Blind Detection of DMRS-based TM mode

In RAN4 #70bis meeting, it has been confirmed the blind detection of DMRS-based TM mode is feasible when both serving cell and interference cell is DMRS mode. However, there is no study on the detection performance when desired signal is CRS based TM. In this case, the DMRS RE of interference cell is colliding with the data RE of desired signal. Thus, the blind detection performance may be worse than the case that desired signal is DMRS-based TM mode. In Annex B Figure 1 to Figure 4, the blind detection performance of DMRS-based TM mode is shown. Detailed assumptions are provided in Annex A.

- For R-ML without TM BD receiver, it is assumed that UE always fallback as MMSE-IRC receiver.

- For R-ML with TM BD, if DMRS mode interference is detected, UE fallback as MMSE-IRC receiver. Otherwise, UE will turn on R-ML receiver, and continue to detect the exact TM (TM2/3/4) and the remaining required information.

Table 1 summarized the required SNR @70% throughput for the simulated cases. It can be observed that when interference to signal ratio (ISR) decreases (i.e. the SNR increases), the blind detection performance of DMRS based TM worsen. Especially, for the cases with 16QAM for serving cell, the performance loss is more than 1.0dB in most cases and up to 2.1dB in worst case.

Observation 1: when serving cell is configured with TM4 (especially with 16QAM), the performance loss caused by DMRS-based TM blind detection is more than 1.0dB and up to 2.1dB in worst case.

Table 1: Required SNR @70% throughput

| INF Level 1 + Rank 1 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} |
| --- | --- | --- | --- | --- |
| R-ML without TM BD | 5.0 | 5.0 | 12.1 | 12.1 |
| R-ML with TM BD (1RB) | 5.1 | 5.2 | 12.9 | 13.1 |
| INF Level 1 + Rank 2 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} |
| R-ML without TM BD | 4.2 | 4.2 | 11.3 | 11.3 |
| R-ML with TM BD (1RB) | 4.4 | 4.4 | 12.2 | 12.4 |
| INF Level 2 + Rank 1 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} |
| R-ML without TM BD | 8.7 | 8.7 | 15.9 | 15.8 |
| R-ML with TM BD (1RB) | 8.8 | 8.9 | 16.9 | 17.3 |
| INF Level 2 + Rank 2 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} |
| R-ML without TM BD | 7.6 | 7.6 | 14.8 | 14.8 |
| R-ML with TM BD (1RB) | 7.8 | 7.8 | 16.5 | 16.9 |

# Blind Detection of TM2, TM3 and TM4

In this section, we discuss the issue on TM2, TM3 and TM4 blind detection. Since TM2, TM3 and TM4 is corresponding to different spatial multiplexing scheme, UE need to distinguish whether interference is TM2, TM3 or TM4 for further interference signal processing. The error detection may significantly impact the overall NAICS receiver performance since it may leads to the wrong effective channel estimation.

In Annex C Figure 5 and Figure 6, the blind detection performance of TM2 mode is presented. In Annex C Figure 7 and Figure 8, the blind detection performance of TM3 mode is presented. Detail simulation assumption is shown in Annex A.

- Desired signal is TM4 Rank 1 transmission

- In Figure 5 and Figure 6, interference signal is TM2 transmission.

- In Figure 7 and Figure 8, interference signal is TM3 transmission.

Furthermore, Table 2 and Table 3 summarized the required SNR @70% throughput for TM2 and TM3 blind detection performance correspondingly. Based on the results, it is observed that

- Observation 2: For the blind detection of TM2, the additional performance loss is up to 0.6dB, on top of modulation format and interference presence blind detection.

- Observation 3: For the blind detection of TM3, the additional performance loss is up to 2.1dB, on top of modulation format and interference presence blind detection.

Table 2: Required SNR @70% throughput for TM2 blind detection performance

| INF Level 1 + TM 2 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} | AVG Gain | Max Gain |
| --- | --- | --- | --- | --- | --- | --- |
| MMSE | 5.5 | 5.5 | 12.3 | 12.3 |  |  |
| R-ML without BD | 4.4 | 5.2 | 11.8 | 12.3 | 0.5 | 1.1 |
| R-ML with INF/MF BD | 4.4 | 5.5 | 11.8 | 12.4 | 0.4 | 1.1 |
| R-ML with TM/INF/MF BD | 4.8 | 5.8 | 12.4 | 13.0 | -0.1 | 0.8 |
| INF Level 2 + TM 2 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} | AVG Gain | Max Gain |
| MMSE | 9.3 | 9.3 | 16.0 | 16.0 |  |  |
| R-ML without BD | 4.0 | 8.0 | 13.1 | 15.6 | 2.5 | 5.3 |
| R-ML with INF/MF BD | 4.0 | 8.6 | 13.2 | 16.1 | 2.2 | 5.3 |
| R-ML with TM/INF/MF BD | 4.1 | 8.7 | 13.4 | 16.4 | 2.0 | 5.2 |

Table 3: Required SNR @70% throughput for TM3 blind detection performance

| INF Level 1 + TM 3 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} | AVG Gain | Max Gain |
| --- | --- | --- | --- | --- | --- | --- |
| MMSE | 5.5 | 5.5 | 12.3 | 12.3 |  |  |
| R-ML without BD | 4.5 | 5.3 | 11.8 | 12.3 | 0.4 | 1.0 |
| R-ML with INF/MF BD | 4.5 | 5.6 | 11.8 | 12.5 | 0.3 | 1.0 |
| R-ML with TM/INF/MF BD | 5.6 | 6.3 | 12.9 | 13.2 | -0.6 | -0.1 |
| INF Level 2 + TM 3 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} | AVG Gain | Max Gain |
| MMSE | 9.3 | 9.3 | 15.9 | 15.9 |  |  |
| R-ML without BD | 3.9 | 8.2 | 13.3 | 15.6 | 2.4 | 5.4 |
| R-ML with INF/MF BD | 4.0 | 9.1 | 13.3 | 16.2 | 2.0 | 5.3 |
| R-ML with TM/INF/MF BD | 6.1 | 9.9 | 15.2 | 16.9 | 0.6 | 3.2 |

# Impact of Resource Allocation Granularity

In RAN4 70Bis meeting, it was agreed that interference parameters are assumed to have granularity of at least 1 PRB pair in time. Further bundling in frequency domain is FFS. In this section, we discuss and evaluate the impact of resource allocation granularity.

In Annex D Figure 9 and Figure 10, the blind detection performance of 1 PRB pair and 3 PRB pair granularity is presented, when both desired signal and interfering signal is TM4 Rank 1transmission. In simulation, UE blindly detected interference presence, RI, PMI and modulation. And Table 4 summarized the required SNR @70% throughput correspondingly.

Table 3: Required SNR @70% throughput (1 PRB pair vs 3 PRB pair)

| INF Level 1 + Rank 1 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} |
| --- | --- | --- | --- | --- |
| MMSE | 5.5 | 5.6 | 12.3 | 12.3 |
| R-ML without BD | 2.1 | 3.7 | 9.9 | 11.0 |
| R-ML with BD (1 PRB Pair) | 2.8 | 5.0 | 10.8 | 12.2 |
| R-ML with BD (3 PRB Pair) | 2.3 | 4.5 | 10.2 | 11.8 |
| INF Level 2 + Rank 1 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} |
| MMSE | 9.4 | 9.4 | 16.1 | 16.1 |
| R-ML without BD | 1.9 | 4.1 | 9.3 | 12.3 |
| R-ML with BD (1 PRB Pair) | 2.3 | 5.1 | 10.0 | 13.8 |
| R-ML with BD (3 PRB Pair) | 2.0 | 4.4 | 9.4 | 12.9 |

Based on the results, it is observed that

- Observation 4: On the impact of resource allocation granularity, 3 PRB pair granularity shows noticeable performance gain (at around 0.4dB~0.9dB) in most cases over 1 PRB pair granularity.

# Conclusion

In this contribution, firstly we investigate the performance of TM blind detection when desired signal is CRS based TM mode, e.g. TM4 Rank 1 transmission. According to the evaluation results, the observations are:

Observation 1: For the blind detection of TM9, the performance loss caused by DMRS-based TM blind detection is more than 1.0dB and up to 2.1dB in worst case when serving cell is 16QAM transmission

Observation 2: For the blind detection of TM2, the additional performance loss is up to 0.6dB, on top of modulation format and interference presence blind detection.

Observation 3: For the blind detection of TM3, the additional performance loss is up to 2.1dB, on top of modulation format and interference presence blind detection.

Based on the observations above, information on which TMs should be considered for UE blind detection would be useful in UE side. Therefore, our proposal is

Proposal 1: In case of desired signal is CRS based TM, higher layer signalling on which TMs to be detected should be provided for NAICS receiver to reduce the performance loss of blind detection and UE implementation complexity.

Furthermore, the impact of resource allocation granularity on blind detection performance is also evaluated. It is observed that

Observation 4: On the impact of resource allocation granularity, 3 PRB pair granularity shows noticeable performance gain (at around 0.4dB~0.9dB) in most cases over 1 PRB pair granularity.

In addition, a larger resource allocation granularity can significantly reduce UE processing complexity and power consumption. Although the noticeable benefit is observed for further frequency bundling, there is a concern that the it may limit the network scheduling flexibility and system performance may degrade if a larger RA granularity (e.g. 1 RBG) is mandatory for NAICS receiver. However, at least whenever a larger resource allocation granularity can be guaranteed by network without limiting scheduling flexibility, it is always beneficial to indicate the information to UE. So, our proposal on resource allocation granularity is

Proposal 2: To reduce UE implementation complexity and improve blind detection robustness, it is beneficial to indicate the larger resource allocation granularity (e.g. 1 RBG) whenever it can be guaranteed by network without impacting scheduling flexibility (e.g. if interfering cell is configured with 1 RBG scheduling strategy).

# Reference

- N/A

# Annex A: Simulation Assumption

Table 7-1 Simulated Interference Combinations

| (Serving/I1/I2) | Interference cell RI | Interference cell MCS | Serving cell RI | Serving cell MCS |
| --- | --- | --- | --- | --- |
| ON/ON/ON pattern | 1 | 5 | 1 | {5} |
|  |  |  |  | {14} |
|  | 1 | 14 | 1 | {5} |
|  |  |  |  | {14} |
|  | 2 | 5 | 1 | {5} |
|  |  |  |  | {14} |
|  | 2 | 14 | 1 | {5} |
|  |  |  |  | {14} |

Table 7-2: Simulation Assumptions

| Parameter | Value |
| --- | --- |
| System bandwidth | 10 MHz |
| RB allocation | 6 |
| Cell ID | [0, 6, 1] |
| Transmission mode on Serving cell | TM4 |
| Transmission mode on Interference cell | TM9, TM2, TM3, TM4 |
| Interference profiles | INF Level 1: INR1 = 7.77dB, INR2 = 2.29dB; INF Level 2: INR1 = 13.91dB, INR2 = 3.34dB |
| MIMO configuration | 2x2 and low correlation |
| Channel model and Doppler frequency for target and interference cells | EPA 5Hz Use different channel seed for between cells |
| CRS configuration | 2 CRS ports. CRS is colliding between serving cell and interference cells |
| PA | -3dB |
| PB | 0dB |
| CSI-RS configuration | None |
| Channel Estimation | Realistic Channel Estimation |
| PMI | Random PMI |
| H-ARQ | 8 HARQ processes |
| PCFICH | CFI = 2 |
| PCFICH/PDCCH detection | Not considered |

# Annex B: Blind Detection Performance of DMRS based TM

Figure 1: Performance under Rank 1 interference and INR1 = 7.77dB, INR2 = 2.29dB

Figure 2: Performance under Rank 2 interference and INR1 = 7.77dB, INR2 = 2.29dB

Figure 3: Performance under Rank 1 interference and INR1 = 13.91dB, INR2 = 3.34dB

Figure 4: Performance under Rank 2 interference and INR1 = 13.91dB, INR2 = 3.34dB

# Annex C: Blind Detection Performance of TM2 and TM3

Figure 5: Performance under TM2 interference and INR1 = 7.77dB, INR2 = 2.29dB

Figure 6: Performance under TM2 interference and INR1 = 13.91dB, INR2 = 3.34dB

Figure 7: Performance under TM3 interference and INR1 = 7.77dB, INR2 = 2.29dB

Figure 8: Performance under TM3 interference and INR1 = 13.91dB, INR2 = 3.34dB

# Annex D: Impact of Resource Allocation Granularity

Figure 9: Performance under TM4 Rank 1 interference and INR1 = 7.77dB, INR2 = 2.29dB

Figure 10: Performance under TM4 Rank 1 interference and INR1 = 13.91dB, INR2 = 3.34dB