3GPP TSG-RAN WG4 RAN4 #71                                         R4-143356

Seoul, Korea, 19 May - 23 May, 2014

Agenda item:	7.14.2.2

Source: 	Samsung

Title:			  On blind detection feasibility of parameter combination for DMRS-based TM

Document for:	Discussion

# 1	Introduction

In 3GPP RAN4 #70Bis meeting, RAN4 identified and agreed on the following dynamic parameter combinations that could be blindly detected for DMRS based TM under the assumption that remaining semi-static parameters, P_A and TM are known, including:

- Presence or absence of interference

- RI and Modulation

- DMRS ports, nSCID

In this paper, we provide our further analysis on the feasibility of TM blind detection and the blind detection performance under 4Tx case when desired signal is DMRS based TM mode.

In this contribution, we provide our analysis on these two aspects above.

# Blind Detection of DMRS-based TM mode

For TM blind detection, when desired signal is DMRS based TM mode, e.g. TM9, UE at least need to detect

- Whether interference is DMRS-based TM mode.

  - If no, then at least UE can fallback to linear receiver as baseline behavior and guarantee no performance loss compared with Rel-11 MMSE-IRC receiver.

  - If yes, then UE can continue the detection of other parameters, e.g. DMRS related information and so on.

In this section, we evaluated the TM blind detection performance for two aspects:

  - When interference is DMRS-based TM mode, whether UE can correctly detect it and enable NAICS receiver

  - When interference is CRS-based TM mode, whether UE can correctly detect it and fallback to MMSE-IRC  receiver.

Detailed assumptions are provided in Annex A.

For interference is configured with TM9 case, the blind detection performance is shown in Figure 1 to Figure 4 in Annex B. And Table 1 summarized the required SNR @70% throughput for the simulated cases. It can be observed that no additional performance loss is observed for the TM blind detection.

For interference is configured with TM4 case, the blind detection performance is shown in Table 2. For R-ML without TM BD receiver, it is assumed that UE always fallback as MMSE-IRC receiver. It can be observed that the additional performance loss is less than 0.05dB for all cases.

Therefore, it can be concluded,

Observation 1: when serving cell is configured with TM9, the performance loss caused by DMRS-based TM blind detection is neglectable.

Table 1: Required SNR @70% throughput when interference cell is TM9

| INF Level 1 + Rank 1 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} |
| --- | --- | --- | --- | --- |
| MMSE-IRC | 5.7 | 5.7 | 12.6 | 12.6 |
| R-ML without BD | 3.7 | 5.0 | 11.4 | 12.3 |
| R-ML with MF+INF ON/OFF BD | 3.8 | 5.5 | 11.5 | 12.7 |
| R-ML with TM+INF ON/OFF+MF BD | 3.8 | 5.5 | 11.5 | 12.7 |
| INF Level 1 + Rank 2 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} |
| MMSE-IRC | 5.8 | 5.8 | 12.8 | 12.8 |
| R-ML without BD | 5.2 | 5.8 | 12.5 | 13.0 |
| R-ML with MF+INF ON/OFF BD | 5.3 | 6.0 | 12.5 | 13.1 |
| R-ML with TM+INF ON/OFF+MF BD | 5.3 | 6.0 | 12.5 | 13.1 |
| INF Level 2 + Rank 1 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} |
| MMSE-IRC | 7.5 | 7.6 | 14.4 | 14.5 |
| R-ML without BD | 3.9 | 5.8 | 10.9 | 13.4 |
| R-ML with MF+INF ON/OFF BD | 4.0 | 6.0 | 11.2 | 13.8 |
| R-ML with TM+INF ON/OFF+MF BD | 4.0 | 6.0 | 11.2 | 13.8 |
| INF Level 2 + Rank 2 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} |
| MMSE-IRC | 9.5 | 9.5 | 16.4 | 16.4 |
| R-ML without BD | 4.9 | 8.3 | 13.5 | 16.0 |
| R-ML with MF+INF ON/OFF BD | 5.6 | 8.9 | 13.9 | 16.4 |
| R-ML with TM+INF ON/OFF+MF BD | 5.6 | 8.9 | 13.9 | 16.4 |

Table 2: Required SNR @70% throughput when interference cell is TM9

| INF Level 1 + Rank 1 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} |
| --- | --- | --- | --- | --- |
| R-ML without BD | 5.57 | 5.64 | 12.44 | 12.57 |
| R-ML with TM+INF ON/OFF+MF BD | 5.60 | 5.63 | 12.44 | 12.57 |
| INF Level 1 + Rank 2 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} |
| R-ML without BD | 6.92 | 6.95 | 13.78 | 13.78 |
| R-ML with TM+INF ON/OFF+MF BD | 6.95 | 6.96 | 13.79 | 13.79 |
| INF Level 2 + Rank 1 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} |
| R-ML without BD | 7.49 | 7.56 | 14.32 | 14.39 |
| R-ML with TM+INF ON/OFF+MF BD | 7.51 | 7.58 | 14.31 | 14.41 |
| INF Level 2 + Rank 2 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} |
| R-ML without BD | 10.58 | 10.61 | 17.47 | 17.52 |
| R-ML with TM+INF ON/OFF+MF BD | 10.61 | 10.61 | 17.47 | 17.53 |

# Blind Detection Performance under 4Tx case

In this section, we provide the simulation results to verify the blind detection performance under 4Tx when desired signal is DMRS based TM mode.

In Annex C Figure 5 to Figure 8, the blind detection performance of TM9 mode is presented. In the simulation, UE blindly detect TM, interference presence, RI, Modulation, DMRS port and also nSCID. Detail simulation assumption is shown in Annex A.

Furthermore, Table 3 summarized the required SNR @70% throughput of blind detection performance correspondingly. Based on the results, it is observed that

Observation 2: For the blind detection of TM9 interference under 4Tx case, it is still feasible to blindly detect the same set of information as 2Tx case.

Table 3: Required SNR @70% throughput for TM9 blind detection performance under 4Tx case

| INF Level 1 + Rank 1 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} |
| --- | --- | --- | --- | --- |
| MMSE | 6.5 | 6.5 | 13.5 | 13.5 |
| R-ML without BD | 4.0 | 5.6 | 12.0 | 13.2 |
| R-ML with TM+INF ON/OFF+MF BD (1RB) | 4.2 | 6.0 | 12.1 | 13.5 |
| INF Level 1 + Rank 2 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} |
| MMSE | 7.8 | 7.8 | 14.9 | 14.9 |
| R-ML without BD | 7.0 | 7.6 | 14.7 | 15.1 |
| R-ML with TM+INF ON/OFF+MF BD (1RB) | 7.2 | 8.0 | 14.8 | 15.4 |
| INF Level 2 + Rank 1 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} |
| MMSE | 8.2 | 8.2 | 15.3 | 15.3 |
| R-ML without BD | 4.4 | 6.2 | 11.5 | 14.2 |
| R-ML with TM+INF ON/OFF+MF BD (1RB) | 4.5 | 6.5 | 11.9 | 14.5 |
| INF Level 2 + Rank 2 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} |
| MMSE | 11.6 | 11.5 | 18.5 | 18.5 |
| R-ML without BD | 6.7 | 10.6 | 16.0 | 18.3 |
| R-ML with TM+INF ON/OFF+MF BD (1RB) | 7.7 | 11.2 | 16.6 | 18.7 |

# Conclusion

In this contribution, we investigate the feasibility of TM blind detection and the blind detection performance under 4Tx case when desired signal is DMRS based TM mode. Our observations are:

Observation 1: When serving cell is configured with TM9, the performance loss caused by DMRS-based TM blind detection is neglectable.

Observation 2: When serving cell is configured with TM9, for the blind detection of TM9 interference under 4Tx case, it is still feasible to blindly detect the same set of information as 2Tx case.

# Reference

- N/A

# Annex A: Simulation Assumption

Table 7-1 Simulated Interference Combinations

| (Serving/I1/I2) | Interference cell RI | Interference cell MCS | Serving cell RI | Serving cell MCS |
| --- | --- | --- | --- | --- |
| ON/ON/ON pattern | 1 | 5 | 1 | {5} |
|  |  |  |  | {14} |
|  | 1 | 14 | 1 | {5} |
|  |  |  |  | {14} |
|  | 2 | 5 | 1 | {5} |
|  |  |  |  | {14} |
|  | 2 | 14 | 1 | {5} |
|  |  |  |  | {14} |

Table 7-2: Simulation Assumptions

| Parameter | Value |
| --- | --- |
| System bandwidth | 10 MHz |
| RB allocation | 6 |
| Cell ID | [0, 6, 1] |
| Transmission mode on Serving cell | TM9 |
| Transmission mode on Interference cell | TM9, TM4 |
| Interference profiles | INF Level 1: INR1 = 7.77dB, INR2 = 2.29dB; INF Level 2: INR1 = 13.91dB, INR2 = 3.34dB |
| MIMO configuration | 2x2 and low correlation |
| Channel model and Doppler frequency for target and interference cells | EPA 5Hz Use different channel seed for between cells |
| CRS configuration | 2 CRS ports. CRS is colliding between serving cell and interference cells |
| PA | -3dB |
| PB | 0dB |
| CSI-RS configuration | None |
| Channel Estimation | Realistic Channel Estimation |
| PMI | Random PMI |
| H-ARQ | 8 HARQ processes |
| PCFICH | CFI = 2 |
| PCFICH/PDCCH detection | Not considered |

# Annex B: Blind Detection Performance of DMRS based TM

Figure 1: Performance under TM9 Rank 1 interference and INR1 = 7.77dB, INR2 = 2.29dB

Figure 2: Performance under TM9 Rank 2 interference and INR1 = 7.77dB, INR2 = 2.29dB

Figure 3: Performance under TM9 Rank 1 interference and INR1 = 13.91dB, INR2 = 3.34dB

Figure 4: Performance under TM9 Rank 2 interference and INR1 = 13.91dB, INR2 = 3.34dB

# Annex C: Blind Detection Performance under 4Tx case

Figure 1: Performance under 4Tx TM9 Rank 1 interference and INR1 = 7.77dB, INR2 = 2.29dB

Figure 2: Performance under 4Tx TM9 Rank 2 interference and INR1 = 7.77dB, INR2 = 2.29dB

Figure 3: Performance under 4Tx TM9 Rank 1 interference and INR1 = 13.91dB, INR2 = 3.34dB

Figure 4: Performance under 4Tx TM9 Rank 2 interference and INR1 = 13.91dB, INR2 = 3.34dB