3GPP TSG-RAN WG4 (Radio) Meeting #71	R4-143501

Seoul, Korea, 19 - 23 April 2014

Agenda Item:	8.1.3

Source: 	Alcatel-Lucent

Title: 	Recommendations on how to include the European regulatory requirements for emissions from the L-band base station in the 3GPP specifications

Document for:	Discussion

# 1.	Introduction

The way to include the European regulatory requirements for emissions from the L-band base station in the 3GPP specifications was further discussed in RAN4#70Bis [1-2]. It was generally agreed to specify the requirements as a “declared level”, using the same approach as the DTT protection requirements for Band 20 as proposed in [3].

However, there were still two open issues to be discussed:

- The inclusion of an annex with a guideline to calculate the EIRP limits as defined in the ECC decision.

- The frequency ranges for the declared emissions levels below 1452 MHz and above 1492 MHz.

In this paper, we provide our views on these open issues to include the ECC decision in the BS specifications.

# 2.	Discussion

Currently, there is an informative annex in the BS specifications [4, 5] on how compliance with the European regulatory requirements for protection of DTT can be assessed for BS operating in Band 20 based on the manufacturer’s declaration together with the deployment characteristics.  This provides a clear guideline for operators to verify the ECC EIRP limits are complied with based on the declared emission levels from the BS manufacturers. No issue has been identified on this approach since the specification of Band 20 in 2009. Therefore, we recommend using the same approach for L-band unless a clearer approach can be identified.

On the frequency ranges for declared emission levels below 1452 MHz and above 1492 MHz, it is recommended in CEPT/ERC/REC 74-01 [6]:

<Start of extraction>

3) that the spurious domain emission limits for radio equipment are considered here to be applicable for the range 9 kHz to 300 GHz. However, for practical measurement purposes only, the frequency range of spurious emissions measurements may be restricted still ensuring that the limits are met. As guidance for practical purposes, the following measurement parameters are normally recommended:

| Table 1 Fundamental frequency range | Frequency range for measurements |  |
| --- | --- | --- |
|  | Lower frequency | Upper frequency (The test should include the entire harmonic band and not be truncated at the precise upper frequency limit stated) |
| 9 kHz - 100 MHz | 9 kHz | 1 GHz |
| 100 MHz - 300 MHz | 9 kHz | 10th harmonic |
| 300 MHz - 600 MHz | 30 MHz | 3 GHz |
| 600 MHz - 5.2 GHz | 30 MHz | 5th harmonic |
| 5.2 GHz - 13 GHz | 30 MHz | 26 GHz |
| 13 GHz - 150 GHz | 30 MHz | 2nd harmonic |
| 150 GHz - 300 GHz | 30 MHz | 300 GHz |

<End of extraction>

It can be seen that while the limits (requirements) are considered to be applicable for the range 9 kHz to 300 GHz, the measurements (testing) may be restricted to a narrower frequency range for practical purposes. We recommend considering a similar approach on the declared emissions levels below 1452 MHz and above 1492 MHz, i.e. the requirements are specified in alignment with the ECC decision, while the testing may be restricted to narrower frequency ranges for practical measurement purposes.

# 3.	Conclusions

In this paper, we have provided our views on the two open issues to include the ECC decision in the 3GPP specifications, and recommended that:

- The inclusion of an annex with a guideline to calculate the EIRP limits as defined in the ECC decision.

- The frequency ranges to which the declared emissions levels below 1452 MHz and above 1492 MHz are specified in alignment with the ECC decision, while the testing may be restricted to narrower frequency ranges for practical measurement purposes.

# References

[1]	R4-142059, “TP for TR 37.814: The EIRP limits in Region 1”, Ericsson.

[2]	R4-142285, “TP for TR 37.814: Regulatory EIRP emission limits in Region 1”, Orange.

[3]	R4-136068, “Proposal on how to include the regulatory requirements for emissions from the L-band basestation in the 3GPP specifications”, Alcatel-Lucent.

[4]		3GPP TS 36.104 v12.3.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”.

[5]	3GPP TS 36.141 v12.3.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing”.

[6]	CEPT/ERC/REC 74-01, “CEPT/ERC/Recommendation 74-01E (Siófok 98, Nice 99, Sesimbra 02, Hradec Kralove 05, Cardiff 11): Unwanted Emissions in the Spurious Domain”.