3GPP TSG RAN WG4 Meeting #71	        R4-143517

# Seoul, Republic of Korea, 19th  to 23th, May 2014

- Agenda Item:	5.1

- Source:	LG Electronics

- Title:	Considerations on modification of contents in TS36.307

Document for:	Approval

- Introduction

LTE CA is introduced in Rel-10 specifications for LTE advanced. And many CA band combinations and various CA features such as TDD-FDD joint operation, intra-NC CA, 2ULs inter-band CA and 3DLs inter-band CA are captured in TS36.307. So these related spectrums are not categorized in the TS36.307 since the related WIs are closed, then the spectrums are listed in the specification. For this reason, it is not simple to include additional CA WIs and/or operating band WIs into TS 36.307.

In this contribution, a revised content of TS36.307 is proposed.

- Current contents of TS36.307

Current contents of TS36.307 are not categorized as various CA features and/or operating bands. Therefore, it is difficult to add a new CA feature and/or operating band WIs.

In the following, we categorized the current and potential future contents of TS36.307.

- Non-CA based operating bands : e.g. Band 26, 27, 28, 30, 31, 44

- Intra-band Contiguous CA bands :

  - CA Class B : CA_23B, CA_27B,

  - CA Class C : CA_3C, CA_7C, CA_39C, CA_41C

  - CA Class D : CA_40D, CA_41D

- Intra-band Non Contiguous CA bands: e.g. CA 2A-2A, CA 3A-3A, 4A-4A , 7A-7A, 23A-23A

- Inter-band CA with 1UL CA bands:

  - Inter-band CA 2DLs/1UL: e.g.CA_1A-8A, CA_2A-13A, CA_3A-19A

  - Inter-band CA 3DLs/1UL: e.g. CA_1A-5A-7A, CA_2A-5A-13A, CA_1A-3A-8A

- Inter-band CA with 2ULs CA bands:

  - Inter-band CA 2DLs/2ULs: e.g. Band 1+5, Band 5+7, Band 3+8, Band 1+19

  - Inter-band CA 3DLs/2ULs : Can be specified in future

- TDD-FDD joint operation  bands: e.g. Band 3+40, Band 8+40, Band 1+41 and Band1+42

- Other release Independent bands:

  - Band 14 HPUE

  - CA of LTE and unlicensed spectrum : Can be specified in future

Based on the observation, we propose to revise the contents of TS36.307 as follows.

Foreword

1. Scope

2. Reference

3. Definitions and Abbreviations

3.1 Definitions

3.2 Abbreviations

4. General

5. Non-CA based operating bands Independent of Release

5.1 Band 26

5.2 Band 27

…

6. Intra- band Contiguous CA bands Independent of Release

6.1 CA Class B

6.1.1 CA_23B

…

6.2 CA Class C

6.3 CA Class D

6.4 CA Class E

7. Intra- band Non Contiguous CA bands Independent of Release

7.1 CA_3A-3A

…

8. Inter- band CA with 1UL CA bands Independent of Release

8.1 Inter-band CA 2DLs:

8.1.1 CA_1A-8A

…

8.2 Inter-band CA 3DLs:

8.2.1 CA_1A-5A-7A

…

9. Inter- band CA with 2ULs CA bands Independent of Release

9.1 Inter-band CA 2DLs:

9.1.1 CA_1A-5A

…

9.2 Inter-band CA 3DLs:

9.2.1 CA_X-Y-Z

…

10. TDD- FDD joint operation bands Independent of Release

10.1 CA_1A-41A

10.2 CA_1A-42A

10.3 CA_3A-40A

10.4 CA_8A-40A

11. Other bands Independent of Release

11.1 Band 14 HPUE

11.2 CA for LTE-U

11.2.1 CA_X-ISM5.8G

…

Annex A (Informative): Frequency arrangement for overlapping operating bands

Annex B (Normative): Common Requirements

Annex C (Informative): Change History

This structure is easier to capture the additional CA feature and/or release independent spectrum issues. So we propose as follow

Proposal: To easily capture the new CA feature and release independent spectrum issues, we propose to use the above structure as new contents of TS36.307.

- Reference

- TS36.307 v11.7.0

- TS36.307 v 10.10.0

- TS36.307 v 12.3.0

- TR 36.860 v0.8.0

- TS 36.101 v12.3.0