3GPP TSG RAN WG4 Meeting #71	        R4-143608

# Seoul, Republic of Korea, 19th  to 23th, May 2014

- Agenda Item:	9.1

- Source:	LG Electronics

- Title:	Duplexer characteristics for S-band UE

Document for:	Information

- Introduction

In the previous RAN4 meeting, duplexer characteristics should be study to define the channel arrangements of new MSS band. And some company provided single duplexer characteristics according to the channel arrangements, but the required insertion loss is quite different compare to common understanding in current market. So we provided the measured ILs from RF component vendors of single duplexer according to the channel arrangement.

- Single Duplexer characteristics

Some RF vendors provided single duplexer characteristics of MSS band. We show the single duplexer characteristics in Table 1 and 2.

Table 1: Tx ILs of Band 1, 30MHz and 90MHz Duplexer for MSS band

| TX I.L. | 1920 MHz – 1980 MHz |  | 1980 MHz – 2010 MHz |
| --- | --- | --- | --- |
| Band 1 | 1.7 dB |  | – |
| MSS 30MHz x 2 | – | – | 2.5 dB |
| MSS 90MHz x 2 | 3.5 dB |  |  |

Table 2: Rx ILs of Band 1, 30MHz and 90MHz Duplexer for MSS band

| RX I.L. | 1920 MHz – 1980 MHz |  | 1980 MHz – 2010 MHz |
| --- | --- | --- | --- |
| Band 1 | 2.3 dB |  | – |
| MSS 30MHz x 2 | – | – | 3.0 dB |
| MSS 90MHz x 2 | 4.5 dB |  |  |

From the duplexer characteristics, RAN4 can decide that single duplexer for MSS 90MHz is not possible solution due to 1.8dB additional ILs compare to band 1 duplexer.

Therefore, we propose as follow

Proposal: RAN4 should not consider using single duplexer for MSS 90MHz channel arrangements in current state of art technology.

- Reference

- R4-142609, “Wayforward for Study item LTE FDD in the bands 1980-2010 MHz and 2170-2200 MHz,” TTA

- TR 36.861 v0.4.0

- R4-141289 , “Band option for MSS spectra,” KDDI