3GPP TSG-WG4 Meeting #71	R4-143670

# Seoul, Republic of Korea, 19th May – 23rd May 2014

Agenda item: 	7.9.2

Source: 	Qualcomm Incorporated

Title: 	Measurement requirements to monitor additional carriers in CELL_DCH state

Document for:	Discussion and Approval

# 1		Introduction

It has been agreed that the performance requirements for increased carrier monitoring will be divided into two performance groups, normal performance group and low performance group. Different performance requirements will be defined by RAN4 for the normal performance group carriers and the low performance group carriers.

An LS[1] was sent to RAN2 to inform RAN4 agreements on increased UE carrier monitoring and also to request RAN2 to start studying signalling to indicate which carriers are in normal performance group and which carriers are in the low performance group.

In this contribution, we evaluate options to determine number of carriers in the normal performance group and the scale factor to scale the requirements for the low performance group for CELL_DCH states as discussed in the way forward in [5]. We also present the corresponding modifications to inter-RAT and inter-frequency requirements.

# 2		Additional Options Considered for Evaluation

There were four option A/B/C/D that were proposed in the way forward document in RAN4#70bis.

Option A proposes configurable number of carriers ‘k’ in the normal performance group. In other words, UTRAN shall signal the parameter ‘k’to the UE. Additionally, the cell reselection performance in normal performance group and low performance group will be determined by a fixed ratio/scale factor. In other words, the scale factoris fixed to a predetermined value.

Option B proposes configurable performance ratio between normal and low performance groups. In other words, UTRAN shall signal the parameter  which determines the cell reselection performance in normal performance group and low performance group. The maximum number of carriers ‘k’ in the normal performance group is set to the legacy maximum number of carriers. In other words, the number of carriers in normal performance group ‘k’ is fixed.

Option C proposes that the max number of carriers ‘k’ in the normal performance group is set to the legacy max carriers i.e. fix k. Additionally, the cell reselection performance in normal performance group and low performance group will be determined by a fixed ratio/scale factor. In other words, the scale factoris fixed to a predetermined value.

Option D proposes configurable number of carriers ‘k’ in the normal performance group. In other words, UTRAN shall signal the parameter ‘k’to the UE. It also proposes configurable performance ratio between normal and low performance groups. In other words, UTRAN shall signal the parameter  which determines the cell reselection performance in normal performance group and low performance group.

# 3   Choice of Option A/B/C/D

The issue with Option A is that the reselection delay in normal performance group for a Rel-12 UE increases as ‘k’ increases and thus a Rel-12 UE performs significantly worse than legacy UE on the normal performance layers. Moreover, if the signaled ‘k’ is larger than the requirement in the spec today, in order to ensure that the normal performance group does not have worse reselection performance than low performance group, must be fixed to a large value thus making the reselection delay on the low performance group very large. Thus, the optimal value of  may be different for different values of ‘k’. Option A offers only partial flexibility to the network but may lead to inefficient value of  for some values of ‘k’.

In Option B, since ‘k’ is being fixed to a value specified in the spec today, the reselection delay in normal performance group for a Rel-12 UE is the same as that for a legacy UE. And since  introduces a tradeoff between reselection performance for the low performance group and normal performance group, signalling  provides flexibility to adjust reselection delay but there is no flexibility for networks to configure ‘k’.

In Option C, since ‘k’ is being fixed to a value specified in the spec today, the reselection delay in normal performance group for a Rel-12 UE is the same as that for a legacy UE. But since  is also fixed, the fixed value for  should be such that reselection performance of normal performance group is low while reselection performance of low performance carriers is not too high. Thus Option C offers no flexibility to the network but this simplifies UE implementation.

Option D is a good option interms of network operation since it offers complete flexibility to the network. But networks have to choose ‘k’ and  carefully in order to ensure that the normal performance group does not have worse reselection delay than low performance group. Moreover, UE vendors should ensure that UE complexity is not a concern. We should also note that reselection delay in normal performance group for a Rel-12 UE increases as ‘k’ increases and thus a Rel-12 UE may have larger reselection delay than legacy UE on the normal performance layers.

For Option D, networks may want to ensure that ‘k’ is at least equal to the legacy number of carriers as supported up to Rel-11 to ensure that Rel-12 UEs have at least as many cells to reselect to as the legacy UE. Also, the upper bound on ‘k’ has to be chosen carefully such that the Rel-12 UE does not have significantly worse reselection performance than the legacy UE for the normal performance group, and, also the normal performance group does not have worse reselection performance than the low performance group.

Regarding the scale factor, the range of values for should be chosen carefully. An analysis is provided in [2].

Proposal 1: Choose Option D for inter-RAT E-UTRA measurements and inter-frequency measurements with increased number of carriers.

# 4   Modified Inter-RAT Measurement Requirements

The following are the modified requirements corresponding to Option D. Letbe a scale factor by which the requirements for the low performance group will be scaled and  be the number of inter-RAT E-UTRA carriers in the normal performance group.  is the scale factor indicating the increase in cell re-selection delay for each additional inter-frequency in the low performance group in CELL_DCH with respect to the cell re-selection delay requirement per existing carrier currently defined in the specification.

## 4.1		Inter-RAT E-UTRA Measurements

If Nfreq,E-UTRA ≤ , keep current requirements for measurements of inter-RAT E-UTRA cells as in section 8.4.2.6 in TS25.133.

If Nfreq,E-UTRA > , the UE shall be able to identify a new detectable cell belonging to the ‘k’ normal performance group inter-frequency carriers in the monitored set within , and for the additional (NE-UTRA -) inter-RAT E-UTRA carriers in the low performance group, the UE shall be able to identify a new detectable cell belonging to the monitored set within

If Nfreq,E-UTRA > , for the ‘k’ normal performance group inter-frequency carriers, the UE physical layer shall be capable of reporting RSRP and RSRQ measurements to higher layers with measurement period given by . For the additional (Nfreq,E-UTRA -) inter-RAT E-UTRA carriers in the low performance group, the measurement period is given by

ms

Proposal 2: UTRAN signals the number of inter-RAT E-UTRA carriers  in the normal performance group

Proposal 3: UTRAN signals the scale factor  for the cell reselection delay in the low performance group.

Proposal 4: Allowed range for inter-RAT E-UTRA carriers is

Proposal 5: Allowed range for  is

Proposal 6: Not all combinations of and are allowed.

# 5   Modified Inter-frequency Measurement Requirements

The following are the modified requirements corresponding to Option D. Letbe a scale factor by which the requirements for the additional frequencies will be scaled and ‘k’ be the number of inter-frequency carriers in the normal performance group.  is the scale factor indicating the increase in cell re-selection delay for each additional inter-frequency in the low performance group in CELL_DCH with respect to the cell re-selection delay requirement per existing carrier currently defined in the specification.

## 5.1	FDD inter frequency measurements

The new cell re-selection delay requirements shall be defined as follows:

If , keep existing requirements as is.

If  the UE shall be able to identify a new detectable cell belonging to the ‘k’ normal performance group inter-frequency carriers in the monitored set within , and, the UE shall be able to identify a new detectable cell belonging to the additional  inter-frequencies in the monitored set within

If , when transmission gaps are scheduled for FDD inter frequency measurements the UE physical layer shall be capable of reporting measurements to higher layers with measurement period given by

, and, the UE physical layer shall be capable of reporting measurements to higher layers for the additional  inter-frequencies to higher layers with measurement period given by

Proposal 7: UTRAN signals the number of inter-frequency carriers ‘k’ in the normal performance group

Proposal 8: Allowed range for inter-frequency carriers is

Proposal 9: Not all combinations of and are allowed.

# 6		Conclusion

This contribution has evaluated option A/B/C/D discussed in the way forward in RAN4#70bis for CELL_DCH state. This contribution has also provided proposed changes to inter-frequency and inter-RAT measurement requirements. Finally, this contribution has the following proposal for the finalization of measurement requirements when the number of carriers to monitor in CELL_DCH state is increased.

Proposal 1: Choose Option D for inter-RAT E-UTRA measurements and inter-frequency measurements with increased number of carriers.

Proposal 2: UTRAN signals the number of inter-RAT E-UTRA carriers  in the normal performance group

Proposal 3: UTRAN signals the scale factor  for the cell reselection delay in the low performance group.

Proposal 4: Allowed range for inter-RAT E-UTRA carriers is

Proposal 5: Allowed range for  is

Proposal 6: Not all combinations of and are allowed.

Proposal 7: UTRAN signals the number of inter-frequency carriers ‘k’ in the normal performance group

Proposal 8: Allowed range for inter-frequency carriers is

Proposal 9: Not all combinations of and are allowed.

# 7		Reference

- R4-142530, “RAN4 agreements on UE increased carrier monitoring for further RAN2 work”

- R4-141481, “Measurement requirements to monitor additional UTRA carriers in Idle/URA_PCH/CELL_PCH states”, Qualcomm Incorporated

- R4-140398, “Increasing the minimum requirements for number of carriers which can be monitored in UTRA idle mode”

- 3GPP TS25.304 v11.4.0, “User Equipment (UE) procedures in idle mode and procedures for cell reselection in connected mode”

- R4-142520, “Agreements and way forward for increased UE carrier monitoring”