3GPP TSG-WG4 Meeting #71	R4-143680

# Seoul, Republic of Korea, 19 - 23 May 2014

Agenda item: 	7.17.1

Source: 	Qualcomm Incorporated

Title: 	New DL reference measurement channels for DCH enhancements

Document for:	Discussion

# 1	Introduction

This contribution proposes some new DL fixed RMC used to define some of the new UE performance requirements as proposed in [1].

# 2	New DL reference measurement channels

## 2.1	Null with and without DCCH

Table 1: DL reference measurement channel physical parameters (Null with and without DCCH)

| Parameter | Unit | Level |
| --- | --- | --- |
| Information bit rate | kbps | 0 |
| DPCH | ksps | 30 |
| Slot Format #I | - | Pilot-free, spreading factor 128 |
| TFCI | - | On |
| Power offsets PO1, PO2 and PO3 | dB | 0 |
| Puncturing | % | 13.9 |

Table 2: DL reference measurement channel, transport channel parameters (Null with and without DCCH)

| Parameter | DTCH | DCCH |
| --- | --- | --- |
| Transport Channel Number | 1 | 2 |
| Transport Block Size | 0 | 100 |
| Transport Block Set Size | 0 | 100 |
| Transmission Time Interval | 20 ms | 40 ms |
| Type of Error Protection | Convolution Coding | Convolution Coding |
| Coding Rate | 1/3 | 1/3 |
| Rate Matching attribute | 256 | 256 (Null with DCCH), 0 (Null without DCCH) |
| Size of CRC | 16 | 16 |
| Position of TrCH in radio frame | fixed | fixed |

## 2.2	Full with and without DCCH

The parameters for the 12.2 Kbps DL reference measurement channel are specified in Table A.11 and Table A.12. The channel coding is shown for information in figure A.5.

Table A.11: DL reference measurement channel physical parameters (Full with and without DCCH)

| Parameter | Unit | Level |
| --- | --- | --- |
| Information bit rate | kbps | 12.2 |
| DPCH | ksps | 30 |
| Slot Format #i | - | Pilot-free, spreading factor 128 |
| TFCI | - | On |
| Power offsets PO1, PO2 and PO3 | dB | 0 |
| Puncturing | % | 14.7 |

Table A.12: DL reference measurement channel, transport channel parameters (Full with and without DCCH)

| Parameter | DTCH | DCCH |
| --- | --- | --- |
| Transport Channel Number | 1 | 2 |
| Transport Block Size | 244 | 100 |
| Transport Block Set Size | 244 | 100 |
| Transmission Time Interval | 20 ms | 40 ms |
| Type of Error Protection | Convolution Coding | Convolution Coding |
| Coding Rate | 1/3 | 1/3 |
| Rate Matching attribute | 256 | 256 (Full with DCCH), 0 (Full without DCCH) |
| Size of CRC | 16 | 16 |
| Position of TrCH in radio frame | Fixed | Fixed |

# 3	Conclusion

This contribution has introduced 4 new DL reference measurement channels called ‘Full with DCCH’, ‘Null with DCCH’, ‘Full without DCCH’ and ‘Null without DCCH’, needed to define new UE performance requirements as proposed in [1]. The new RMCs with DCCH are based on existing 12.2kbps and 0kbps RMC with new DL pilot-free slot-format (spreading-factor 128) and CRC-size of 16 for both DTCH and DCCH. The new RMCs without DCCH are based on the new RMCs with DCCH but setting the rate-matching attribute of the DCCH to zero.

# 5	Reference

- R4-143683, “Further discussion on RAN4 requirements due to introduction of DCH enhancements”, Qualcomm Incorporated, RAN4#71