3GPP TSG-RAN WG4 Meeting #71                           R4-143726

# Seoul, Korea, 19 – 23 May, 2014

Source: 	CMCC

Title: 			   Simulation results for SU-MIMO advanced receivers

Agenda Item:	7.12.2

Document for:	Discussion

- Introduction

In RAN4 #70bis meeting, a way forward was agreed on SU-MIMO, including typical scenarios, and candidate reference receiver as follows [1]:

- Typical scenarios

  - Prioritize single cell high geometry scenarios to verify UE advanced receiver implementations

    - Multi-cell scenario is FFS.

  - Prioritize medium antenna correlation

  - Current 36.101 single cell multi-layer spatial multiplexing FRC test setups can be used as the starting point for aligning simulation results for demodulation

  - Other fading propagation channels and MCS values can be studied

- Reference receiver

Candidate reference receiver set: CWIC/R-ML/SLIC

In this contribution, link level simulation results of advanced receivers are provided under single cell MU-MIMO scenario in order to align results.

- Simulation results

In this section, link level simulation results of reference receivers (CWIC/R-ML/SLIC/MMSE) are provided for the following cases:

- Test 1: TM3

  - 2x2 and medium antenna correlation

  - EVA70 channel fading

  - 16QAM, 1/2, R.11 FDD

  - Different receivers: MMSE, SLIC, R-ML, L-CWIC

- Test 2: TM4

  - 2x2 and medium antenna correlation

  - EPA5 channel fading

  - 64QAM, 1/2, R.35 FDD

  - Different receivers: MMSE, SLIC, R-ML, L-CWIC

- Test 3: TM4

  - 2x2 and medium antenna correlation

  - ETU70 channel fading

  - 16QAM, 1/2, R.11 FDD

  - Different receivers: MMSE, SLIC, R-ML, L-CWIC

The simulation results of the above 3 test cases are illustrated in Figure 1~3 respectively.

Figure 1: Throughput of advanced receivers for SU-MIMO in test 1

Figure 2: Throughput of advanced receivers for SU-MIMO in test 2

Figure 3: Throughput of advanced receivers for SU-MIMO in test 3

According to the simulation results, the performance gains of reference receivers compared to the baseline MMSE receiver are summarized in Table1.

Table 1 Performance gain of advanced receivers compared to MMSE

| Reference receiver type | Test 1 SNR gain at 70% TP (dB) | Test 2 SNR gain at 70% TP (dB) | Test 3 SNR gain at 70% TP (dB) |
| --- | --- | --- | --- |
| SLIC | 1.116 | 0.725 | 0.812 |
| R-ML | 2.286 | 2.24 | 2.26 |
| CWIC | 2.03 | 3.45 | 3.19 |

- Conclusions

In this contribution, link level simulation results of advanced receivers are provided under single cell SU-MIMO scenario in order to align results.

Based on the simulation results, the performance gains of reference receivers over MMSE receiver are summarized as below:

| Reference receiver type | Test 1 SNR gain at 70% TP (dB) | Test 2 SNR gain at 70% TP (dB) | Test 3 SNR gain at 70% TP (dB) |
| --- | --- | --- | --- |
| SLIC | 1.116 | 0.725 | 0.812 |
| R-ML | 2.286 | 2.24 | 2.26 |
| CWIC | 2.03 | 3.45 | 3.19 |

- References

- R4-142376, “Way forward on SU-MIMO”, Huawei, HiSilicon, Ericsson, NVIDIA, Qualcomm, Samsung, Intel, NTT DOCOMO, MediaTek, April 2014