3GPP TSG-RAN WG4 Meeting #71 	R4-143810

# Seoul, Republic of Korea, 19 - 23 May 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.101 | CR | 1038 | rev | - | Current version: | 11.9.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Finalization of Band XXVI coexistence requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | E850_UB-core |  |  |  |  |  |  | Date: |  | 2014-05-19 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | Guard bands for additional coexistence requirements for band XXVI are TBD. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Guard bands for additional coexistence requirements for band XXVI are proposed for single uplink and dual uplinks. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Additional coexistence requirements remain incomplete. As a result, band XXVI is practically incomplete. Isolated impact analysis It is understood that there is no UE supporting Band XXVI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3.1.1, 6.6.3.1A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR 34.121-1 CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

##### 6.6.3.1.1	Additional requirement with a guard band

These requirements are applicable only for frequencies which are greater than Fguard MHz away from the UE transmit carrier frequency.

# Table 6.13a: Additional spurious emissions requirements with a guard band

| Operating Band | Frequency Bandwidth | Measurement Bandwidth | Minimum requirement | Guard Band (Fguard MHz) |
| --- | --- | --- | --- | --- |
| XXVI | 806 MHz f 813.5 MHz | 6.25 kHz | -42 dBm | 6 MHz |
|  | 806 MHz f 816 MHz | 6.25 kHz | -42 dBm | 6 MHz |
|  | 851 MHz f 859 MHz | 1 MHz | -32 dBm | 10 MHz |
|  | 851 MHz f 859 MHz | 6.25 kHz | -53 dBm | 10 MHz |

-------------------------------------------------- Unchanged sections skipped -----------------------------------------------------------

##### 6.6.3.1A.1	Additional requirement with a guard band for DC-HSUPA

These requirements are applicable only for frequencies which are greater than Fguard MHz away from the UE transmit center carrier frequency.

# Table 6.13B: Additional spurious emissions requirements with a guard band

| Operating Band | Frequency Bandwidth | Measurement Bandwidth | Minimum requirement | Guard Band (Fguard MHz) |
| --- | --- | --- | --- | --- |
| XXVI | 806 MHz f 813.5 MHz | 6.25 kHz | -42 dBm | 14.5 MHz |
|  | 806 MHz f 816 MHz | 6.25 kHz | -42 dBm | 14.5 MHz |
|  | 851 MHz f 859 MHz | 1 MHz | -32 dBm | 23.5 MHz |
|  | 851 MHz f 859 MHz | 6.25 kHz | -53 dBm | 22.5 MHz |

------------------------------------------------------------- End of change ------------------------------------------------------------