3GPP TSG-RAN WG4 #74	R4-150677

Athens, Greece, 9th Feb – 13th Feb 2015

Source: 	Ericsson

Title:	Consideration on simulation setup for CRS-IM demodulation (COMP)

Agenda Item:	7.5.2

Document for:	Discussion

# Introduction

In RAN#66, LTE_CRSIM-Perf WI is approved [1].  In one companion paper [2], the general overview on the test purposes, target scenarios, and test case list are proposed. In the study item phase, TM10 was not studied in the study item phase. In Rel-11 COMP work item phase, extensive studies have been investigated. However, no consensus was achieved. In this paper, we provide our view on CRS-IC for TM10.

# Serving cell CRS-IC vs Generic CRS-IC

Before RAN4#70bis meeting, RAN4 have discussed 5 meeting cycles for whether serving cell CRS-IC (SC-CRS-IC) provides gains in realistic CoMP scenarios, and 3 meeting cycles for whether SC-CRS-IC provides system level benefits in realistic COMP scenarios in Rel.11. Based on the majority view of RAN4, there are no performance requirements assume SC (serving cell) CRS-IC in COMP Rel-11 due to limit gain for serving cell CRS-IC. Most of companies prefer to have generic CRS-IC. Extensive system level simulation results are provided for generic CRS-IC in [3]. System level simulation results show that:

- CRS-IC provides large gains in terms of performance in DPS and DPB scenarios, but also for No CoMP operation of TM10. The gains come from the fact that non-colliding CRS is a major contributor to the overall interference, and interference coordination without reducing the CRS interference is not useful.

Based on above observation, in this work item, we just focus on the generic CRS-IC, rather than SC-CRS-IC.

Proposal 1: Generic CRS-IC is assumed as the reference receiver for TM10 operation.

# CRS configuration

For CRS configuration, both colliding CRS and non-colliding CRS have been discussed in Rel-11 CoMP WI. For colliding CRS, the CRS doesn’t interfere any PDSCH data RE. CRS cancelation can only improve the channel characteristic estimation. Hence, for colliding CRS case, the cancelation of CRS cannot bring too much benefit. For non-colliding CRS, the CRS interferes the PDSCH data RE. The cancelation of CRS can remove the interference and thus improve the PDSCH demodulation performance. Extensive link level simulation results have validated the above observations. Hence, in this work item, we just focus on the non-colliding CRS case.

Proposal 2: For TM 10, non-colliding CRS is assumed, wherein the CRS between serving and dominant aggressor/interfering cell as well as CRS between dominant aggressor/interfering cells are assumed to be non-colliding.

# CRS related signaling for TM10 and corresponding UE behavior

For non-TM10 mode, as a baseline assumption, Rel-11 CRS-AssistanceInfo is provided. However, for TM10, the CRS interference may come from:

  - Serving cell

  - Transmission point within COMP cooperating set

  - Aggressor cells outside COMP cooperating set

For serving cell, the full CRS interference is available. Hence, this information is not necessary to explicitly provided by the network side.

For Transmission point within COMP cooperating set, in the CSI-RS-ConfigNZP-r11 signalling, the following information is provided:

  - Cell identity

  - CRS ports count

  - MBSFN subframe configuration

Based on this information, full knowledge about CRS of the transmission point within COMP cooperating set is known. In case QCL information is provided, the CRS information for the corresponding transmission point is not necessary to provide.

For the aggressor cells outside COMP cooperating set, Rel-11 CRS assistance information may be provided.

Hence, for UE to support TM10, when TM10 is configured,  not only the CRS explicitly indicated in the Rel-11 CRS assistance information shall be cancelled, but also the CRS implicitly indicated in the COMP related signaling can be removed whatever is the strongest.

Proposal 3: For UE to support TM10, when TM10 is configured,  not only the CRS explicitly indicated in the Rel-11 CRS assistance information shall be cancelled, but also the CRS implicitly indicated in the COMP related signaling can be removed.

# Consideration on the Setup

Considering the basic assumption, UE can handle up to two cells’ CRS interference. For TM10, we can configure three points in the test. The first point is the serving cell; the second point is the transmission point which can transmit PDSCH, and the third point is the aggressor cell. DPS scheme is configured. The point which transmits PDSCH can dynamically switch between the firs point and the second point. The following behavior is expected:

Case 1: The first transmission point is transmitting PDSCH. In this case, the CRS from the second transmission point and the CRS from the third transmission point shall be cancelled.

Case 2:  The second transmission point is transmitting PDSCH. In this case, the CRS from serving cell and CRS from the third transmission point shall be cancelled.

Case 2

Case 1

Interference the UE can cancel

Strongest interferes are canceled only, depending on the conditions)

PDSCH and CRS

PDCCH

Serrving cell

Figure 1: The system setup for CRS-IM for TM10

The basic setup is shown in Figure 1.

Proposal 4: Consider DPS or DPB as the transmission scheme in the test setup for TM10.

# TM10 test setup for CRS-IC receiver

| Parameter |  | Unit | TP 1 | TP 2 | TP3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | ρA | dB | 0 | 0 | 0 |
|  | ρB | dB | 0 (Note 1) | 0 | 0 |
|  | σ | dB | -3 | -3 | -3 |
| Beamforming model |  |  | N/A | As specified in clause B.4.1 | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | (Note 2) | Antenna ports 0,1 |
| CSI reference signals 0 |  |  | Antenna ports {15,16} | N/A | N/A |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | N/A | N/A |
| CSI reference signal 0 configuration |  |  | 0 | N/A | N/A |
| CSI reference signals 1 |  |  | N/A | Antenna ports {15,16} | N/A |
| CSI-RS 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | N/A | 5 / 2 | N/A |
| CSI reference signal 1 configuration |  |  | N/A | 8 | N/A |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | 2/ 0010000000000000 | 2/ 0010000000000000 | N/A |
| Zero-power CSI-RS1 configuration ICSI-RS / ZeroPower CSI-RS bitmapS |  | Subframes/bitmap | 2/ 0000010000000000 | 2/ 0000010000000000 | N/A |
|  |  | dB | TBD | TBD | TBD |
| at antenna port |  | dBm/15kHz | -98 | -98 | -98 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell Id |  |  | 0 | 127 | 128 |
| Number of control OFDM symbols |  |  | 2 | 2 | 1 |
| Timing offset between TPs |  |  | N/A | Reference Value in Table 8.3.1.3.2-3 | 3 us |
| Frequency offset between TPs |  | Hz | N/A | 0 | 300Hz |
| Number of allocated resource blocks |  | PRB | 50 | 50 | The load is TBD |
| PDSCH transmission mode |  |  | 10 | 10 | TM3 |
| Probability of occurrence of PDSCH transmission(Note 3) |  | % | 30 | 70 | N/A |
| Symbols for unused PRBs |  |  | OCNG (Note 4) | OCNG (Note 4) | N/A |
| Note 1: Note 2: REs for antenna ports 0 and 1 have zero transmission power. Note 3: PDSCH transmission from TPs shall be randomly determined independently for each subframe. Probabilities of occurrence of PDSCH transmission from TPs are specified. The probability of occurrence of PQI set in each TP is equal. Note 4: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |  |

Table 1: Test Parameters for timing offset compensation with DPS transmission

Table 8.3.1.3.2-2: Configurations of PQI and DL transmission hypothesis for each PQI set

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 | ZP CSI-RS 0 | PDSCH | Blanked |
| PQI set 1 | CSI-RS 0 | ZP CSI-RS 1 | PDSCH | Blanked |
| PQI set 2 | CSI-RS 1 | ZP CSI-RS 0 | Blanked | PDSCH |
| PQI set 3 | CSI-RS 1 | ZP CSI-RS 1 | Blanked | PDSCH |

Table 8.3.1.3.2-3: Performance Requirements with DPS transmission

| Test Number | Timing offset(us) | Reference Channel | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TP 1 | TP 2 | TP 1 | TP 2 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | TBD | TBD | OP.1 FDD | OP.1 FDD | EPA5 | EPA5 | 2x2 Low | 70 | TBD | ≥2 |
| Note 1: The propagation conditions for TP 1and TP 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of TP 1 and TP 2. Note 3: SNR corresponds to of both TP 1 and TP 2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |

# Conclusion

In this contribution, generic view on TM10 test under CRS-IM WI is discussed. We have the following proposals:

Proposal 1: Generic CRS-IC is assumed as the reference receiver for TM10 operation.

Proposal 2: For TM 10, non-colliding CRS is assumed, wherein the CRS between serving and dominant aggressor/interfering cell as well as CRS between dominant aggressor/interfering cells are assumed to be non-colliding.

Proposal 3: For UE to support TM10, when TM10 is configured,  not only the CRS explicitly indicated in the Rel-11 CRS assistance information shall be cancelled, but also the CRS implicitly indicated in the COMP related signaling can be removed.

Proposal 4: Consider DPS or DPB in the test setup for TM10.

# Reference

- RP-142263, New Work Item proposal: Perf. Part: CRS Interference Mitigation for LTE Homogenous Deployments, Ericsson, Dec. 2014.

- R4-150674, Overview for CRS-IM WI, Feb. 2015.

- R4-140697, the use of CRS-IC in Comp, Ericsson, Feb. 10 ~ Feb. 14, 2014.