3GPP TSG-RAN WG4 Meeting #76bis	R4- 155668

Sophia-Antipolis, France, 12 – 16 Oct, 2015

Source: 	Huawei, HiSilicon

Title: 	Further discuss the issue on PRACH performance under high speed scenario

Agenda Item:	9.5.5

Document for:	Discussion

Introduction

In RAN4 meeting #76, the issue on PRACH performance under high speed scenario was widely discussed and the agreement was achieved in [1] as following,

- Issues for the PRACH under the high speed scenario:

  - The current cyclic shift restriction set defined in TS36.211 for high speed cells is limited to frequency offset within PRACH, i.e., +/-1.25KHz, the Doppler shift out of the range of [-1.25KHz, +1.25KHz] would cause the detection ambiguity.

RAN4 has identified the issue on degraded PRACH performance under high speed scenario. As for the solutions to improving uplink random access performance, there was solution proposed in [2], summarized as following,

  - New cyclic shift restriction set that is suitable for uplink frequency offset up to twice of the PRACH Subcarrier Spacing, .e., +/-2.5KHz;

In this contribution, we will further discuss the issue on PRACH performance and share our view about the methods for improving uplink random access performance.

Discussion on methods for improving uplink random access performance

Unidirectional eNB deployments

Doppler shift for UE close to RRH sites

The impact of unidirectional eNB deployment on random access was discussed in [5]. In this deployment configuration the UE receive DL transmissions from an antenna in one direction and its UL transmission is received by an antenna in another direction as the figure2 described in [5]. In TDD mode, the UE transmits on uplink with the tuned downlink carrier frequency (which is the initial carrier frequency plus Doppler shift that UE experienced on downlink), as the Doppler shift experienced on uplink with opposite sign. As a result, the Doppler shift eNodeB received is the sum of Doppler shift that UE experienced on downlink and Doppler shift experienced on uplink. As the two Doppler shifts have opposite sign, the eNodeB experiences nominal UL carrier frequency without Doppler shifts when the absolute values of the two Doppler shifts are equal.

However, when the UE speed is 400km/h or higher and the carrier is 3.5 GHz, we can get the Doppler shift is 1458Hz on downlink in table 1. When the UE is close to RRH sites (we copy the figure2 in [5] here), the Doppler shift experienced on uplink is very tiny. As the result, the RRH receives the uplink signal with Doppler shift almost the same as the Doppler shift (1458Hz) on downlink. We give our simulation results of Doppler shift RRH experienced in Figure 2 based on scenario in the Figure 1. At this scenario, the Doppler shift the RRH experienced is also more than one time PRACH subcarrier interval with unidirectional eNodeB deployments.

Table 1: Doppler frequency for different bands

| UE speed (km/h) | Doppler frequency |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.9GHz carrier |  | 2.3 GHz carrier |  | 2.6 GHz carrier |  | 3.5 GHz carrier |  |
| DL/UL | Downlink | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink | Uplink |
| 150 | 264 | 528 | 319 | 639 | 361 | 722 | 486 | 972 |
| 200 | 352 | 704 | 426 | 852 | 481 | 963 | 648 | 1296 |
| 250 | 440 | 880 | 532 | 1065 | 602 | 1204 | 810 | 1620 |
| 300 | 528 | 1056 | 639 | 1278 | 722 | 1444 | 972 | 1944 |
| 350 | 616 | 1231 | 745 | 1491 | 843 | 1685 | 1134 | 2269 |
| 400 | 704 | 1407 | 852 | 1704 | 963 | 1926 | 1296 | 2593 |
| 450 | 792 | 1583 | 958 | 1917 | 1083 | 2167 | 1458 | 2917 |

Figure 1 (figure2 in [5]): Path loss and Doppler shifts associated with a moving UE with UL RX and DL TX in opposite directions. From Dmin = 30m, Ds = 1000m, f = 2.7GHz and speed 500km/h.

Figure 2 Doppler shift RRH experienced based on scenario in the figure2 in [5]

Based on the discussion in above, we observe that

Observation 1; At the scenario with UE speed more than 400km/h and the carrier frequency 3.5GHz,

----the Doppler shift the RRH experienced is more than one time PRACH subcarrier interval with unidirectional eNodeB deployments

Doppler shift for UE handover

Figure 3 UE at Cell edge

It is well known that, to maintain signal quality, UE always perform handover process. And the first step is to measure the signal quality from target handover eNodeB. So, UE should receive the signal from target handover eNodeB e.g. RRH2 in figure 3. However, in unidirectional eNodeB deployment, UE can only transmit uplink signal to RRH2 and receive downlink signal from RRH1.

To perform handover process, UE should not transmit uplink signal to RRH2, but receive downlink signal from RRH2. As a result, signal received by eNodeB could experience two times Doppler shift. So, the Doppler shift in unidirectional eNodeB deployment is not reduced compared to in SFN model for UE at cell edge.

Based on the discussion above, we observed that

Observation 2: for UE at cell edge, the Doppler shift in unidirectional eNodeB deployment is not reduced compared to in SFN model.

And, in the existing network, there is also existing similar problem while the UE velocity is high. So we should develop new solution to enhance PRACH.

Based on our observation and discussion, we propose that

Proposal 1: random access scheme should be enhanced to cope with Doppler shift great than one times PRACH subcarrier interval.

Develop new cyclic shift restriction set

The Doppler frequency impact on LTE random access was analysed in detail in [4]. We further discuss the high Doppler shift impact on correlation (one random access sequence and the sequence distorted by Doppler shift) peak. In the following, we give our simulation result with the assumptions . From the simulation result, we can observe that there will be five correlation peaks while the Doppler shift beyond the range of [-1.25KHz, +1.25KHz].And the difference between Figure 4 and Figure 5 is that in Figure 4 the great peak distributes atwhile the Doppler shift is and the great peak distributes at while the Doppler shift is in Figure 5.

To avoid the interference to other UE or eliminated the detection ambiguity, the cyclic shift zone corresponding to five correlation peaks should be reserved to one UE, which is different from the existing high speed restricted PRACH sequence set (based on the assumption of three unambiguous detection window). So, one of the feasible solutions is to develop new cyclic shift restriction set. This solution has good compatibility with the existing LTE system. And the uplink random access performance could be improved without great change at UE and eNodeB.

Figure 4:

Figure 5:

Capacity for new cyclic shift restriction set

As agrees in previous, the parameters for SFN scenario is listed in [6], we copy the parameters in the following,

Table 6.2.3.1-1: Parameters for SFN scenarios

| Parameter | Value |
| --- | --- |
|  | [1000] m |
|  | [10] m |
|  | 350 km/h |
|  | TBD Hz |

Table 6.2.3.1-2: Parameters for SFN scenarios

| Parameter | Value |
| --- | --- |
|  | [500] m |
|  | [5] m |
|  | 350 km/h |
|  | TBD Hz |

Based on the design principle in [7] as below,

(1)

Where  is the radius of cell (km/s), is the max delay spread, is the length of Zadoff-Chu sequence, (us)is the duration of PRACH, is the guard samples for pulse shaped filter().

In SFN scenario, equals to and ( is velocity of light). Based on equation (1), we get the range of  (at scenario,)

(2)

and get the range of  (at scenario,)

(3)

Based on the calculation results, we can get that the eNodeB needn’t configure too great value for  at SFN scenario. Table 2 gives the available random access sequences number of different search windows for different .

Table 2: Available random access sequence number

|  | Supporting cell Radius | 3 search window | 5 search window (base on Alcatel-Lucent proposal) |  |
| --- | --- | --- | --- | --- |
| 15 | 1.1km | 13618 | 3128 | 48 |
| 18 | 1.5km | 11100 | 2518 | 39 |
| 22 | 2km | 8902 | 1956 | 30 |
| 26 | 2.7km | 7248 | 1584 | 24 |
| 46 | 5km | 3568 | 714 | 11 |
| 55 | 6km | 2846 | 526 | 8 |
| 82 | 10km | 1620 | 232 | 3.6 |

In SFN scenario, there is only one neighbor cell. So, as long as the value  is equal to or more than 2, the PRACH detection performance can be ensured.

From the table 1, we can observe that the number of random access available sequence is enough to avoid interference from neighbour cells in the range of possible configured  in SFN scenario. Even though, the cell radius is reaching 10km, there are enough random access sequences to use.

Observation 3: the number of random access available sequence is enough to avoid interference from neighbour cells

Proposal 2: it is feasible to develop new cyclic shift restriction set when the Doppler shift beyond the range of [-1.25KHz, +1.25KHz].

Accordingly, we consider define PRACH detection requirements in RAN4.

Conclusions

In this contribution, we further discuss the issue on PRACH performance and share our view about the methods for improving uplink random access performance.

Based on the analysis and simulation results,

We observe that

Observation 1; At the scenario with UE speed more than 400km/h and the carrier frequency 3.5GHz,

----the Doppler shift the RRH experienced is more than one time PRACH subcarrier interval with unidirectional eNodeB deployments

Observation 2: for UE at cell edge, the Doppler shift in unidirectional eNodeB deployment is not reduced compared to in SFN model.

Observation 3: the number of random access available sequence is enough to avoid interference from neighbour cells

And we propose that

Proposal 1: random access sequence should be enhanced to cope with Doppler shift great than one times PRACH subcarrier interval.

Proposal 2: it is feasible to develop new cyclic shift restriction set when the Doppler shift beyond the range of [-1.25KHz, +1.25KHz].

Accordingly, we consider define PRACH detection requirements in RAN4.

Reference

[1] Huawei, HiSilicon, R4-155145, “HST ad hoc minutes”, 3GPP TSG-RAN WG4 Meeting #76, Beijing, China, 24 – 28 Aug, 2015.

[2] Alcatel-Lucent, R4-154364, “Cyclic Shift Restriction Set for High Speed Cells”, 3GPP TSG-RAN WG4 Meeting #76, Beijing, China, 24 – 28 Aug, 2015.

[3] Ericsson, R4-154620,“Analysis of Unidirectional deployment for BS”, 3GPP TSG-RAN WG4 Meeting #76, Beijing, China, 24 – 28 Aug, 2015.

[4] Huawei, HiSilicon, R4-154252,“ Discussion on PRACH performance under high speed scenario” , 3GPP TSG-RAN WG4 Meeting #76, Beijing, China, 24 – 28 Aug, 2015.

[5] Ericsson, R4-154620 ,“Further analysis of PRACH using unidirectional deployment”, 3GPP TSG-RAN WG4 Meeting #76, Beijing, China, 24 – 28 Aug, 2015.

[6] TR36.878, R4-154762, “Study on performance enhancements for high speed scenario in LTE”, 3GPP TSG-RAN WG4 Meeting #76, Beijing, China, 24 – 28 Aug, 2015.

[7] Matthew Baker, “ LTE-The UMTS Long Term Evolution From Theory to Pratice”