3GPP TSG-RAN WG4 Meeting #76bis	    R4-15xxxx

# Sophia Antipolis, France, 12 - 16 Oct, 2015

Agenda Item:	9.5.2

Source: 	Samsung

Title: 	Cell search simulation results for leakyy cable

Document for:	Discussion

Introduction

In last RAN4 meeting, the leaky cable channel model for high speed scenario was identified in [1], and RRM simulation assumptions for leaky cable were argeed in [2].

In this contribution, we provide the simulation results of cell search delay for leaky cable channel according to the agreed assumptions.

Smulation Results

The detailed simulation assumptions are listed in Table A.1~4 in Annex A.

Following Table 1 shows the simulation results of 90th percentile of cell search delay under AWGN channel and leaky cable channel. The simulation results are also illustrated in Figure 1 ~ 4.

Table 1: 90th percentile of cell search delay (ms)

| Channel Case | AWGN |  |  | Leaky cable |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SINR= -6dB | SINR= -7dB | SINR= -8dB | SINR= -6dB | SINR= -7dB | SINR= -8dB |
| 1 | 10 | 40 | 190 | 100 | 190 | 510 |
| 2 | 30 | 40 | 120 | 80 | 160 | 350 |
| 3 | 10 | 20 | 40 | 30 | 30 | 40 |
| 4 | 10 | 40 | 250 | 90 | 170 | 490 |
| 5 | 40 | 70 | 180 | 70 | 140 | 240 |
| 6 | 40 | 80 | 190 | 80 | 140 | 240 |
| 7 | 20 | 30 | 50 | 30 | 40 | 40 |
| 8 | 40 | 80 | 190 | 70 | 140 | 260 |

Figure 1: 90th percentile of cell detection time (AWGN, Synchronous)

Figure 2: 90th percentile of cell detection time (AWGN, Asynchronous)

Figure 3: 90th percentile of cell detection time (leaky cable, Synchronous)

Figure 4: 90th percentile of cell detection time (leaky cable, Asynchronous)

Based on the above simulation results, it can be observed:

- Comparing the different channel condition, leaky cable channel for desired cell (AWGN with 875Hz offset for interfering cells) shows much worse performance for synchronous cases. However, the legacy cell search delay requirement, i.e. 600 ms, can still be satisfied for these cases with the side condition SINR = -6dB (cell 3 Ior/Noc = 1.25dB).

- For asynchronous cases, leaky cable channel show slightly worse performance than AWGN channel.

The above observations are summarized as:

Observation 1: High speed UE under leaky cable channel can meet the legacy cell search requirements, i.e. 600ms.

Conclusion

In this contribution, we provide the simulation results of cell search performance for high speed UE under leaky cable channel. Based on the simulation results, one observation is made.

Observation 1: High speed UE under leaky cable channel can meet the legacy cell search requirements, i.e. 600ms.

Reference

- R4-155178, TP: Channel model for leaky cable in tunnel, Huawei, HiSilicon

- R4-154778,	TP for TR 36.878: RRM Simulation assumptions for leaky cable, Huawei,HiSilicon

Annex A: Simulation Assumptions

Table A.1: Link Simulation Parameters for cell identification

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 | Channel 1 |
| Data and Control PSD relative to RS PSD | dB | 0 | 0 | 0 |
| P-SCH and S-SCH PSD relative to RS PSD | dB | 0 | 0 | 0 |
| Number of RB’s |  | 6 | 6 | 6 |
| RB Utilization | % | 100 | 100 | 100 |
| Data Modulation | - | QPSK | QPSK | QPSK |
| Frame Structure Type | - | 1 | 1 | 1 |
| CP Length | - | Normal | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 | 0 |
| 1) Relative Delay of 1st Path (synchronous) | μs | 0 | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | μs | 0 | 1.5 ms | 3.0 ms |
| Ior/Noc | dB | 5.18 | 0.29 | Test 1: 1.25 Test 2: 0.25 Test 3: -0.75 |
| Number of Tx antennas | - | 1 | 1 | 1 |
| P-SCH Sequence ID | - | See Table A.3, A.4 | See Table A.3, A.4 | See Table A.3, A.4 |
| S-SCH Sequence ID [2] | - | See Table A.3, A.4 | See Table A.3, A.4 | See Table A.3, A.4 |
| Propagation Condition | - | AWGN with 875Hz offset | AWGN with 875Hz offset | Leakyy cable |
| Ioc Model | - | AWGN |  |  |

Table A.2: Other simulation assumption parameters for cell identification

| Simulation parameters | Comments/values |
| --- | --- |
| Prior knowledge of Cell 1 and Cell 2 by the UE | Yes |
| Cell 1, 2, 3 carrier frequency | Same |
| False detect threshold | Required as in a real UE implementation |
| UE having apriori knowledge of system being synchronous or synchronous (by signaling) | No |
| Duty cycle | 100% (to represent non-DRX case) |
| Performance criterion for comparison | 90th percentile acquisition time for “correct” cell detection of both P-SCH and S-SCH sequence id’s. |
| CP length detection | Both short or long CP may be present, hence UE needs to detect CP length |
| Receive antennas | 2 (uncorrelated) |

Table A.3: Cell ID Combinations to be simulated

| case # | Cell 3 (Desired Cell) |  | Cell 1 (Interferer 1) |  | Cell 2 (Interferer 2) |  | Scenario |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | psc3 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 2 | psc1 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 3 | psc1 | ssc1a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 4 | psc3 | ssc1a, ssc1b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 5 | psc3 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Asynchronous |
| 6 | psc1 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Asynchronous |
| 7 | psc1 | ssc1a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Asynchronous |
| 8 | psc3 | ssc1a, ssc1b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Asynchronous |

Table A.4: PSC, SSC indices for simulations

| Label | Code index |
| --- | --- |
| psc1 | 29 |
| psc2 | 25 |
| psc3 | 34 |

| Label | Code index | Cell group index |
| --- | --- | --- |
| (ssc1a, ssc1b) | (6, 8) | 36 |
| (ssc2a, ssc2b) | (10, 12) | 40 |
| (ssc3a, ssc3b) | (7, 9) | 37 |
| (ssc1a, ssc3b) | (6, 9) | 65 |