3GPP TSG-RAN WG4 Meeting #76bis	R4-155741

Sophia - Antipolis, France, 12 – 16 Oct, 2015

Source: 	ZTE

Title: 	Results for Demodulation Performance Requirements for D2D Discovery

Agenda Item:	6.1.2

Document for:	Discussion

- Introduction

In the RAN4 #76 meeting, simulation results for D2D demodulation performance requirements were submitted and were summarized in [1]. The results show a large span of SNR at 5%BLER.

In this contribution, we provide alignment and impairment results for demodulation performance requirements for D2D discovery according to CR [2].

- Results and discussion

  - Single D2D link for Discovery

The purpose of the requirements is to verify the PSDCH demodulation performance with a single active PSDCH link under different operating scenarios and channel conditions.

Results cover different test cases below:

- Case1 for Test1 (11.2.1/11.2.2/11.4.1) in CR[2]

- Case2 for Test1 (11.4.1) in CR[2]

Table 1 Cases for single D2D link

|  | Channel BW | Modulation | Coding | Info Bits | CRC | #PRBs | #HARQ ReTx | Time seperation | Frequency Hopping | Channel | Time offset | Freq offset |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case1 | 5MHz | QPSK | Turbo | 232 | 24 | 2 | 0 | N/A | N/A | EPA5 | +1 usec | +200Hz |
| Case2 | 10MHz |  |  |  |  |  |  |  |  |  | +1 usec | +300Hz |

Figure 1 and 2 show the case1 and case2 with simulation parameters above. Table 2 shows SINR at 30% BLER of case 1and case 2.

Figure 1 Case 1 for Single D2D link for Discovery           Figure 2 Case 2 for Single D2D link for Discovery

Table 2 SINR at 30% BLER

|  | Case 1 | Case 2 |
| --- | --- | --- |
| @30% BLER | 3.41dB | 3.41dB |

Observation 1：The differences of frequency offset of TX UE (w.r.t. Rx synch source) between 200Hz and 300Hz have little impact on the demodulation performance.

  - Two-link Power Imbalance for Discovery

Based on the agreements in CR [2], link level evaluation cover

- Case 25 for Test 1 (11.3.1/11.3.2) in CR[2]

Table 3 Cases for Two-link Power Imbalance

|  | Channel BW | Modulation | Coding | Info Bits | CRC | #PRBs | #HARQ ReTx | Time seperation | Frequency Hopping | Channel | Time offset | Freq offset |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case25 | 5MHz | QPSK | Turbo | 232 | 24 | 2 | 0 | N/A | N/A | AWGN | 0 | 0 |

* ICS is chosen as -21dBc.

The evaluation result based on Table 3 is provided in Figure 3 and Table 4 gives the SINR at 30% BLER.

Figure 3 Case 25 for Two-link Power Imbalance for Discovery

Table 4 SINR at 30% BLER

|  | Case 25 |
| --- | --- |
| @30% BLER | -0.21dB |

The alignment results and impairment results are summarized in Table 5.

Table 5 Alignment and impairment results for D2D discovery

|  | Alignment results | impairment results |
| --- | --- | --- |
| Case 1 | 3.41 dB | 3.91 dB |
| Case 2 | 3.41 dB | 3.91 dB |
| Case 25 | -0.21 dB | 0.31 dB |

- Conclusion

In this contribution, we provide alignment and impairment results for demodulation performance requirements for D2D discovery.

- References

R4-154342, “Summary of D2D demodulation performance simulation results”, Qualcomm Incorporated, 3GPP TSG RAN WG4 meeting#76

R4- 155206, “CR on demodulation performance requirements for D2D Discovery”, Qualcomm Incorporated, 3GPP TSG RAN WG4 meeting#76