3GPP TSG-RAN Working Group 4 Meeting #76bis	R4-155921

Sophia Antipolis, France, 12th October – 16th October 2015

Source:	DISH Network

Title:	Text Proposal for TR 36.869: Section 7.2 Interoperability by Means of CA

Agenda item:	8.2.2

Document for:	Approval

## 1	Introduction

In RAN #67, a new work item for AWS extension band for LTE was approved [1].  The agreed band plan is:

- 70+90 (1710 - 1780 MHz / 2110 - 2200 MHz, fixed duplex with downlink intra-band CA across 90 MHz span)

One of the objectives of the WI is to “support interoperability across the complete band (1710-1780/2110-2200MHz) by means of intra-band CA”.

In this contribution, we propose a way forward to ensure interoperability objectives are met in the new AWS extension band and an accompanying text proposal for TR 36.869.

## 2	Supporting Interoperability in the AWS Extension Band

The fundamental premise of interoperability is a single band class covering all the frequencies in the band.  This implies UEs supporting Band 66 will be capable of accessing the full 70+90 MHz frequency range regardless of differing UE capabilities.  Currently, Band 66 consists of 70 MHz of uplink spectrum (1710-1780 MHz) and 90 MHz of downlink spectrum (2110-2200 MHz); it is an asymmetric band plan that requires features to ensure that the band can be effectively utilized in its entirety.  RAN4 agreed and RAN approved on a band plan with a fixed duplex spacing of 400 MHz with the intension of supporting interoperability by means of intra-band CA [2].

It has been proposed [3,4] to make 2DL/1UL downlink intra-band carrier aggregation support mandatory for UEs supporting Band 66 in order to support interoperability; however, the proposals were not agreed.  Given the current band arrangement with a fixed duplex spacing, mandating intra-band CA for UEs seems to be the only option for fulfilling interoperability and also consistent with the WI objective of “support interoperability across the complete band (1710-1780/2110-2200MHz) by means of intra-band CA”.  Nonetheless, there may be some concerns that mandating intra-band CA precludes low cost (i.e. non-CA) UEs and MTC devices from being supported in the band.  Therefore, a compromise solution to address both interoperability and low cost UE concerns in the band is proposed, which is to mandate intra-band CA to CA-capable UEs only.  The proposed compromise solution achieves two goals:

- allows introduction of low cost (non-CA) UEs and MTC devices in the band, and

- ensures a Band 66 UE base which supports the full utilization of 90 MHz downlink and is fully interoperable

## 3 				Conclusions and Proposals

It is understood by the band plan definition that all Band 66 UEs will be capable of accessing the full 70+90 MHz frequency range regardless of differing UE capabilities:

Observation:  All Band 66 UEs must be able to access the full 70+90MHz frequency range

The current Band 66 arrangement with a fixed duplex spacing necessitates intra-band CA in order to utilize the full 90 MHz of downlink spectrum and meet the interoperability objectives.  In order to address potential concerns regarding low cost UE and MTC support in the band, the following compromise solution is proposed:

Proposal:  Mandate all CA-capable UEs supporting Band 66 to support 2DL/1UL intra-band CA specified in the TR 36.869

## References

RP-150428, “New WI: AWS-Extension Band for LTE”, Ericsson, Nokia Networks, Verizon Wireless, AT&T, DISH Network, Rogers, Telus, Samsung, Huawei, HiSilicon, Alcatel-Lucent, LGE, Nokia Corporation, Intel, T-Mobile, US Cellular, 3GPP RAN#67, Shanghai, China, March 2015

TR 36.849, “Study on Advanced Wireless Services (AWS) extension band for LTE v12.0.0”, Qualcomm, 3GPP RAN#66, Maui, U.S.A, December 2014

R4-151326, “AWS band carrier aggregation”, Nokia Networks, 3GPP RAN4 #74bis, Rio, Brazil, April 2015

R4-154371, “On AWS Interoperability”, DISH Network, 3GPP RAN4 #76, Beijing, China, August 2015

## << Unchanged sections omitted >>

## << Start of changed sections >>

## 7.2				Interoperability by means of CA

It is observed that all Band 66 must be able to access the full 70+90MHz frequency range.  Interoperability will be met by mandating all CA-capable UEs supporting Band 66 to support 2DL/1UL intra-band CA specified in this technical report.

## << End of changed sections >>