3GPP TSG-RAN WG4 Meeting #76bis	 R4-155934

Sophia-Antipolis, France, 12 to 16 October 2015

Agenda Item:	9.5.5

Source: 	Alcatel Lucent

Title: 	TP for TR36.878: New Cyclic Shift Restriction Set for Very High Speed Cells

Document for:	Approval

# Introduction

In RAN4#76, it was identified that the current cyclic shift restriction set defined in TS36.211 [1] for high speed cells is limited to frequency offset within PRACH, i.e., +/-1.25KHz, the Doppler shift out of the range of [-1.25KHz, +1.25KHz] would cause the detection ambiguity [2,3,4]. Also, in RAN4#76, a new Cyclic Shift Restriction Set for Very High Speed Cells was proposed [5] that can be used for Doppler shift up to the range of [-2.5KHz, +2.5KHz]. In this document, we present the TP for the new Cyclic Shift Restriction Set proposed in [5] as a possible essential enhancement option  for TR36.878 [6].

# Text Proposal for 36.878

=========================================

6.5.3	Possible essential enhancement options

6.5.3.1	A new restricted set of cyclic shifts for extra high speed cells

It is well known that the cyclic shift distortion caused by the frequency offset may create false alarm correlation peaks during PRACH detection. The solution adopted in LTE to address this issue is to define a restricted set of the cyclic shifts for each root Zadoff-Chu sequence [1]. The usage of the exiting restricted sets makes it possible to retain an acceptable false alarm rate, while maintaining a high detection performance even for high-speed UEs with Doppler shifts within [-1.25KHz, +1.25KHz]. However, the exiting restricted sets are no longer suitable when Doppler shifts exceed the range of [-1.25KHz, +1.25KHz], as identified in [2, 3, 4].

In the following, a new cyclic shift restriction set is presented for uplink Doppler frequency offset up to twice of the PRACH Subcarrier Spacing, i.e., [-2.5KHz, +2.5KHz].

From the  root Zadoff-Chu sequence  for PRACH preambles

where  is the length of the Zadoff-Chu sequence for PRACH preambles, PRACH preambles  with zero correlation zones of length can be generated by the use of the cyclic shifts  of the new restricted set, according to

where the cyclic shifts  of the new restricted set are given by

where  is obtained by a parameter zeroCorrelationZoneConfig configured by the network  [6], and the parameters for the new restricted sets of cyclic shifts are derived from , which is given by

where  is the smallest non-negative integer that fulfils  for the root Zadoff-Chu sequenceZC root sequence. is actually the cyclic shift corresponding to a Doppler shift of magnitude of PRACH subcarrier spacing of 1.25KHz.

For , the parameters  are derived as follows:

For , the parameters  are derived as follows::

For all other values of , there are no cyclic shifts in the new restricted set.

# References

- 3GPP TS 36.211, “E-UTRA: Physical channels and modulation”

- R4-154363, “Impact of High Doppler on PRACH Detection”, Alcatel-Lucent

- R4-152604, “BS performance evaluation under new scenarios”, Huawei

- R4-152961, “Physical layer limits for Doppler frequency management”, Ericsson

- R4-154364, “New Cyclic Shift Restriction Set for Very High Speed Cells,” Alcatel-Lucent

- 3GPP TS 36.331, “E-UTRA: Radio Resource Control (RRC); Protocol specification”

- R4-15xxxx, 3GPP TR 36.878 V1.0.1 (2015-09), Study on performance enhancements for high speed scenario in LTE

- R4-155933, “Further discussion of the New Cyclic Shift Restriction Set for Very High Speed Cells,” Alcatel-Lucent