3GPP TSG-RAN WG4 Meeting #76bis	R4-156261

Sophia Antipolis, France, 12 - 16 October 2015

Source:	Nokia Networks

Title:	Interruptions with Carrier Aggregation in LAA

Agenda Item:	7.13.4

Document for:	Discussion

# Introduction

In [1] it was agreed that the following TS 36.133 sections may be impacted due to the introduction of Licensed-Assisted Access in unlicensed spectrum:

- Section 7.6 Radio Link monitoring

- Section 7.7 SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation

- Section 7.8 Interruptions with Carrier Aggregation

- 7.9 Maximum Transmission Timing Difference in Carrier Aggregation

- Section 8.1 Measurement gap

- New Section 8.x Requirements for LAA measurement including RSRP/RSRQ and RSSI

- Section 8.2 Capabilities for Support of Event Triggering and Reporting Criteria

- New Section 9.1.x Measurement Accuracy including RSRP/RSRQ and RSSI

In this paper we discuss interruptions with carrier aggregation defined in section 7.8, when LAA in unlicensed spectrum is introduced.

# Discussion

In the last meeting it was discussed whether interruptions are needed for LAA SCells. Some companies stated that because LAA SCells are planned to be on 5 GHz band, they are likely operated on different chipsets than PCells and therefore interruptions would not be generated.

LAA is currently being specified and there are so far no products on the market. The situation is very similar to when CA was specified. Also at that time it was discussed whether control of the second receiver would impact activity on the PCell. Initially this was not seen as a problem. Later during the CA discussions it was recognized that interruptions on the serving cell could happen in certain implementations, and in order to enable good UE power saving opportunities RAN4 introduced UE autonomous interrupts for CA.

In RAN4 we have now introduced UE autonomous interrupts in connection for a large range of CA and DC situations. Additionally RAN2 introduced signalling making it possible for the UE to indicate that it may benefit from interrupts and network could allow additional interrupts compared to RAN4 requirements.

Based on the long discussion history in RAN4 concerning gaps and the fact that specification is still ongoing we believe the best approach in RAN4 is to continue the baseline CA approach of having minimum requirements concerning UE interrupts also for LAA SCells.

Additionally, as RAN4 should make specifications mandating specific UE implementations it cannot be assumed that LAA support will always be implemented on a different chip. LAA may also be supported in 3.5 GHz band besides 5 GHz band in the future. Thus, even though interruptions would not be seen necessary and specified within Rel-13 LAA implementation, they might very likely be needed in the future.

Therefore we see that it is better to introduce allowance of interrupts also for LAA SCells and support this in the specification as early as possible. Therefore interruptions should be specified for LAA SCells already at this point.

Proposal 1: Interruption requirements should be defined for LAA SCells within Rel-13 timeframe.

There should not be any significant differences in the actions done by the UE during the activation of LAA SCell in comparison to an SCell in licensed spectrum. Interruption time in SCell in unlicensed spectrum should hence be comparable to SCells in licensed spectrum. These actions are not dependent on functionalities that are characteristic for operation in unlicensed band. Therefore Rel-12 interruption can be reused as baseline when introducing interruption requirements for LAA.

Proposal 2: Rel-12 interruption can be reused baseline when introducing interrupt requirements for LAA SCell in unlicensed spectrum.

# Conclusions

In this contribution we have proposed the following about interruptions with carrier aggregation when introducing LAA SCells:

Proposal 1: Interruption requirements should be defined for LAA SCells within Rel-13 timeframe.

Proposal 2: Rel-12 interruption can be reused baseline when introducing interrupt requirements for LAA SCell in unlicensed spectrum.

# References

[1] R4-155131: Way forward on RRM impact on LAA; Huawei, HiSilicon, Ericsson, Alcatel-Lucent, Intel, Nokia Networks; 3GPP TSG-RAN WG4 Meeting #76, Beijing, China, 24 – 28 August, 2015.