3GPP TSG RAN4 Meeting #76bis	R4-156330

Sophia Antipolis, France, 12th – 16th Sep, 2015

Title: 	MSD for CA_1A-40A

Source: 	Huawei, HiSilicon

Agenda item:	7.43.2.2

Document for:	Discussion

# 1	Introduction

This contribution provides our MSD analysis for CA_1A-40A.

# 2	Discussion

CA_1A-40A is somewhat a tricky combination because of the frequency arrangement. From 3GPP perspective there are basically two possible technologies to combine these bands; a triplexer based technology and a diplexer based technology. Our analysis is based on triplexer. As a reference contribution [1] has evaluated also diplexer based solution

and found that the MSD values are quite similar with triplexer and diplexer based solutions.

Figure 1 Reference architecture

2.1 Component parameters

Table1 and table2 contain the triplexer and RX filter parameters used in this study.

Table 1 Triplexer parameters

| Parameter |  | Component A [TYP/WORST] |  | Component B [TYP/WORST] |  | Component C [TYP/WORST] |  | Average [TYP/WORST] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IL | B1 TX | 2.3/2.6 |  | 1.9/2.6 |  | 2.3/2.7 |  | 2.3/2.6 |
|  | B1 RX | 2.1/2.7 |  | 2.3/3.0 |  | 2.4/3.0 |  | 2.3/2.8 |
|  | B40 TRX | 3.2/3.6 |  | 3.3/3.7 |  | 3.1/3.7 |  | 3.1/3.7 |
| X-band ISO | B1=>B40 | B1TX | 46/42 | B1TX | 46/43 | B1TX | 60/56 | 50.5/47 |
|  |  | B40RX | 46/42 | B40RX | 55/50 | B40RX | 55/51 | 52/47.5 |
|  | B40=>B1 | B40TX | 50/46 | B40TX | 39/34 | B40TX | 60/56 | 49.5/45.5 |
|  |  | B1RX | 42/38 | B1RX | 51/45 | B1RX | 52/48 | 48.5/43.5 |

Table 2 RX filter parameters

| Parameter |  | Component A [TYP/WORST] | Component B [TYP/WORST] | Component C [TYP/WORST] | AVERAGE [TYP/WORST] |
| --- | --- | --- | --- | --- | --- |
| B40 filter | IL | 1.8/3.2 | 1.5/3.2 | 2.1/3.7 | 1.8/3.4 |
|  | B1 TX attenuation | 32/28 | 44/35 | 44/40 | 40/34.5 |
|  | B1 RX attenuation | 32/28 | 47/40 | 44/40 | 41/36 |
| B1 receive filter | IL | 1.6/2.0 | 1.4/2.0 | 2.0/2.4 | 1.7/2.1 |
|  | B40 attenuation | 35/31 | 32/28 | 38/30 | 35/29.5 |

Table 3 contains general RF parameters used in this study. Please note that B40 has two switches in the main branch signal path and only one switch in the diversity path.

Table 3 Other parameters

|  | B40 | B1 |
| --- | --- | --- |
| Antenna ISO | 10 |  |
| RX IIP2 | 45 | 45 |
| PA noise | -125 | -130 |
| RX/TX LO Phase noise | -150 | -150 |
| Switch loss (div) | 1.2 (0.7) | 0.7 |
| Trace loss | 1.0 | 1.0 |
| LB+HB diplexer IL | 1.2 | 1.2 |

Table 4 contains our MSD estimates based on the parameters shown in tables above.

Table 4 MSD estimates

|  | PRX+DRX noise after MRC |  | MSD |  |
| --- | --- | --- | --- | --- |
| RX BW [MHz] | B40 | B1 | B40 | B1 |
| 5 | -94.1 | -88.3 | 3.9 | 10.7 |
| 10 | -92.9 | -87.5 | 3.1 | 8.5 |
| 15 | -91.4 | -86.9 | 2.8 | 7.3 |
| 20 | -90.3 | -86.3 | 2.7 | 6.7 |

# 3	Conclusion

MSD for CA_1A-40A was analyzed with the following results.

|  | MSD |  |
| --- | --- | --- |
| RX BW [MHz] | B40 | B1 |
| 5 | 3.9 | 10.7 |
| 10 | 3.1 | 8.5 |
| 15 | 2.8 | 7.3 |
| 20 | 2.7 | 6.7 |

# 6	References

[1] R4-153965, “MSD estimation on CA_1A_40A”, Qualcomm Inc.