3GPP TSG-RAN WG4 (Radio) Meeting #76bis	R4-156396

Sophia Antipolis, France, 12 – 16 October 2015

Agenda Item:	7.29.2

Source: 	Alcatel-Lucent, TeliaSonera, Nokia Networks

Title: 	TP for TR 36.895: Updates of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (20 + 700SDL)

Document for:	Approval

# 1.	Introduction

The WI proposal to support LTE Advanced 2 Band Carrier Aggregation (2DL/1UL) of Band 20 and Band xy (new band European 700SDL) was approved in RAN#68 [1]. One objective of the WI is to specify the band-combinations specific Radio Frequency (RF) requirements for this new Carrier Aggregation (CA) band combination.

The impact of Harmonics and InterModulation Distortion (IMD) products caused by LTE Advanced Base Station (BS) supporting carrier aggregation of this new bandwidth combination to the receiver of own or different BS was investigated in [2]. In this paper, we provide a text proposal to update the coexistence studies in the European 700 Supplemental Downlink band Technical Report [3].

# 2.	Text Proposal

<Start of change>

## 8.1 	Co-existence studies

Table 8.1-1 shows the second and third order DL harmonics and intermodulation products when two simultaneous DLs are active in Band 20 and 700MHz SDL.

Table 8.1-1: Band 20 and Band 700MHz SDL DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 738 | 758 | 791 | 821 |
| 2nd order harmonics frequency range (MHz) | 1476 | 1516 | 1582 | 1642 |
| 3rd order harmonics frequency range (MHz) | 2214 | 2274 | 2373 | 2463 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | (f2_low + f1_low) | (f2_high + f1_high) |
| IMD frequency limits (MHz) | 33 | 83 | 1529 | 1579 |
| 3rd order IMD products | (2*f1_low – f2_high) | (2*f1_high – f2_low) | (2*f2_low – f1_high) | (2*f2_high – f1_low) |
| IMD frequency limits (MHz) | 655 | 725 | 824 | 904 |
| 3rd order IMD products | (2*f1_low + f2_low) | (2*f1_high + f2_high) | (2*f2_low + f1_low) | (2*f2_high + f1_high) |
| IMD frequency limits (MHz) | 2267 | 2337 | 2320 | 2400 |
| 3rd order IMD products | (f1_low – f2_high + f2_low) | (f1_high + f2_high – f2_low) | (f2_low – f1_high + f1_low) | (f2_high + f1_high – f1_low) |
| IMD frequency limits (MHz) | 708 | 788 | 771 | 841 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1_low – f2_BWmax) | (f1_high + f2_BWmax) | (f2_low – f1_BWmax) | (f2_high + f1_BWmax) |
| IMD frequency limits (MHz) | 718 | 778 | 771 | 841 |

It can be seen from Table 8.1-1 that 2nd harmonics caused by BS supporting carrier aggregation of Band 20 and Band 700MHz SDL may fall into the BS receive band of Band 24, while 3rd harmonic will fall into the BS receive band of Band 40.

It can be seen from Table 8.1-1 that none 2nd IMD products caused by BS supporting carrier aggregation of Band 20 and 700MHz SDL will fall into the BS receive band of currently defined 3GPP bands, while 3rd IMD products may fall into the BS receive band of Bands 5, 6, 8, 12-14, 17-20, 26-28, 30, 40 and 44. Bands 5, 6, 12-14, 17-19, 26-27, 30, 40 and 44 are not intended for use in the same geographical area as Bands 20 and 700MHz SDL.

The analysis below gives further information on the actual channel bandwidths and carrier positions for that CA combinations which can cause DESENSE to the BS own receive blocks.

Assume that the 2 CC transmitted by the Band (20 + 700SDL) BS are a and b MHz channel bandwidth and c and d MHz from the edges of Bands 20 and 700SDL DL frequency bands as shown in Figure 8.1-1 below:

f2-high

f2-low

f1-high

f1-low

1st CC

2nd CC

738

821

c MHz

d MHz

a MHz

b MHz

MHz

Figure 8.1-1: CC transmitted by the Band (20 + 700SDL) BS

And the corresponding BS receive block are as shown in Figure 8.1-2 below:

f2-low

f2-high

UL CC

832

862

d MHz

b MHz

MHz

Figure 8.1-2: CC received by the Band 20 BS

Then the 3rd order IMD products caused in the BS by transmitting of the 2 CC can be calculated as shown in Table 8.1-2 below:

Table 8.1-2: Band (20 + 700SDL) BS DL 3rd order IMD products

| 3rd order IMD products | IMD frequency limits (MHz) |
| --- | --- |
| (2*f1-low – f2-high) | 655 + 2*c + d |
| (2*f1-high – f2-low) | 655 + 2*c + d + 2*a + b |
| (2*f2-low – f1-high) | 904 – 2*d – c – 2*b – a |
| (2*f2-high – f1-low) | 904 – 2*d – c |
| (f1-low – f2-high + f2-low) | 738 + c – b |
| (f1-high + f2-high – f2-low) | 738 + c + b + a |
| (f2-low – f1-high + f1-low) | 821 – d – a – b |
| (f2-high + f1-high – f1-low) | 821 – d + a |

Comparing the IMD frequency limits in Table 8.1-2 with the BS receive blocks in Figure 8.1-2, it can be deduced that the 3rd order IMD products caused in the BS by transmitting of the 2 CC will only fall into the BS own receive block of Band 20 under one of the transmit configurations shown in Table 8.1-3 below:

Table 8.1-3: Band (20 + 700SDL) BS transmit configurations with 3rd IMD falling within Band 20 BS own receive blocks

| 1st CC channel bandwidth (a MHz) | 2nd CC channel bandwidth (b MHz) | 1st and 2nd CC position (c and d MHz) |
| --- | --- | --- |
| 5 | 5 | 27 < c + d <= 40 |
| 5 | 10 | 17 < c + d <= 35 |
| 10 | 5 | 22 < c + d <= 35 |
| 5 | 15 | 7 < c + d <= 30 |
| 10 | 10 | 12 < c + d <= 30 |
| 15 | 5 | 17 < c + d <= 30 |
| 5 | 20 | c + d <= 25 |
| 10 | 15 | 2 < c + d <= 25 |
| 15 | 10 | 7 < c + d <= 25 |
| 20 | 5 | 12 < c + d <= 25 |
| 10 | 20 | c + d <= 20 |
| 15 | 15 | c + d <= 20 |
| 20 | 10 | 2 < c + d <= 20 |
| 15 | 20 | c + d <= 15 |
| 20 | 15 | c + d <= 15 |
| 20 | 20 | c + d <= 10 |

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Bands 8, 20 and 28 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 20 + 700MHz SDL BS transmitter does not share the same antenna with Bands 8, 20 and 28 BS receiver.

Therefore, it is recommended that Bands 20 + 700MHz SDL BS transmitter should not share the same antenna with Bands 8, 20 and 28 BS receiver to prevent BS receiver desensitization, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 8, 20 and 28 BS receiver desensitization. PIM may be also caused outside the antenna system by e.g. metal objects close to the antennas and cause possible DESENSE into the own receiver, see TR 37.808 Section 6.1.

Table 8.1-4 gives the harmonic products for Bands 20. None of the harmonic products fall into the own UE receive bands.

Table 8.1-4: B20 harmonic products

| UE UL carriers | f2_low | f2_high |
| --- | --- | --- |
| UL frequency (MHz) | 832 | 862 |
| 2nd order harmonics frequency range (MHz) | 1664 to 1724 |  |
| 3rd order harmonics frequency range (MHz) | 2496 to 2586 |  |

<End of change>

# References

[1]	RP-150861, “New WI proposal: LTE Advanced 2 Band Carrier Aggregation (2DL/1UL) of Band 20 and Band xy (new band European 700SDL)”, TeliaSonera.

[2]	R4-155077, “	TP to TR 36.895: Co-existence studies for LTE Advanced inter-band Carrier Aggregation of Band 20 and 700MHz SDL (738-758MHz)”, Nokia Networks.

[3]	R4-155897, “TR 36.895 V0.2.0 European 700 Supplemental Downlink band (738-758 MHz) in E-UTRA”, TeliaSonera.