3GPP TSG-RAN WG4 (Radio) Meeting #76bis	R4-156846

Sophia Antipolis, France, 12 – 16 October 2015

Agenda Item:	9.3

Source: 	Sprint, Alcatel-Lucent

Title: 	Way Forward on System level simulation methodology and assumptions for coexistence study on new Band 41 UE power class supporting +26 dBm

Document for:	Approval

# Introduction

At RAN#76 and again at RAN#76bis several companies submitted contributions on simulation methodology and assumptions [5][6][7][8][9][10]. This Way Forward proposes a common set of parameters for companies to use in simulations.

# Discussion

All of the simulation assumptions are based on TS 36.942 [3].  Table 1 summarizes the simulation assumptions contributions submitted the RAN4#76bis.

| Company | ISD (km) | Antennas (dBi) | CLx-ile Values | Propagation Model | Other Features | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| ALU [9] | 1.0 1.5 2.8 6.3 (rural) 8.0 (rural) | BS: 17 23dBm UE: -3.5 26dBm UE: -3.5 | 117,120 123,126 133,136 118,121 122,125 | Urban & Suburban - 3GPP Macro cell Urban Hata (TR 25.942) Rural - Modified Hata Model Case 1: d 0.04 km Case 2: d 0.1 km Case 1: d 0.6 km (8) Case 2: d  0.6 km |  | Offset CLx-ile values - high power will primarily be used at cell edge. |
| Ericsson [8] | 6 (rural) 8 (rural) | BS: 15 23dBm UE: -6 26dBm UE: -1 | 117 122 | Rural - Modified Hata Model Case 1: d 0.6 km Case 2: d  0.6 km |  | Single CLx-ile values - higher HPUE power in inner cell Different antennas for UE classes |
| CMCC [10] | 6.0 (rural) | BS: 15 23dBm UE: 0 26dBm UE: 0 | 117 (10MHz) | TS36942 Okumura-Hata L (R)= 69.55 +26.16log10(f)–13.82log10(Hb)+[44.9-6.55log10(Hb)]log(R) – 4.78(Log10 (f))2+18.33 log10 (f) -40.94 where: R is the base station-UE separation in kilometres f is the carrier frequency in MHz Hb is the base station antenna height above ground in metres 101.95+34.01log(R) (values for F and Hb not shown in contribution) F = 2600 MHz Hb = 45 meters | Variable percentage of HPUEs in population | Single CLx-ile values - higher HPUE power in inner cell |
| CTC [5] | 0.75 1.7 5.0 (rural) | BS: 15 23dBm UE: 0 26dBm UE: 0 | 109 | Urban & Suburban - 3GPP Macro cell Urban Hata (TR 25.942) Rural - Modified Hata Model Case 1: d 0.04 km Case 2: d 0.1 km | Indoor Penetration | Single CLx-ile values - higher HPUE power in inner cell |

#### 22

# Way Forward

Proposal 1: Adopt the simulation assumptions found in Section 3.1 through 3.6

## 3.1	Cell layouts

Base stations with 3 sectors per site are placed on a hexagonal grid with distance of 3*R, where R is the cell radius (see Figure 3.1), with wrap around. The number of sites shall be equal to or higher than 19 [3]. Uncoordinated macro cellular deployment is assumed, where interfering UE may be at cell edge of the serving base station but close to the victim base station (hence transmitting with highest power and causing highest interference).

Figure 3.1: Uncoordinated macro cellular deployment

The inter-site distances considered in this study are provided in Table 2.1 below.

Table 3.1: Inter-site distances and Propagation model

| Environment | ISD (KM) | ISD (miles) |
| --- | --- | --- |
| Urban | .75 | .47 |
| Suburban | 2.8 | 1.74 |
| Rural | 6 | 3.73 |
| Rural | 8 | 5 |

## Macro cell Propagation model - Urban and Suburban Areas

The propagation model is a derived from TS 36.942

Considering a carrier frequency of 2.6 GHz and a base station antenna height of 15 m above average rooftop level, the propagation model is given by the following equation:

(2)

where:

R is the base station-UE separation in kilometres

## Macro cell Propagation model - Rural Area

The propagation model is a derived from TS 36.942

For rural area, the Hata model in [3] is not applicable for a carrier frequency of 2.6 GHz, while the modified Hata model can be used [5]:

Case 1:		d  0.6 km

Case 2:		d  0.6 km

where: d is the base station-UE separation in kilometres

## Power control modelling

In TR 36.942, the following power control equation is used for the uplink coexistence simulations:

(10)

where Pmax is the maximum transmit power, Rmin is the minimum power reduction ratio to prevent UEs with good channels to transmit at very low power level, CL is the coupling loss defined as max{path loss-G_Tx-G_Rx, MCL}, where path loss is propagation loss plus shadowfading, G_TX is the transmitter antenna gain in the direction of the receiver, G_RX is the receiver antenna gain in the direction of the transmitter and CLx-ile is the x-percentile CL value. With this power control equation, the x percent of UEs that have the highest coupling loss will transmit at Pmax. Finally, 0<<=1 is the balancing factor for UEs with bad channel and UEs with good channel. For 26 dBm UE, it is assumed that Rmin has 3dB more dynamic range.

The power control parameters were specified in TS 36.942 for carrier frequency of 2 GHz and 500 m cell range, these have been adjusted for different ISD’s and UE power classes.

Table 3.4.1: Modified power control algorithm parameters for +23 dBm UE

(a) For 0.75 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 109 | 112 |
| Set 1’ | 1 | 117 | 120 |
| Set 2 | 0,8 | 133 | 137 |

(b) For 2.8 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 133 | 136 |
| Set 2 | 0,8 | 149 | 153 |

(c) For 6 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 133 | 136 |
| Set 2 | 0,8 | 149 | 153 |

(d) For 8 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 133 | 136 |
| Set 2 | 0,8 | 149 | 153 |

Table 3.4.2: Modified power control algorithm parameters for +26 dBm UE

(a) For 0.75 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 109 | 112 |
| Set 1’ | 1 | 117 | 120 |
| Set 2 | 0,8 | 133 | 137 |

(b) For 2.8 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 133 | 136 |
| Set 2 | 0,8 | 149 | 153 |

(c) For 6 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 136 | 139 |
| Set 2 | 0,8 | 153 | 157 |

(d) For 8 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 136 | 139 |
| Set 2 | 0,8 | 153 | 157 |

## Other simulation assumptions

Other simulation assumptions are summarized in Table 3.5 below:

Table 3.5: Simulation parameters for Band 41 system

(a) With 23 dBm UE

|  | Base Station | UE |
| --- | --- | --- |
| Carrier frequency | 2600 MHz |  |
| Channel bandwidth | 20 MHz, 10 MHz |  |
| Inter-site distance | Use Table 3.1 |  |
| Cell layout | Wrap-around 19 tri-sector cells, uncoordinated |  |
| Frequency reuse | 1x3x1 |  |
| Lognormal fading | 10 dB |  |
| Shadowing correlation | Between cells: 0.5, between sites: 1.0 |  |
| MCL (including antenna gain) | 70 dB (urban and suburban areas) 80 dB (rural area) |  |
| Antenna gain and horizontal antenna pattern | 17 dBi, = 65 degrees, Am = 20 dB | Omni-directional antenna with 0 dBi. |
| Noise figure | 5 dB | 9 dB |
| Transmit power | 46 dBm | 23 dBm |
| Antenna height | 45 m | 1.5 m |
| ACLR | 45 dB | Use Table 5.2 in TR 36.942 ACLR1: 30+X, ACLR2: 43+X Where X is 1 dB |
| ACS | 45 dB | 27 dB |

(b) With 26 dBm UE

|  | Base Station |  | HPUE |
| --- | --- | --- | --- |
| Carrier frequency | 2600 MHz |  |  |
| Channel bandwidth | 20MHz, 10 MHz |  |  |
| Inter-site distance | Use Table 2.1 |  |  |
| Cell layout | Wrap-around 19 tri-sector cells, uncoordinated |  |  |
| Frequency reuse | 1x3x1 |  |  |
| Lognormal fading | 10 dB |  |  |
| Shadowing correlation |  | Between cells: 0.5, between sites: 1.0 |  |
| MCL (including antenna gain) |  | 70 dB (urban and suburban areas) 80 dB (rural area) |  |
| Antenna gain and horizontal antenna pattern | 17 dBi, = 65 degrees, Am = 20 dB |  | Omni-directional antenna with 0 dBi. |
| Noise figure | 5 dB |  | 9 dB |
| Transmit power | 46 dBm |  | 26 dBm |
| Antenna height | 45 m |  | 1.5 m |
| ACLR | 45 dB |  | Use Table 5.2 in TR 36.942 ACLR1: 30+X, ACLR2: 43+X Where X is 1 dB |
| ACS | 45 dB |  | 27 dB |

- Simulations should assume the worst case of 100% HPUEs in the scenarios with HPUEs.

## Simulation procedure

For the co-existence study, the following procedure shall be performed:

1)	Run the Band 41 UL to UL coexistence study, assuming parameters of both systems are according to Table 3.5(a). Power control parameters in Table 3.4.1 are used. This corresponds to the coexistence of two commercial networks operating in adjacent channel and with similar deployment parameters. This is used as the reference. Band 41victim system performance degradation results in this scenario are used as the baseline. 

Provide a CDF plot of UE transmit power.

2)	Run the Band 41 UL to UL coexistence study, assuming +26 dBm power class UE is deployed in Band 41 interfering system only, and obtain the victim system performance degradation results. The simulation parameters in Tables 3.5(a) and 3.5(b) are used for the victim and interfering system, respectively. And the power control parameters in Tables 3.4.1 and 3.4.2 are used for the victim and interfering system, respectively. 

Provide a CDF plot of UE transmit power.

3)	Compare the Band 41 victim system performance degradation obtaining in steps 1) and 2), choose the 26 dBm UE ACLR value so that the victim system performance degradation due to 26 dBm UE in 2) is the same as 1).

# 4.	Text Proposal

Proposal 2 : Approve the following text proposal adding the simulation assumptions in TR36.866

TEXT PROPOSAL:

<start of text proposal >

## 5.6.1 	Simulation assumptions

### 5.6.1.1	Macro cell Propagation model

### 5.6.1.1.1 Macro cell Propagation model - Urban and Suburban Areas

The propagation model is a derived from TS 36.942

Considering a carrier frequency of 2.6 GHz and a base station antenna height of 15 m above average rooftop level, the propagation model is given by the following equation:

(2)

where:

R is the base station-UE separation in kilometres

### 5.6.1.1.2 Macro cell Propagation model - Rural Area

The propagation model is a derived from TS 36.942

For rural area, the Hata model in [3] is not applicable for a carrier frequency of 2.6 GHz, while the modified Hata model can be used [5]:

Case 1:		d  0.6 km

Case 2:		d  0.6 km

where: d is the base station-UE separation in kilometres

### 5.6.1.2	Power Control Modelling

In TR 36.942, the following power control equation is used for the uplink coexistence simulations:

(10)

where Pmax is the maximum transmit power, Rmin is the minimum power reduction ratio to prevent UEs with good channels to transmit at very low power level, CL is the coupling loss defined as max{path loss-G_Tx-G_Rx, MCL}, where path loss is propagation loss plus shadowfading, G_TX is the transmitter antenna gain in the direction of the receiver, G_RX is the receiver antenna gain in the direction of the transmitter and CLx-ile is the x-percentile CL value. With this power control equation, the x percent of UEs that have the highest coupling loss will transmit at Pmax. Finally, 0<<=1 is the balancing factor for UEs with bad channel and UEs with good channel. For 26 dBm UE, it is assumed that Rmin has 3dB more dynamic range.

The power control parameters were specified in TS 36.942 for carrier frequency of 2 GHz and 500 m cell range, these have been adjusted for different ISD’s and UE power classes.

Table 5.6.1.2-1: Modified power control algorithm parameters for +23 dBm

(a) For 0.75 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 109 | 112 |
| Set 1’ | 1 | 117 | 120 |
| Set 2 | 0,8 | 133 | 137 |

(b) For 2.8 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 133 | 136 |
| Set 2 | 0,8 | 149 | 153 |

(c) For 6 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 133 | 136 |
| Set 2 | 0,8 | 149 | 153 |

(d) For 8 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 133 | 136 |
| Set 2 | 0,8 | 149 | 153 |

Table 5.6.1.2-2: Modified power control algorithm parameters for +26 dBm

(a) For 0.75 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 109 | 112 |
| Set 1’ | 1 | 117 | 120 |
| Set 2 | 0,8 | 133 | 137 |

(b) For 2.8 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 133 | 136 |
| Set 2 | 0,8 | 149 | 153 |

(c) For 6 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 136 | 139 |
| Set 2 | 0,8 | 152 | 156 |

(d) For 8 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 136 | 139 |
| Set 2 | 0,8 | 152 | 156 |

### 5.6.1.3 	Cell Layout

Base stations with 3 sectors per site are placed on a hexagonal grid with distance of 3*R, where R is the cell radius (see Figure 5.6.1.3-1), with wrap around. The number of sites shall be equal to or higher than 19 [3]. Uncoordinated macro cellular deployment is assumed, where interfering UE may be at cell edge of the serving base station but close to the victim base station (hence transmitting with highest power and causing highest interference).

Figure 5.6.1.3-1: Uncoordinated macro cellular deployment

The inter-site distances considered in this study are provided in Table 3.1 below.

Table 5.6.1.3-1: Inter-site distances and Propagation model

| Environment | ISD (KM) | ISD (miles) |
| --- | --- | --- |
| Urban | .75 | .47 |
| Suburban | 2.8 | 1.74 |
| Rural | 6 | 3.73 |
| Rural | 8 | 5 |

### 5.6.1.4	Other Simulation Assumptions

Other simulation assumptions are summarized in Table 5.6.1.4-1 below:

Table 5.6.1.4-1: Simulation parameters for Band 41 system

(a) With 23 dBm UE

|  | Base Station | UE |
| --- | --- | --- |
| Carrier frequency | 2600 MHz |  |
| Channel bandwidth | 20 MHz, 10 MHz |  |
| Inter-site distance | Use Table 3.1 |  |
| Cell layout | Wrap-around 19 tri-sector cells, uncoordinated |  |
| Frequency reuse | 1x3x1 |  |
| Lognormal fading | 10 dB |  |
| Shadowing correlation | Between cells: 0.5, between sites: 1.0 |  |
| MCL (including antenna gain) | 70 dB (urban and suburban areas) 80 dB (rural area) |  |
| Antenna gain and horizontal antenna pattern | 17 dBi, = 65 degrees, Am = 20 dB | Omni-directional antenna with 0 dBi. |
| Noise figure | 5 dB | 9 dB |
| Transmit power | 46 dBm | 23 dBm |
| Antenna height | 45 m | 1.5 m |
| ACLR | 45 dB | Use Table 5.2 in TR 36.942 ACLR1: 30+X, ACLR2: 43+X Where X is 1 dB |
| ACS | 45 dB | 27 dB |

(b) With 26 dBm UE

|  | Base Station |  | HPUE |
| --- | --- | --- | --- |
| Carrier frequency | 2600 MHz |  |  |
| Channel bandwidth | 10 MHz |  |  |
| Inter-site distance | Use Table 2.1 |  |  |
| Cell layout | Wrap-around 19 tri-sector cells, uncoordinated |  |  |
| Frequency reuse | 1x3x1 |  |  |
| Lognormal fading | 10 dB |  |  |
| Shadowing correlation |  | Between cells: 0.5, between sites: 1.0 |  |
| MCL (including antenna gain) |  | 70 dB (urban and suburban areas) 80 dB (rural area) |  |
| Antenna gain and horizontal antenna pattern | 17 dBi, = 65 degrees, Am = 20 dB |  | Omni-directional antenna with 0 dBi. |
| Noise figure | 5 dB |  | 9 dB |
| Transmit power | 46 dBm |  | 26 dBm |
| Antenna height | 45 m |  | 1.5 m |
| ACLR | 45 dB |  | Use Table 5.2 in TR 36.942 ACLR1: 30+X, ACLR2: 43+X Where X is 1 dB |
| ACS | 45 dB |  | 27 dB |

- Simulations should assume the worst case of 100% HPUEs in the scenarios with HPUEs.

### 5.6.1.5	Simulation Procedure

For the co-existence study, the following procedure shall be performed:

1)	Run the Band 41 UL to UL coexistence study, assuming parameters of both systems are according to section 5.6.1.4. Power control parameters in section 5.6.1.2 are used. This corresponds to the coexistence of two commercial networks operating in adjacent channel and with similar deployment parameters. This is used as the reference. Band 41victim system performance degradation results in this scenario are used as the baseline.

Provide a CDF plot of UE transmit power.

2)	Run the Band 41 UL to UL coexistence study, assuming +26 dBm power class UE is deployed in Band 41 interfering system only, and obtain the victim system performance degradation results. The simulation parameters in Tables 5.6.1.4-1 (a) and 5.6.1.4-1 (b) are used for the victim and interfering system, respectively. And the power control parameters in section 5.6.1.2 are used for the victim and interfering system, respectively.

Provide a CDF plot of UE transmit power.

3)	Compare the Band 41 victim system performance degradation obtaining in steps 1) and 2), choose the 26 dBm UE ACLR value so that the victim system performance degradation due to 26 dBm UE in 2) is the same as 1).

<End of text proposal >

# 5.	Proposals

# Proposal 1: Adopt the simulation assumptions found in Section 3.1 through 3.6

Proposal 2 : Approve the text proposal in section 4 adding the simulation assumptions in TR36.866

# 6	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	R4-060470 LTE UE power class

[3]	3GPP TR 36.942 v13.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios”.

[4]	Report ITU-R SM.2028-1, “Monte Carlo simulation methodology for the use in sharing and compatibility studies between different radio services or system.

[5] 	R4-155446, ”Discussion on coexistence study for B41 HPUE”, 	China Telecom

[6]	R4-155995, “Discussion co-existence requirement for HPUE in B41”, 	CMCC

[7]	R4-156402, “System level simulation methodology and assumptions for coexistence study on new Band 41 UE power class supporting +26 dBm”, 	Alcatel-Lucent

[8] 	R4-156495, “TP to TR ab.cde: Parameters for Band 41 HPUE simulations”, 	Ericsson

[9] 	R4-154000, “System level simulation methodology and assumptions for coexistence study on new Band 41 UE power class supporting +26 dBm“,Alcatel-Lucent

[10] 	R4-154501, ”Discussion on ACLR requirements for Band 41 UE power class supporting +26 dBm”, CMCC