3GPP TSG RAN WG4 Meeting #76bis	R4-156884

Sophia Antipolis, France, October 14 – 16, 2015

Agenda Item:	9.5.1

Source: 	Ericsson, MediaTek

Title:	TP Unidirectional RRH arrangement

Document for:	pCR (Approval)

# Introduction

This contribution contains a text proposal for TR 36.838 [1] on inclusion of the Unidirectional RRH arrangement for high-speed train single frequency network scenarios. The text proposal follows in the section below.

# Text Proposal for 36.878

< Added section below >

### 6.53.3 Unidirectional SFN scenario (RRHs sharing the same cell id, beams aligned in same direction)

In this section the channel model for the Unidirectional RRH arrangement for SFN is described in terms of time-variable Doppler shifts, tap delays and relative received power levels. The channel model is based on a 3-tap propagation model, see Figure 6.2.3.3-1. For this model, DL TX and UL RX beams, respectively, are aligned in the same direction along the high-speed train track, and for the baseline configuration the DLTX and ULRX beams are oriented in the same direction along the track.

The objective with the unidirectional RRH arrangement is to provide a stable downlink carrier frequency as experienced by the UE when travelling at high speed. This is achieved by arranging the RRHs in such manner that the strongest signal received by the UE has a nearly constant Doppler shift without sign-alternation. A stable downlink frequency as experienced by the UE leads to that uplink transmissions from the same UE are received by the RRHs with a nearly constant frequency offset. Additionally, all UEs travelling onboard the same train share the same Doppler and frequency offset characteristics.

The Unidirectional SFN scenario is foreseen to serve both SI Objectives:

- Objective 1 (support of UE speed up to 350km/h), and

- Objective 2 (support of UE speed above 350km/h).

Figure 6.5.3.3-1: Unidirectional SFN scenario in baseline configuration, using a 3-tap channel.

The Unidirectional SFN scenario is based on network deployment where directional antennas are used and where it therefore can be controlled at which point a UE leaves one beam and enters the next. It is essential that side-lobes directed towards the track are received at lower power than the signal from the main-lobe. Hence the antenna characteristics and particularly the antenna radiation pattern is of importance, and is a limiting factor for which geometries with respect to inter-site distance DS and minimum track-to-RRH distance Dmin that can be supported. For the evaluation of the Unidirectional SFN scenario an exemplary antenna with FBR and FSR of somewhat more than 30dB has been selected, although antennas with lower FBR and/or FSR can be used provided that DS and Dmin are chosen accordingly. The antenna radiation pattern of the exemplary antenna is shown in Figure 6.2.3.3-2 and described further below.

Figure 6.5.3.3-2: Assumed antenna radiation pattern for RRH DL TX and UL RX antennas in Unidirectional SFN scenario.

The principle of Unidirectional SFN is further illustrated in Figures 6.5.3.3-2b and 6.5.3.3-2c. Figure 6.5.3.3-2b shows coverage of the antenna models for RRHs positioned along a track. Figure 6.5.3.3-2c zooms in on the area where coverage overlaps for adjacent RRHs, and it therefore is important to avoid that the UE receives side-lobes from one RRH at higher power level than it receives the main-lobe from another RRH.

Figure 6.5.3.3-2b: Illustration of coverage provided by consecutive RRHs in a unidirectional SFN scenario.

Figure 6.5.3.3-2c: Illustration of coverage overlap of adjacent RRHs. Geometries needs to be selected in such manner that the UE receives side-lobes at lower power level than it receives the main-lobe.

The signal received by the UE for path i can be modelled as follows:

yit=ej2παi∙pit∙ej2π(fct+0tddτfi(τ)dτ)∙sbt-dit

where αi is a semi-static channel phase,  pit is the combined effect of beamforming and path-loss, fc is the nominal carrier frequency, fi(t) is the instantaneous Doppler shift, dit is the delay due to propagation and deliberate delay of the transmitted signal, and where sbt is the baseband signal transmitted in the SFN cell. The parameters are described below.

- Doppler shift

The RRHs are lined up along the railway track at equal distances to the track and identical inter-site distances. Path 0 denotes the path from the previously passed RRH, path 1 the path from the just recently passed RRH, path 2 the path from the closest approaching RRH; see Figure 6.5.3.3-1.

Doppler shifts for the paths i=0..2, are given by

fit=-fdcosθit⁡ 																(6.5.3.3-1)

in which  is the maximum Doppler frequency. The cosine angles θit are given by

cosθit=Ds+vtDmin2+Ds+vt2,  i=0,  0<t≤Ds/vvtDmin2+vt2,      i=1,      0<t≤Ds/v -Ds-vtDmin2+Ds-vt2,      i=2,     0<t≤Ds/v cosθit modDsv,  ∀i,     t>Ds/v         						(6.5.3.3-2)

where Ds is the inter-site distance in meters between two RRHs; Dmin is the shortest distance in meters between the RRH and the railway track; v is the velocity of the train in m/s; and t is the time in seconds. Ds/v

Doppler shifts and cosine angles are given by equations 6.5.3.3-1 and 6.5.3.3-2, where the required parameters are listed in Tables 6.5.3.3-1, 6.5.3.3-2, 6.5.3.3-3 and 6.5.3.3-4, respectively.

Table 6.5.3.3-1: Parameters for Unidirectional SFN scenarios

| Parameter | Value |
| --- | --- |
| Ds | [1000] m |
| Dmin | [30] m |
| v | 350 km/h |
| fd | 875 Hz |
| γ | 5º |
| Δ | 3.3µs |

Table 6.5.3.3-2: Parameters for Unidirectional SFN scenarios

| Parameter | Value |
| --- | --- |
| Ds | [500] m |
| Dmin | [15] m |
| v | 350 km/h |
| fd | 1250 Hz |
| γ | 5º |
| Δ | 1.7µs |

Table 6.5.3.3-3: Parameters for Unidirectional SFN scenarios

| Parameter | Value |
| --- | --- |
| Ds | [1000] m |
| Dmin | [30] m |
| v | [500 km/h] |
| fd | 875 Hz |
| γ | 5º |
| Δ | 3.3µs |

Table 6.5.3.3-4: Parameters for Unidirectional SFN scenarios

| Parameter | Value |
| --- | --- |
| Ds | [500] m |
| Dmin | [15] m |
| v | [500 km/h] |
| fd | 1250 Hz |
| γ | 5º |
| Δ | 1.7µs |

Figure 6.5.3.3-3: Normalized Doppler shift trajectory for Unidirectional SFN scenarios in Tables 6.2.3.3-1 to 6.2.3.3-4. Note1: The signs are incorrect – to be updated. Note2: The curve for θ3 is to be ignored.

- Relative power

The signal power received by the UE from each respective path is given by

pit=Gφit2∙Dmin2Dmin2+(Ds+vt)2,    i=0,   0<t≤Ds/vDmin2Dmin2+(vt)2,   i=1,       0<t≤Ds/vDmin2Dmin2+(Ds-vt)2,   i=2,  0<t≤Ds/vpit modDsv,  ∀i,  t>Ds/v						(6.5.3.3-3)

where G(φ) is a function describing the RRH beam characteristics (amplitude and phase) at angle φ relative to the main lobe, and φi(t) is said angle for the ith path at time t.

For the purpose of evaluation, the beam is modelled as a linear array with beam shaping weights provided in Table 6.2.3.3-5, by which G(φ) is modelled by a FIR filter. See Figure 6.2.3.3-2 for a sketch of  Gφ2.

Table 6.5.3.3-5: Coefficients for modelling beam shape characteristics

| Array weight k | Coefficient ak |
| --- | --- |
| 0 | 0.019194052903732 |
| 1 | 0.033761601544153 |
| 2 | 0.056016046672771 |
| 3 | 0.080084528366365 |
| 4 | 0.101814080796633 |
| 5 | 0.116978896579721 |
| 6 | 0.123420002605498 |
| 7 | 0.116978896579721 |
| 8 | 0.101814080796633 |
| 9 | 0.080084528366365 |
| 10 | 0.056016046672771 |
| 11 | 0.033761601544153 |
| 12 | 0.019194052903732 |
| Note1: The beam shape is modelled as an Equiripple filter with 0.1π wide passband with 0dB gain, 0.2π wide transition bands, and 1.5π wide stopband with 30dB attenuation. Passband and stopband penalties are weighed 1:10. Note2: The assumed beam shape does not mandate a particular design of RRH antennas; it is selected merely for evaluation of the scenario. |  |

The antenna radiation pattern at direct path angle φ between UE and RRH is calculated

Gφ2=k=0K-1akej2πφ2.									(6.5.3.3-3b)

The angle of the UE to the main beam can be determined from the UE angle θi∈[0,π] to the RRH and orientation γ of the main beam towards the track as illustrated in Figure 6.5.3.3-4b, resulting in φit=θit-γ.

Figure 6.5.3.3-4b: Definition of angles for evaluation of antenna gain for path i.

Figure 652.3.3-4: Path-loss for Unidirectional SFN scenarios in Tables 6.2.3.3-1 to 6.2.3.3-4.

Note: The curve for p3 is to be ignored.

Figure 6.5.3.3-4: Received signal power per path taking into account path-loss and beam-shape. The curves are applicable for Unidirectional SFN scenarios in Tables 6.2.3.3-2 and 6.2.3.3-4. 
Note: The curve for path 3 is to be ignored.

- Tap delay

The time delay of each path is given by

dit=Dmin2+(Ds+vt)2c, i=0, 0<t≤Ds/vDmin2+(vt)2c+Δ, i=1, 0<t≤Ds/vDmin2+(Ds-vt)2c+2Δ,i=2, 0<t≤Ds/vdit modDsv,  ∀i,     t>Ds/v 								(6.5.3.3-4)

where c is the speed of light, and Δ is an introduced successive delay of transmission timing. The purpose of the introduced delay is to secure that a UE, when entering the next beam, finds the timing of that beam within ±0.5CP.

- Tap phase

The phase αi of each path is randomized uniformly within [0,2π] at t=0. Then for each time passing and RRH, path 0 inherits the phase from path 1, path 1 inherits the phase from path 2, and a new phase is randomized for path 2. This scheme is used in order to keep the same channel phase for each respective beam when the UE is passing an RRH.

< End of added section >

# References

- 3GPP TR 36.878 V1.0.1