3GPP TSG RAN WG4 Meeting #80bis	R4-167869

Ljubljana, SI, 10 - 14 Oct 2016

Agenda Item:	6.3.4.2.1

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	Ideal NB-IoT PRACH demodulation performance

Document for:	Discussion

# Introduction

RAN4 has been working on NPRACH performance requirements for a number of meetings (e.g., [1-5]), including NPRACH detection performance and ToA timing estimation performance.  It was observed that significant gap exists between NPRACH detection performance and timing estimation performance [1,2], the cell-ID and signature have significant impact on NPRACH ToA estimation performance [3], total NPRACH error probability (joint probability of missed detection and error timing estimation) is higher than targets and it is channel dependent [4].

In RAN4#80, a WF on NPRACH performance requirements was agreed [5], where it states

The SNR for minimum performance shall be defined to satisfy

- Option 1: Missed detection probability <=1%, including: detecting different preamble than the one that was sent, not detecting a preamble at all, or detecting the correct preamble detection but with the wrong timing estimation.

- Option 2: Pmd<=1%, and Pte<=1%, where Pmd includes: detecting different preamble than the one that was sent, or not detecting a preamble at all.

- Other options are not excluded

Timing limit

- Option 1: 2.5us

- Option 2: 5us

- Other options (such as 3.125us) are not excluded

  - Note: 3.75us is also used in this paper

Repetition number:

- Greater or equal to 8

Cell-ID and signature:

- Fixed

In addition, the NPRACH simulation assumptions were updated as follows:

| Parameter | Values |
| --- | --- |
| NPRACH preamble format | 0 |
| Number of Repetitions | 8, 32 |
| Number of Subcarriers | 12 |
| Antenna configuration | 1 x 2 |
| Cell-ID | 0 |
| NPRACH signature | 0 |
| frequency offset | 0Hz (AWGN); [200Hz] EPA1 |
| Propagation channels | AWGN, EPA1 |
| Detection performance | Missed detection rate (1%) with false alarm rate (0.1%) |
| Timing estimation | Timing error probability with limits in Slide 3 |

In this paper, we provide the ideal simulation results with the updated simulation assumptions, and then further discuss the NPRACH performance requirements based on the agreed WF.

# Simulation Results

Based on the Options listed in the agreed WF (R4-167115), the simulation results may be provided for the following scenarios for defining the NPRACH performance requirements.

Scenario 1: Option 1 (Pmd+Pte<1%) with timing limit 2.5us and repetition 8;

Scenario 2: Option 1 (Pmd+Pte<1%) with timing limit 3.75us and repetition 8;

Scenario 3: Option 1 (Pmd+Pte<1%) with timing limit 5us and repetition 8;

Scenario 4: Option 2 (Pmd<=1%, Pte<=1%) with timing limit 2.5us and repetition 8;

Scenario 5: Option 2 (Pmd<=1%, Pte<=1%) with timing limit 3.75us and repetition 8;

Scenario 6: Option 2 (Pmd<=1%, Pte<=1%) with timing limit 5us and repetition 8;

Scenario 7: Option 1 (Pmd+Pte<1%) with timing limit 2.5us and repetition 32;

Scenario 8: Option 1 (Pmd+Pte<1%) with timing limit 3.75us and repetition 32;

Scenario 9: Option 1 (Pmd+Pte<1%) with timing limit 5us and repetition 32;

Scenario 10: Option 2 (Pmd<=1%, Pte<=1%) with timing limit 2.5us and repetition 32;

Scenario 11: Option 2 (Pmd<=1%, Pte<=1%) with timing limit 3.75us and repetition 32;

Scenario 12: Option 2 (Pmd<=1%, Pte<=1%) with timing limit 5us and repetition 32;

Table 1 NPRACH requirements for Scenario 1: Pmd+Pte<1%, timing limit 2.5us, repetition 8

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 |
| 1 | 2 | AWGN | 0 | -1.97 | -1.97 |
|  |  | EPA1 Low | 200 Hz | 5.50 | 5.50 |
|  | 4 | AWGN | 0 | -4.58 | -4.58 |
|  |  | EPA1Low | 200 Hz | -0.78 | -0.78 |
|  | 8 | AWGN | 0 | -6.87 | -6.87 |
|  |  | EPA1Low | 200 Hz | -4.41 | -4.41 |

Table 2 NPRACH requirements for Scenario 2: Pmd+Pte<1%, timing limit 3.75us, repetition 8

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 |
| 1 | 2 | AWGN | 0 | -4.64 | -4.64 |
|  |  | EPA1 Low | 200 Hz | 4.17 | 4.17 |
|  | 4 | AWGN | 0 | -6.96 | -6.96 |
|  |  | EPA1Low | 200 Hz | -2.15 | -2.15 |
|  | 8 | AWGN | 0 | -8.98 | -8.98 |
|  |  | EPA1Low | 200 Hz | -6.17 | -6.17 |

Table 3 NPRACH requirements for Scenario 3: Pmd+Pte<1%, timing limit 5us, repetition 8

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 |
| 1 | 2 | AWGN | 0 | -5.83 | -5.83 |
|  |  | EPA1 Low | 200 Hz | 3.74 | 3.74 |
|  | 4 | AWGN | 0 | -8.05 | -8.05 |
|  |  | EPA1Low | 200 Hz | -2.62 | -2.62 |
|  | 8 | AWGN | 0 | -9.99 | -9.99 |
|  |  | EPA1Low | 200 Hz | -6.55 | -6.55 |

Table 4 NPRACH requirements for Scenario 4: Pmd<1%, Pte<1%, timing limit 2.5us, repetition 8

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 |
| 1 | 2 | AWGN | 0 | -1.97 | -1.97 |
|  |  | EPA1 Low | 200 Hz | 4.29 | 4.29 |
|  | 4 | AWGN | 0 | -4.58 | -4.58 |
|  |  | EPA1Low | 200 Hz | -1.06 | -1.06 |
|  | 8 | AWGN | 0 | -6.87 | -6.87 |
|  |  | EPA1Low | 200 Hz | -4.46 | -4.46 |

Table 5 NPRACH requirements for Scenario 5: Pmd<1%, Pte<1%, timing limit 3.75us, repetition 8

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 |
| 1 | 2 | AWGN | 0 | -4.64 | -4.64 |
|  |  | EPA1 Low | 200 Hz | 3.14 | 3.14 |
|  | 4 | AWGN | 0 | -6.96 | -6.96 |
|  |  | EPA1Low | 200 Hz | -2.94 | -2.94 |
|  | 8 | AWGN | 0 | -8.98 | -8.98 |
|  |  | EPA1Low | 200 Hz | -6.62 | -6.62 |

Table 6 NPRACH requirements for Scenario 6: Pmd<1%, Pte<1%, timing limit 5us, repetition 8

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 |
| 1 | 2 | AWGN | 0 | -5.87 | -5.87 |
|  |  | EPA1 Low | 200 Hz | 3.14 | 3.14 |
|  | 4 | AWGN | 0 | -8.18 | -8.18 |
|  |  | EPA1Low | 200 Hz | -2.94 | -2.94 |
|  | 8 | AWGN | 0 | -10.13 | -10.13 |
|  |  | EPA1Low | 200 Hz | -6.83 | -6.83 |

Table 7 NPRACH requirements for Scenario 7: Pmd+Pte<1%, timing limit 2.5us, repetition 32

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 |
| 1 | 2 | AWGN | 0 | -7.25 | -7.25 |
|  |  | EPA1 Low | 200 Hz | -0.31 | -0.31 |

Table 8 NPRACH requirements for Scenario 8: Pmd+Pte<1%, timing limit 3.75us, repetition 32

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 |
| 1 | 2 | AWGN | 0 | -9.17 | -9.17 |
|  |  | EPA1 Low | 200 Hz | -1.34 | -1.34 |

Table 9 NPRACH requirements for Scenario 9: Pmd+Pte<1%, timing limit 5us, repetition 32

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 |
| 1 | 2 | AWGN | 0 | -9.92 | -9.92 |
|  |  | EPA1 Low | 200 Hz | -1.62 | -1.62 |

Table 10 NPRACH requirements for Scenario 10: Pmd<1%, Pte<1%, timing limit 2.5us, repetition 32

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 |
| 1 | 2 | AWGN | 0 | -7.25 | -7.25 |
|  |  | EPA1 Low | 200 Hz | -1.20 | -1.20 |

Table 11 NPRACH requirements for Scenario 11: Pmd<1%, Pte<1%, timing limit 3.75us, repetition 32

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 |
| 1 | 2 | AWGN | 0 | -9.18 | -9.18 |
|  |  | EPA1 Low | 200 Hz | -1.96 | -1.96 |

Table 12 NPRACH requirements for Scenario 12: Pmd<1%, Pte<1%, timing limit 5us, repetition 32

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 |
| 1 | 2 | AWGN | 0 | -10.12 | -10.12 |
|  |  | EPA1 Low | 200 Hz | -1.96 | -1.96 |

# Summary

Further simulation results, together with the performance of total timing error probability, are provided in the contribution to study possible specification for NPRACH performance requirements.  From our study and our previous investigations, e.g., in [5], we would like to propose:

Proposal 1:	Define the SNR value at the minimum performance requirement for NPRACH to satisfy both Pmd≤1% and Pte≤1%.

Proposal 2:	Set timing limit=3.75us for timing estimation error probability Pte.

Proposal 3:	Pick #Rep=8 for NPRACH performance requirement specification.

# References

- R4-164350, “Investigation on NPRACH performance”, Nokia

- R4-164351, “Considerations on NPRACH performance requirements”, Nokia

- R4-79AH-0067, “Impact to NPRACH ToA estimation performance”, Nokia

- R4-166616, Further analysis on the NPRACH performance specification, Nokia

- R4-167115, WF on NPRACH performance requirements, Nokia, Ericsson, ZTE

- 3GPP TS 36.211

- 3GPP TS 36.213

- 3GPP TS 36.321

- 3GPP TS 36.331