TSG-RAN Working Group 4 (Radio) meeting #80bis	 R4-168553

Ljubljana, Slovenia, 10 - 14 Oct, 2016

Source:	Airbus DS SLC

Title:	Band 68 compliance with ECC Decision (16)02: impact on 3GPP specifications

Agenda item:	8.32.2

Document for:	Approval

## 1	Introduction

At the last RAN4 meeting, a liaison statement from CEPT/ECC on “ETSI standards / 3GPP specifications for BB-PPDR equipment; technical conditions in CEPT” [1] was received.

A response has been agreed and sent back in [2]. Regarding the work associated with the band 698-703MHz / 753-758MHz, it was proposed that “Compliance with the said emission limit and the resulting output power performance for BB-PPDR operation will be considered by RAN4 for a possible amendment of the 3GPP specifications.”

This contribution further investigates the number of 3GPP specifications that would be impacted and the amount of work needed to introduce the compliance with the emission limits described in ECC Decision (16)02 [3].

## 2	Discussion

## 2.1	Impacted 3GPP specifications

The table below lists the 3GPP Technical Specifications that were impacted by the work item LTE_FDD_700_ARAB-core (the list was derived from the table available on the 3GU Portal). For each specification, it is indicated in the last column whether the compliance with the emission limits described in ECC Decision (16)02 [3] would require an update of the specification.

| Spec # | TDoc Title | TSG TDoc # | Need update |
| --- | --- | --- | --- |
| 25.104 | Introduction of Band 68 | R4-161332 | N |
| 25.461 | Introduction of Band 68 into 25.461 | R3-160217 | N |
| 25.466 | Introduction of Band 68 into 25.466 | R3-160216 | N |
| 36.101 | Introduction of Band 68 for Arab region into 36.101 | R4-161482 | Y |
| 36.104 | Introduction of Band 68 into 36.104 | R4-161475 | N |
| 36.113 | Introduction of Band 68 into 36.113 | R4-160274 | N |
| 36.124 | Introduction of Band 68 in TS 36.124 | R4-160289 | N |
| 37.104 | Introduction of Band 68 into 37.104 | R4-161476 | N |
| 37.113 | Introduction of Band 68 into 37.113 | R4-160275 | N |

From the above table, only the technical specification 36.101 is impacted.

## 2.2 Impacted sections of TS36.101

The compliance to the emission limits defined in ECC Decision (16)02 [3] could be defined through the introduction of a new NS value in TS36.101.

Thus the section “6.2.4	UE maximum output power with additional requirements” would be impacted: a new line shall be added in table 6.2.4-1, pointing to

- A new table in section 6.2.4 defining A-MPR for this new NS value.

- A new subsection 6.6.3.3, defining the additional requirements applicable for the new NS value and matching the requirements defined in ECC Decision (16)02 [3]

In addition, the Table 6.6.3.2-1 defining the requirements for spurious emission band UE co-existence shall be updated.

## 3	Conclusion

The required changes to Band 68 in order to take into account the ECC Decision (16)02  [3] were investigated: the Technical Specification TS36.101 was the only specification for which some changes were identified. The impacted sections were also listed.

Given the limited impact on the 3GPP specifications, it is proposed that the work to introduce new requirements to band 68 that complies with ECC Decision (16)02 [3] is done under TEI work item.

Similar approach was used for Band 31 (for which requirements were originally defined for Brazil) when European Requirements were introduced later under work item TEI12 (see TS36.101, CR 2862 in [4]).

## 4	References

[1]	R4-165137 Liaison Statement from CEPT ECC: ETSI standards / 3GPP specifications for BB-PPDR equipment; technical conditions in CEPT – Eric Fournier, Chairman CEPT Electronic Communications Committee

[2]	R4-167132 Reply LS to CEPT ECC on 3GPP specifications for BB-PPDR equipment; technical conditions in CEPT - RAN4

[3]	Decision (16)02: Harmonised technical conditions and frequency bands for the implementation of Broadband Public Protection and Disaster Relief (BB-PPDR) systems - http://www.erodocdb.dk/Docs/doc98/official/pdf/ECCDEC1602.PDF

[4]	R4-150996 Band 31 update - Orange, Huawei, Nokia Corporation