3GPP TSG-RAN WG4 Meeting #78bis	R4-161590

San Jose Del Cabo, Mexico, 11 – 15 April, 2016

Agenda Item:	7.3.2

Source: 	Nokia, U.S. Cellular

Title: 	TP for TR 36.714-03-01: BS Coexistence Studies of Harmonics and Intermodulation Products caused by LTE 3DL/1UL Advanced Carrier Aggregation with Bands 2, 4, 5 and 12A (RAN#71)

Document for:	Approval

# 1.	Introduction

The revised basket WI including support of the LTE Advanced 3 Downlink/1 Uplink (3DL/1UL) Carrier Aggregation (CA) of 3 band combinations with Bands 2, 4, 5 and 12A were approved in RAN#71 [1]. The required BS coexistence studies of harmonics and IMD products for the 3 band combinations were provided into the Rel-13 3DL/1UL TR 36.853-13 [2] in RAN4#77 [3-5].

In this contribution, we provide a text proposal to put the BS coexistence studies into the Rel-14 3DL/1UL TR 36.714-03-01 [6] to avoid unnecessary confusion.

# 2.	Text Proposal

<New clause>

6.x	CA_3DL_2A-12A-12A_1UL_BCS0

### 6.x.1	Channel bandwidths per operating band for CA

Table 6.x.1-1: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_2A-12A-12A | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  | 12 |  |  | Yes |  |  |  |  |  |
|  | 12 |  |  | Yes |  |  |  |  |  |

### 6.x.2	Co-existence studies

The 3rd order IMD products caused in the BS by transmitting CA_2A-12A-12A DL carriers can be calculated as shown in Table 6.x.2-1 below:

Table 6.x.2-1: CA_2A-12A-12A DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 729 | 746 | 1930 | 1990 |
| 3rd order IMD products | (f1-low + f2-low – f3-high) |  |  | (f1-high + f2-high – f3-low) |  |  |
| IMD frequency limits (MHz) | 438 |  |  | 532 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 1913 |  |  | 2007 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 1913 |  |  | 2007 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 3388 |  |  | 3482 |  |  |

It can be seen from Table 6.x.2-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting CA_2A-12A-12A may fall into the BS receive band of Bands 1, 22, 23, 25, 31, 33, 36, 37, 39 and 42. However, Bands 1, 22, 31, 33 and 39 are not intended for use in the same geographical area as Bands 2 and 12. Moreover, co-location of CA_2A-12A-12A transmitter and Band 36 or 37 transceiver implies FDD/TDD co-location on adjacent frequencies which requires the use of certain site-engineering solutions to avoid mutual interference. Therefore, the focus here is on the 3rd order IMD products falling into Bands 23, 25 and 42.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 42 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that CA_2A-12A-12A BS transmitter does not share the same antenna with Band 42 BS receiver. But there is not a large frequency gap between Band 2 DL and Bands 23 and 25 UL, and hence Band 23 or 25 BS receiver desensitization may still be an issue.

Therefore, it is recommended that CA_2A-12A-12A BS transmitter should not share the same antenna with Band 23, 25 or 42 BS receiver to prevent BS receiver desensitization, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 23, 25 or 42 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

6.y	CA_3DL_4A-12A-12A_1UL_BCS0

### 6.y.1	Channel bandwidths per operating band for CA

Table 6.y.1-1: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_4A-12A-12A | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  | 12 |  |  | Yes |  |  |  |  |  |
|  | 12 |  |  | Yes |  |  |  |  |  |

### 6.y.2	Co-existence studies

The 3rd order IMD products caused in the BS by transmitting CA_4A-12A-12A DL carriers can be calculated as shown in Table 6.y.2-1 below:

Table 6.y.2-1: CA_4A-12A-12A DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 729 | 746 | 2110 | 2155 |
| 3rd order IMD products | (f1-low + f2-low – f3-high) |  |  | (f1-high + f2-high – f3-low) |  |  |
| IMD frequency limits (MHz) | 618 |  |  | 697 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 2093 |  |  | 2172 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 2093 |  |  | 2172 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 3568 |  |  | 3647 |  |  |

It can be seen from Table 6.y.2-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting CA_4A-12A-12A may fall into the BS receive band of Bands 42 and 43.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 42 or 43 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that CA_4A-12A-12A BS transmitter does not share the same antenna with Band 42 or 43 BS receiver.

Therefore, it is recommended that CA_4A-12A-12A BS transmitter should not share the same antenna with Band 42 or 43 BS receiver to prevent BS receiver desensitization, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 42 or 43 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

6.z	CA_3DL_5A-12A-12A_1UL_BCS0

### 6.z.1	Channel bandwidths per operating band for CA

Table 6.z.1-1: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_5A-12A-12A | 5 |  |  | Yes | Yes |  |  | 20 | 0 |
|  | 12 |  |  | Yes |  |  |  |  |  |
|  | 12 |  |  | Yes |  |  |  |  |  |

### 6.z.2	Co-existence studies

The 3rd order IMD products caused in the BS by transmitting CA_5A-12A-12A DL carriers can be calculated as shown in Table 6.z.2-1 below:

Table 6.z.2-1: CA_5A-12A-12A DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 729 | 746 | 869 | 894 |
| 3rd order IMD products | (f1-low + f2-low – f3-high) |  |  | (f1-high + f2-high – f3-low) |  |  |
| IMD frequency limits (MHz) | 564 |  |  | 623 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 852 |  |  | 911 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 852 |  |  | 911 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 2327 |  |  | 2386 |  |  |

It can be seen from Table 6.z.2-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting CA_5A-12A-12A may fall into the BS receive band of Bands 8, 20 and 40. However, Bands 5 and 12 are not intended for use in the same geographical area as Bands 8, 20 and 40. Therefore, except the frequency bands impacted by harmonics and IMD products from the constituent 2 DL CA configurations, the 3rd order IMD products caused by BS supporting CA_5A-12A-12A will not fall into the BS receive band of any other frequency band operating in the same geographical area.

<End of change>

# References

[1]	RP-160582, “Revised WID: LTE Advanced inter-band CA Rel-14 for 3DL/1UL”, Huawei, Hisilicon.

[2]	RP-160535, “	TR 36.853-13 v1.0.1 on Evolved Universal Terrestrial Radio Access (E-UTRA); LTE-Advanced 3 Band Carrier Aggregation (CA) (3DL/1UL)”, ZTE Corporation.

[3]	R4-156399, “TP for TR 36.853-13: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (2 + 12 + 12)”, Alcatel-Lucent, U.S. Cellular.

[4]	R4-156400, “TP for TR 36.853-13: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (4 + 12 + 12)”, Alcatel-Lucent, U.S. Cellular.

[5]	R4-156401, “TP for TR 36.853-13: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (5 + 12 + 12)”, Alcatel-Lucent, U.S. Cellular.

[6]	R4-16xxxx, “TR 36.714-03-01 v0.0.2: LTE Advanced inter-band CA Rel-14 for 3DL/1UL”, Huawei, Hisilicon.